This project requires a hub and some Ethernet cables.

## Choosing Machines for your LAN

- Select two adjacent machines in the room.  Choose one to be the Internet Gateway.  The other one is the Client.  Use the virtual machines.  You cannot do this project on the host system.

# Testing the Internet Gateway

- Start the Internet Gateway.  Open Internet Explorer and verify that you can connect to the Internet.  If you cannot, call the instructor over to fix the problem before proceeding any further.

# Testing the Client

- Start the Client.  Open Internet Explorer and verify that you can connect to the Internet.  If you cannot, call the instructor over to fix the problem before proceeding any further.

## Wiring the LAN (Local Area Network)

Hub

To the Internet

Client

Internet

Gateway

Put a hub on the table between your machines.  Plug the power into the hub and make sure its front panel lights light up.

- Look at the back panel of one of your machines.  There will be a blue cable plugged in – that’s the Internet connection.  There may also be a red cable – if there is, unplug it and replace it when this project is over.

- Remove the blue Ethernet cable from the back of the Client machine, and replace it with a short patch cord that goes to a numbered port on the hub.  It does not matter which port on the hub you use – they are all the same, except the Uplink port.  If there is an Uplink port, don’t use it – just leave it empty.

- Leave the blue cable connected on the Internet Gateway machine.  Connect a patch cord from the second Ethernet port (on an expansion card) to a numbered port on the hub.  Your final network should look like the diagram to the right on this page.

## Using the Network Setup Wizard to Create an Internet Gateway

- Do these steps on your Internet Gateway machine.

- On your Internet Gateway click Start, My Computer.  In the My Computer window, in the left pane, click My Network Places.

- In the left pane of the My Network Places window, in the Network Tasks section, click the Set up a home or small office network link.

- At the Welcome to the Network Setup Wizard screen, click Next.

- At the Before you continue… screen, click Next.

- At the Select a connection method screen, select This computer connects directly to the Internet.  The other computers on my network connect to the Internet through this computer and click Next.

- At the Select your Internet connection screen, accept the default selection and click Next.

- At the Give this computer a description and a name screen, leave the description unchanged and verify that the Computer name is the same as the label on the front of your system unit with a V at the end (for example, S214-01V), and click Next.

- At the Name your network screen, enter a name of WORKGROUP and click Next.

- If a File and printer sharing screen appears, select Turn on file and printer sharing and click Next.

- At the Ready to apply network settings screen, read the settings, and if they look correct, click Next.

- At the Please wait screen, wait for the animation to end.  It might take a minute or longer.

- At the You’re almost done screen, click Just finish the wizard.  I don’t need to run the wizard on other computers and click Next.

- At the Completing the Network Setup Wizard screen, click Finish.

- If you are prompted to restart your machine, do so.

## Testing the Internet Gateway’s Internet Connection

- Open Internet Explorer on the Internet Gateway machine.  You should be able to access the Internet.

## Using the Network Setup Wizard to Connect the Client Machine to the Internet Gateway

- Do these steps on your Client machine.

- Click Start, My Computer.  In the My Computer window, in the left pane, click My Network Places.

- In the left pane of the My Network Places window, in the Network Tasks section, click the Set up a home or small office network link.

- At the Welcome to the Network Setup Wizard screen, click Next.

- At the Before you continue… screen, click Next.

- There are two possibilities for the next screen:

  - If you see a Select a connection method screen, select This computer connects to the Internet through a residential gateway or through another computer on my network and click Next.

  - If you see a Do you want to use the shared connection? screen, select Yes, use the existing shared connection… and click Next.

- At the Give this computer a description and a name screen, leave the description unchanged and verify that the Computer name is the same as the label on the front of your system unit with a V at the end (for example, S214-01V), and click Next.

- At the Name your network screen, enter a name of WORKGROUP and click Next.

- If a File and printer sharing screen appears, select Turn on file and printer sharing and click Next.

- At the Ready to apply network settings screen, read the settings, and if they look correct, click Next.

- At the Please wait screen, wait for the animation to end.  It might take a minute or longer.

- At the You’re almost done screen, click Just finish the wizard.  I don’t need to run the wizard on other computers and click Next.

- At the Completing the Network Setup Wizard screen, click Finish.

- If you are prompted to restart your machine, do so.

## Testing the Client’s Internet Connection

- Open Internet Explorer on the Client machine.  You should be able to access the Internet.

## Shutting Down the Gateway Machine

- Shut down the gateway machine.  Then try to use the Internet from the client machine.  You can’t, of course, because their only path to the Internet goes through the gateway machine.

## Restarting the Gateway Machine

- Start the gateway machine.  Then try to use the Internet from the client machine.  They are again connected to the Internet.

Last modified 10-26-06