Required Equipment

### Jacket Stripper (red handle, holes from .40 to 1.3 mm) used to remove the jacket from the outside of the cable

### Miller Fiber Stripper (yellow handle, labeled Clauss Fiberoptic Stripper) used to remove the coating from the bare fiber, leaving only the core and cladding

### Cleaver (looks like a small stapler) used to break the fiber off cleanly

### Cleaning equipment:

### Kimwipes

### Alcohol bottle

### Two cables: one Singlemode and one Multimode

### An SC-to-SC adapter as shown in the photo above

I. Preparing the Fiber

Your instructor will show you the details of these steps:

- Safety Rule: Wear glasses or safety glasses

- Safety Rule: Make sure there is sticky tape on the table to hold fiber shards

- Find the two cables to be spliced together.  Choose one of them to start with.

- Strip the jacket off about 5 inches of cable with the Jacket Stripper

- Strip the coating off about 2 inches of the fiber with the Miller Fiber Stripper

- Clean the fiber with a Kimwipe and alcohol, squeeze hard and make it squeak

- Cleave the fiber with the cleaver.  Carefully remove the glass shard and put it on the sticky tape.  Be careful not to touch the end of the cleaved fiber to any object – you need to keep it clean.

- Insert the fiber into the fusion splicer.

- Take the other cable and repeat steps 4-8 to prepare it.

Fusion Splicing

- Just press the Splice button on the splicer.  The process is automatic.  Watch its progress on the screen.