# Goals of This Chapter

## List the advantages of networked computing relative to standalone computing

## Distinguish between client/server and peer-to-peer networks

## List elements common to all client/server networks

## Describe several specific uses for a network

## Identify some of the certifications available to networking professionals

## Identify the kinds of skills and specializations that will help you excel as a networking professional

Why Use Networks?

## Network

### Group of computers and devices

#### Connected by transmission media

## Stand-alone computer

### Not connected to other computers

### Uses local software and data

## Advantages of networks over standalone computers

### Device sharing by multiple users

#### Saves money and time

### Central network management

Types of Networks

## Models vary according to:

### Computer positioning

### Control levels over shared resources

### Communication and resource sharing schemes

## Network models

### Peer-to-Peer

### Client/server

# Peer-to-Peer Network

## Typical in a home with several computers

## Direct computer communication

### Equal authority

## Individual resource sharing

### May share resources

### May prevent access to resources

## Each computer can send data to every other computer on the network

## Advantages

### Simple configuration

### Inexpensive to set up

## Disadvantages

Not flexible

Peer-to-Peer Network

### Not necessarily secure

### Not practical for large installations

## Resource sharing method

### Modify file sharing controls

#### A user responsibility

### Not centrally controlled

#### Potential variations and security issues

## Environments

### Small home or office

### Large networks using the Internet

#### Gnutella, Freenet, original Napster

#### BitTorrent software

# Obama’s Helicopter Secrets Revealed

## Because of misconfigured peer-to-peer file sharing

## Link Ch 1a on the course Web page

### samsclass.info

### Click on CNIT 106

# Client/Server Networks

## Central computer (server)

### Facilitates communication and resource sharing

## Clients (other computers)

### Personal computers

#### Known as workstations

## Central resource sharing controlled by server

### Data sharing, data storage space, devices

### No direct sharing of client resources

## Computer roles

### Server

### Clients

#### Run local applications

#### Store data locally

#### Use server shared applications, data, devices

#### Use server as intermediary

## Communication

### Switches or routers

## Server requirement

Network operating system

Client/Server Network

#### Manages client data, resources

#### Ensures authorized user access

#### Controls user file access

#### Restricts user network access

#### Dictates computer communication rules

#### Supplies application to clients

## Server examples

### UNIX, Linux, Microsoft Server 2003 and 2008, MAC OS X Server

## Server features relative to clients

### More memory, processing, storage capacity

### Equipped with special hardware

#### Provides network management functions

## Disadvantages relative to peer-to-peer networks

### Complex in design and maintenance

## Advantages relative to peer-to-peer networks

### User credential assigned from one place

### Multiple shared resource access centrally controlled

### Central problem monitoring, diagnostics, correction capabilities

### User response time optimization capabilities

### Efficient processing on large networks

### Scalability

## Popular in medium- and large-scale organizations

# LANs, MANs, and WANs

## LAN (local area network)

### Network confined to a relatively small space

### 1980s -- LANs became popular as peer-to-peer based

### Today -- Larger and more complex client/server network

## MAN (metropolitan area network)

### Network extends beyond building boundaries

### Larger than LAN

### Connects clients and servers from multiple buildings

## WAN (wide area network)

### Connects two or more geographically distinct LANs or MANs

### Comparison to LANs

#### Use slightly different transmission methods and media

#### Use greater variety of technologies

### Network connection

#### Separate offices in same organization

#### Separate offices in different organizations

# Warriors of the Net Video

## Links Ch 1b, 1c

Elements Common to Client/Server Networks

## Client

### Network computer requesting resources or services from another network computer

### Client workstation human user

### Client software installed on workstation

## Server

### Network computer managing shared resources

### Runs network operating system

## Workstation

### Personal computer

#### May or may not be connected to network

## NIC (network interface card)

### Device inside computer

#### Connects computer to network media

#### Allows communication with other computers

## NOS (network operating system)

### Server software

### Enables server to manage data, users, groups, security, applications, and other networking functions

# Ethernet NIC

## Host

### A computer that enables other computers to share resources

## Node

### Client, server, or other device

#### Communicates over a network

#### Identified by unique number (network address)

## Connectivity device

### Allows multiple networks or multiple parts of one network to connect and exchange data

### Such as a switch or router

## Segment

### Group of nodes

#### Use same communications channel for traffic

## Backbone

### Connects segments and significant shared devices

### “A network of networks”

## Topology

### Computer network physical layout

### Ring, bus, star or hybrid formation

# A LAN Backbone

## Protocol

### Standard method or format for communication between networked devices

## Data packets

### Distinct data units exchanged between nodes

## Addressing

### Scheme for assigning  unique identifying number to every node

## Transmission media

### Means through which data is transmitted and received

# Common Network Topologies

# Transmission Media

How Networks Are Used

## Network services

### Functions provided by a network

### Most visible

#### E-mail

### Other vital services

#### Printer sharing, file sharing, Internet access and Web site delivery, remote access capabilities, the provision of voice (telephone) and video services, network management

# File and Print Services

## File services

### Capability of server to share data files, applications and disk storage space

## File server

### Provides file services

## File services provide foundation of networking

## Print services

### Share printers across network

### Saves time and money

# Access Services

## Allow remote user network connection

## Allow network users to connect to machines outside the network

## Remote user

### Computer user on different network or in different geographical location from LAN’s server

## Network operating systems include built-in access services

# Access Services

## Provide LAN connectivity when WAN connection is not cost-effective

### External staff used to diagnose problems

## Allow external users to use network resources and devices

### Same as if logged on to office workstation

# Communications Services

## Convergence

### Multiple types of communications services on the same network

#### Phone calls, fax, text messages, video…

## Unified communications

### Centralized management of network-based communications

## E-mail

### Oldest and most frequently used

## Mail server

### Computer responsible for mail services

#### Coordinates storage and transfer of e-mail

## Additional tasks of mail servers

### Intercept spam

### Handle objectionable content

### Route messages according to rules

### Provide Web-based client

### Notify administrators or users if certain events occur

### Schedule e-mail transmission, retrieval, storage, maintenance functions

### Communicate with mail servers on other networks

## Mail server runs specialized mail server software

# Internet Services

## Supplying Web pages

### Servers work together to bring Web pages to user’s desktop

### Web server

#### Computer installed with appropriate software to supply Web pages to many different clients upon demand

## Other Internet services

### File transfer capabilities, Internet addressing schemes, security filters, means for directly logging on to other Internet computers

# Management Services

## Small network management

### Single network administrator

### Network operating system’s internal functions

## Today’s larger network management

### Centrally administered network management tasks

## Other important services

### Traffic monitoring and control

### Load balancing

### Hardware diagnosis and failure alert

### Asset management

### License tracking

### Security auditing

### Address management

### Backup and restoration of data

Becoming a Networking Professional

## Job market

### Many job postings for computer professionals

### Expertise levels required vary

## To prepare for entering job market

### Master general networking technologies

### Select areas of interest

### Study those specialties

### Hone communication and teamwork skills

### Stay abreast of emerging technologies

# Mastering the Technical Challenges

## Networking positions utilizing logical and analytical thinking

### Obtain skill sets desired

## Positions in high demand

### Consider a general knowledge of all

### Specialize in a few

## Determine appropriate personal learning methods

## Obtain hands-on experience

# Developing Your “Soft Skills”

## Soft skills

### Not easily measurable

### Important to networking projects

#### Customer relations

#### Oral and written communications

#### Dependability

#### Teamwork

#### Leadership abilities

# Pursuing Certification

## Certification process

### Master material

#### Pertaining to particular hardware system, operating system, programming language, software application

### Proving mastery

#### Pass exams

## Professional organizations

### CompTIA

## Vendors

### Microsoft , Cisco

## Benefits

### Better salary

### Greater opportunities

### Professional respect

### Access to better support

## Drawback

### Number of people obtaining and pursuing them

### Cheating and fraud (strangely not mentioned in the book)

# Finding a Job in Networking

## Job research methods

### Search the Web

### Read the newspaper

### Visit a career center

### Network

### Attend career fairs

### Enlist a recruiter

# Joining Professional Associations

## Provide varying benefits

### Connect with people having similar interests

### New learning opportunities

### Specialized information access

### Tangible assets (free goods)

### Publications

### Technical workshops and conferences

### Free software, prerelease software

### Expensive hardware lab access

Last modified 1-20-07 0:12