# Objectives

## Identify organizations that set standards for networking

## Describe the purpose of the OSI model and each of its layers

## Explain specific functions belonging to each OSI model layer

# Objectives (cont’d.)

## Understand how two network nodes communicate through the OSI model

## Discuss the structure and purpose of data packets and frames

## Describe the two types of addressing covered by the OSI model

Networking Standards Organizations

## Standard

### Documented agreement

### Technical specifications/precise criteria

### Stipulates design or performance of particular product or service

## Standards are essential in the networking world

### Wide variety of hardware and software

#### Ensures network design compatibility

## Standards define minimum acceptable performance

### Not ideal performance

## Many different organizations oversee computer industry standards

### Organizations may overlap responsibilities

#### Example: ANSI and IEEE set wireless standards

## Network professional’s responsibility

### Be familiar with groups setting networking standards

### Understand critical aspects of standards required by own networks

# ANSI

## ANSI (American National Standards Institute)

### 1000+ representatives from industry and government

### Determines standards for electronics industry and other fields

## Requests voluntarily compliance with standards

## Obtaining ANSI approval requires rigorous testing

## ANSI standards documents available online

# EIA and TIA

## EIA (Electronic Industries Alliance)

### Trade organization

#### Representatives from United States electronics manufacturing firms

### Sets standards for its members

### Helps write ANSI standards

### Lobbies for favorable computer and electronics industries legislation

## TIA (Telecommunications Industry Association)

### Formed in 1988

#### EIA subgroup merged with former United States Telecommunications Suppliers Association (USTSA)

### Focus of TIA

#### Standards for information technology, wireless, satellite, fiber optics, and telephone equipment

## TIA/EIA 568-B Series

### Guidelines for installing network cable in commercial buildings

# IEEE

## IEEE (Institute of Electrical and Electronics Engineers)

### International engineering professionals society

## Goal of IEEE

### Promote development and education in electrical engineering and computer science fields

## Hosts symposia, conferences, and chapter meetings

## Maintains a standards board

## IEEE technical papers and standards

### Highly respected

# IEEE Student Chapter at CCSF

## Last semester, the chapter arranged a tour of a data center

## I'll let you know what's coming up this semester

# ISO

## ISO (International Organization for Standardization)

### Headquartered in Geneva, Switzerland

### Collection of standards organizations

#### Representing 57 countries

## Goal of ISO

### Establish international technological standards to facilitate global exchange of information and barrier free trade

## Widespread authority

# ITU

## ITU (International Telecommunication Union)

### Specialized United Nations agency

### Regulates international telecommunications

### Provides developing countries with technical expertise and equipment

### Founded in 1865

#### Joined United Nations in 1947

### Members from 191 countries

## Focus of ITU

### Global telecommunications issues

### Worldwide Internet services implementation

# ISOC

## ISOC (Internet Society)

### Founded in 1992

### Professional membership society

### Establishes technical Internet standards

## Current ISOC concerns

### Rapid Internet growth

### Keeping Internet accessible

### Information security

### Stable Internet addressing services

### Open standards

## ISOC oversees groups with specific missions

### IAB (Internet Architecture Board)

#### Technical advisory group

#### Overseeing Internet’s design and management

### IETF (Internet Engineering Task Force)

#### Sets Internet system communication standards

#### Particularly protocol operation and interaction

#### Anyone may submit standard proposal

#### Elaborate review, testing, and approval processes

# IANA and ICANN

## IP (Internet Protocol) address

### Address identifying computers in TCP/IP based (Internet) networks

### Reliance on centralized management authorities

## IP address management history

### Initially: IANA (Internet Assigned Numbers Authority)

### 1997: Three RIRs (Regional Internet Registries)

#### ARIN (American Registry for Internet Numbers)

#### APNIC (Asia Pacific Network Information Centre)

#### RIPE (Réseaux IP Européens)

## IP address management history (cont’d.)

### Late 1990s: ICANN (Internet Corporation for Assigned Names and Numbers)

#### Private nonprofit corporation

#### Remains responsible for IP addressing and domain name management

#### IANA performs system administration

## Users and business obtain IP addresses from ISP (Internet service provider)

# ICANN's Stormy History

## 2002: ICANN was sued by one of their own board members to force them to disclose financial documents

## 2003: ICANN sued Verisign to make them shut down their "Site Finder" service, which captured all mistyped URLs and showed them Verisign ads

## 2004 Verisign sued ICANN saying they had overstepped their authority

## 2008 Ten worst spam offenders notified by ICANN

### Links Ch 2a, 2b, 2c

The OSI Model

## Model for understanding and developing network computer-to-computer communications

## Developed by ISO (1980s)

## Divides network communications into seven layers

### Physical, Data Link, Network, Transport, Session, Presentation, Application

## Protocol interaction

### Layer directly above and below

## Application layer protocols

### Interact with software

## Physical layer protocols

### Act on cables and connectors

# The OSI Model (cont’d.)

## Theoretical representation describing network communication between two nodes

## Hardware and software independent

## Every network communication process represented

## PDUs (protocol data units)

### Discrete amount of data

### Application layer function

### Flow through layers 6, 5, 4, 3, 2, and 1

## Generalized model and sometime imperfect

# Mnemonics for the OSI Model

## Bottom Up

### Please

### Do

### Not

### Throw

### Sausage

### Pizza

### Away

## Top Down

### All

### People

### Seem

### To

### Need

### Data

### Processing

# Application Layer

## Top (seventh) OSI model layer

## No software applications

## Protocol functions

### Facilitates communication

#### Between software applications and lower-layer network services

### Network interprets application request

### Application interprets data sent from network

# Presentation Layer

## Protocol functions

### Accept Application layer data

### Formats data to make it understandable to different applications and hosts

## Examples

### Text encoding methods ASCII and ANSI

### Compression methods like JPEG and MP3

### Data encryption and decryption

# Session Layer

## Protocol functions

### Coordinate and maintain communications between two nodes

## Session

### Connection for ongoing data exchange between two parties

#### Example: between Web browser client and Web server

## Functions

### Establishing and keeping alive communications link

#### For session duration

### Determining if communications ended

#### Determining where to restart transmission

### Terminating communications

# Transport Layer

## Protocol functions

### Accept data from Session layer

### Provides flow control, segmentation, and error control

## Primary Transport Layer Protocols: TCP and UDP

# TCP

## Require acknowledgements from receiver to ensure data was received correctly

## Checksum

### Unique character string allowing receiving node to determine if arriving data unit exactly matches data unit sent by source

### Ensures data integrity

## UDP: A connectionless protocol

### Do not establish connection with another node before transmitting data—no handshake

### Make no effort to ensure data is delivered free of errors

### Faster than connection-oriented protocol

### Useful when data must be transferred quickly, such as streaming music or video

# Segmentation

### Breaking large data units received from Session layer into multiple smaller units called segments

### Increases data transmission efficiency

## MTU (maximum transmission unit)

### Largest data unit network will carry

### Ethernet default: 1500 bytes

### Discovery routine used to determine MTU

## Reassembly

### Process of reconstructing segmented data units

## Sequencing

### Method of identifying segments belonging  to the same group of subdivided data

# Error in Textbook

## On page 50, the textbook says that the Network layer translates IP addresses like 147.144.51.121 into MAC addresses like 00-30-48-82-11-BD

## That is false.  MAC addresses are used at layers 2 and 1 only.  Layer 3 works with IP packets which have had the MAC addresses removed from them.

# Network Layer

## Packet formation

### Transport layer segment is encapsulated by a Network layer packet

#### Network layer adds logical addressing information—IP address

## Routing

### Determine path from point A on one network to point B on another network

## Routing considerations

### Delivery priorities, network congestion, quality of service, cost of alternative routes

## Common Network layer protocol

### IP (Internet Protocol)

## Fragmentation

### Network layer protocol (IP) subdivides Transport layer segments received into smaller packets

### This is usually avoided by adjusting the Maximum Segment Size in the Transport layer, because it makes data transfer inefficient

#### Link Ch 2e

# Data Link Layer

## Function of protocols

### Divide data received into distinct frames for transmission in Physical layer

### Add Physical address to the frame: MAC addresses like 00-30-48-82-11-BD

## Frame

### Structured package for moving data

#### Includes raw data (payload), sender’s and receiver’s physical addresses, error checking and control information

## Frames may be damaged as they pass through the Physical layer, so the Data Link layer has error-checking

### Error checking

#### Frame check sequence

#### CRC (cyclic redundancy check)

## Possible glut of communication requests

### Data Link layer controls flow of information

#### Allows NIC to process data without error

## Two Data Link layer sublayers

### LLC (Logical Link Control) sublayer

### MAC (Media Access Control) sublayer

## MAC address components

### Block ID

#### Six-character sequence unique to each vendor

### Device ID

#### Six-character number added at vendor’s factory

## MAC addresses frequently depicted in hexadecimal format

# MAC Address is printed on NIC

# Example of MAC Address

## Whole MAC address: 00-30-48-82-11-BD

### Block ID

#### 00-30-48

#### Identifies the vendor

### Device ID

#### 82-11-BD

#### Different for each NIC from the same vendor

## If two NICs have the same MAC address, they have problems networking

### This can happen with cheaply made refurbished NICs, or with copied virtual machines

# Physical Layer

## Functions of protocols

### Accept frames from Data Link layer

### Generate signals as changes in voltage at the NIC

## Copper transmission medium

### Signals issued as voltage

## Fiber-optic cable transmission medium

### Signals issued as light pulses

## Wireless transmission medium

### Signals issued as electromagnetic waves

## Physical layer protocols responsibility when receiving data

### Detect and accept signals

### Pass on to Data Link layer

### Set data transmission rate

### Monitor data error rates

### No error checking

## Devices operating at Physical layer

### Hubs and repeaters

## NICs operate at both Physical layer and Data Link layers

Applying the OSI Model

# Communication Between Two Systems

## Data transformation

### Original software application data differs from the bits sent onto the cable by the NIC

#### Header data added at each layer

## Each layer has a different data structure, called a Protocol Data Unit (PDU)

# Segments

### Generated in Transport layer

### Unit of data resulting from subdividing larger PDU from the Session layer

### Addressed with TCP Ports (when using TCP)

# Packets

### Generated in Network layer

### Data with logical addressing information added to segments

### Addressed with IP addresses

# Frames

### Generated in Data Link layer

### Composed of several smaller components or fields

### Addressed with MAC addresses

## Encapsulation

### Occurs in each layer

### Process of wrapping one layer’s PDU with protocol information

#### Allows interpretation by lower layer

# Frame Specifications

## Frames

### Composed of several smaller components or fields

## Frame characteristic dependencies

### Network type where frames run

### Standards frames must follow

## Ethernet

### Developed by Xerox

### Four different types of Ethernet frames

### Most popular: IEEE 802.3 standard

## Token ring

### Developed by IBM

### Relies upon direct links between nodes and ring topology

### Nearly obsolete

### Defined by IEEE 802.5 standard

## Ethernet frames and token ring frames differ

### Will not interact with each other

### Devices cannot support more than one frame type per physical interface or NIC

IEEE Networking Specifications

## IEEE’s Project 802

### Effort to standardize physical and logical network elements

#### Frame types and addressing

#### Connectivity

#### Networking media

#### Error-checking algorithms

#### Encryption

#### Emerging technologies

## 802.3: Ethernet

## 802.11: Wireless

Last modified 8-26-09