# Objectives

## Explain basic data transmission concepts, including full duplexing, attenuation, latency, and noise

## Describe the physical characteristics of coaxial cable, STP, UTP, and fiber-optic media

## Compare the benefits and limitations of different networking media

## Explain the principles behind and uses for serial connector cables

## Identify wiring standards and the best practices for cabling buildings and work areas

Transmission Basics

## Transmit

### Issue signals along network medium

## Transmission

### Process of transmitting

### Signal progress after transmitted

## Transceiver

### Transmit and receive signals

# Analog and Digital Signaling

## Important data transmission characteristic

### Signaling type: analog or digital

## Volt

### Electrical current pressure

## Electrical signal strength

### Directly proportional to voltage

### Signal voltage

## Signals

### Current, light pulses, electromagnetic waves

## Analog data signals

### Voltage varies continuously

### Properties

# Amplitude, frequency, wavelength, phase

## Amplitude

### Analog wave’s strength

## Frequency

### Number of times amplitude cycles over fixed time period

### Measure in hertz (Hz)

## Wavelength

### Distance between corresponding wave cycle points

### Inversely proportional to frequency

### Expressed in meters or feet

# Phase

### Wave’s progress over time in relationship to fixed point

## Analog signal benefit over digital

### More variable

#### Convey greater subtleties with less energy

## Drawback of analog signals

### Varied and imprecise voltage

#### Susceptible to transmission flaws

## Digital signals

### Pulses of voltages

#### Positive voltage represents a 1

#### Zero voltage represents a 0

## Binary system

### 1s and 0s represent information

## Bit (binary digit)

### Possible values: 1 or 0

### Digital signal pulse

## Byte

### Eight bits together

## Computers read and write information

### Using bits and bytes

## Find decimal value of a bit

### Multiply the 1 or 0 by 2x (x equals bit’s position)

## Convert byte to decimal number

### Determine value represented by each bit

### Add values

## Convert decimal number to a byte

### Reverse the process

## Convert between binary and decimal

### By hand or calculator

# Binary to Decimal conversion

#### Binary numbers are sometimes written prefixed with 0b

#### 0b0001 = 0*8 + 0*4 + 0*2 + 1*1 = 1

#### 0b1010 = 1*8 + 0*4 + 1*2 + 0*1 = 10

#### 0b1011 = 1*8 + 0*4 + 1*2 + 1*1 = 11

# Decimal to Binary

#### 14 = 8+4+2 = 1*8 + 1*4 + 1*2 + 0*1 = 0b1110

#### 3   =    2+1   = 0*8 + 0*4 + 1*2 + 1*1 = 0b0011

#### 6   =     4+2  = 0*8 + 1*4 + 1*2 + 0*1 = 0b0110

# Larger Values

#### 128= 1*128 + 0*64 + 0*32 +0*16 +0*8 +0*4 + 0*2 +0*1

#### 128 = 0b10000000

#### 162= 128 + 32 + 2

#### 162= 1*128 + 0*64 + 1*32 +0*16 +0*8 +0*4 + 1*2 +0*1

#### 162 = 0b10100010

#### 0b1111000 =

#### 1*128 + 1*64 + 1*32 +1*16 +0*8 +0*4 + 0*2 +0*1 0 =

#### 128 + 64 + 32 +16 = 240

# Binary on Quizzes

## I'll add a binary question to the next quiz, and maybe the one after that, as extra credit

## I recommend that you learn binary conversion, but I don't require it

## Digital signal benefit over analog signal

### More reliable

### Less severe noise interference

## Digital signal drawback

### Many pulses required to transmit same information

## Overhead

### Nondata information

#### Required for proper signal routing and interpretation

#### Such as addressing information

# Data Modulation

## Data relies on digital transmission

## Network connection may handle only analog signals

## Modem

### Accomplishes translation

### Modulator/demodulator

## Data modulation

### Technology modifying analog signals

### Make data suitable for carrying over communication path

## Carrier wave

### Combined with another analog signal

### Produces unique signal

#### Transmitted from one node to another

### Preset properties

### Purpose

#### Convey information

## Information wave (data wave)

### Added to carrier wave

### Modifies one carrier wave property

## Frequency modulation (FM)

### Carrier frequency modified

#### By application of data signal

## Amplitude modulation (AM)

### Carrier signal amplitude modified

#### By application of data signal

# Simplex, Half-Duplex, and Duplex

## Simplex

### Signal transmission: one direction

### Like broadcast TV

## Half-duplex transmission

### Signal transmission: both directions

#### One at a time

### One communication channel

#### Shared for multiple nodes to exchange information

## Full-duplex

### Signals transmission: both directions simultaneously

### Used on data networks

## Channel

### Distinct communication path between nodes

### Separated physically or logically

## Full duplex advantage

### Increases speed

# Multiplexing

### Multiple signals

### Travel simultaneously over one medium

## Subchannels

### Logical multiple smaller channels

## Multiplexer (mux)

### Combines many channel signals

## Demultiplexer (demux)

### Separates combined signals

### Regenerates them

## TDM (time division multiplexing)

### Divides channel into multiple time intervals

Figure 3-7 Time division multiplexing

## Statistical multiplexing

### Transmitter assigns slots to nodes

#### According to priority, need

### More efficient than TDM

Figure 3-8 Statistical multiplexing

## FDM (frequency division multiplexing)

### Unique frequency band for each communications subchannel

### Two types

#### Cellular telephone transmission

#### DSL Internet access

Figure 3-9 Frequency division multiplexing

## WDM (wavelength division multiplexing)

### One fiber-optic connection

### Carries multiple light signals simultaneously

Figure 3-10 Wavelength division multiplexing

## DWDM (dense wavelength division multiplexing)

### Used on most modern fiber-optic networks

### Extraordinary capacity

# Relationships Between Nodes

## Point-to-point transmission

### One transmitter and one receiver

## Point-to-multipoint transmission

### One transmitter and multiple receivers

### Broadcast transmission

#### One transmitter and multiple, undefined receivers

#### Used on wired and wireless networks

## Simple and quick

### Nonbroadcast

#### One transmitter and multiple, defined receivers

# Throughput and Bandwidth

## Throughput

### Measures amount of data transmitted during given time period

### Capacity or bandwidth

### Quantity of bits transmitted per second

## Bandwidth (strict definition)

### Measures difference between highest and lowest frequencies medium can transmit

### Range of frequencies

### Measured in hertz (Hz)

# Throughput

# Baseband and Broadband

## Baseband transmission

### Digital signals sent through direct current (DC) pulses applied to wire

### Requires exclusive use of wire’s capacity

#### Transmit one signal (channel) at a time

### Example: Ethernet

## Broadband transmission

### Signals modulated

#### Radiofrequency (RF) analog waves

#### Uses different frequency ranges

### Does not encode information as digital pulses

# Transmission Flaws

## Noise

### Any undesirable influence degrading or distorting signal

## Types of noise

### EMI (electromagnetic interference)

#### EMI/RFI (radiofrequency interference)

### Cross talk

#### NEXT (near end cross talk)

#### Potential cause: improper termination

### Environmental influences

#### Heat

## Attenuation

### Loss of signal’s strength as it travels away from source

## Signal boosting technology

### Analog signals pass through amplifier

#### Noise also amplified

### Regeneration

#### Digital signals retransmitted in original form

#### Repeater: device regenerating digital signals

### Amplifiers and repeaters

#### OSI model Physical layer

## Latency

### Delay between signal transmission and receipt

## Causes

### Cable length

### Intervening connectivity device

## RTT (round trip time)

### Time for packet to go from sender to receiver, then back from receiver to sender

### Measured in milliseconds

## May cause network transmission errors

Common Media Characteristics

## Selecting transmission media

### Match networking needs with media characteristics

## Physical media characteristics

### Throughput

### Cost

### Size and scalability

### Connectors

### Noise immunity

# Throughput

## Most significant transmission method factor

## Causes of limitations

### Laws of physics

### Signaling and multiplexing techniques

### Noise

### Devices connected to transmission medium

## Fiber-optic cables allows faster throughput

### Compared to copper or wireless connections

# Cost

## Precise costs difficult to pinpoint

## Media cost dependencies

### Existing hardware, network size, labor costs

## Variables influencing final cost

### Installation cost

### New infrastructure cost versus reuse

### Maintenance and support costs

### Cost of lower transmission rate affecting productivity

### Cost of obsolescence

# Noise Immunity

## Noise distorts data signals

### Distortion rate dependent upon transmission media

#### Fiber-optic: least susceptible to noise

## Limit impact on network

### Cable installation

#### Far away from powerful electromagnetic forces

### Select media protecting signal from noise

### Antinoise algorithms

# Size and Scalability

## Three specifications

### Maximum nodes per segment

### Maximum segment length

### Maximum network length

## Maximum nodes per segment dependency

### Attenuation and latency

## Maximum segment length dependency

### Attenuation and latency plus segment type

## Segment types

### Populated: contains end nodes

### Unpopulated: No end nodes

#### Link segment

## Segment length limitation

### After certain distance, signal loses strength

#### Cannot be accurately interpreted

# Connectors and Media Converters

## Connectors

### Hardware connecting wire to network device

### Specific to particular media type

### Affect costs

#### Installing and maintaining network

#### Ease of adding new segments or nodes

#### Technical expertise required to maintain network

## Media converter

### Hardware enabling networks or segments running on different media to interconnect and exchange signals

Coaxial Cable

Figure 3-16 Coaxial cable

Central metal core (often copper)

### Surrounded by insulator

## Braided metal shielding (braiding or shield)

## Outer cover (sheath or jacket)

## High noise resistance

## Advantage over twisted pair cabling

### Carry signals farther before amplifier required

## Disadvantage over twisted pair cabling

### More expensive

## Hundreds of specifications

### RG specification number

### Differences: shielding and conducting cores

#### Transmission characteristics

## Conducting core

### American Wire Gauge (AWG) size

## Data networks usage

### RG-6: Used in modern cable TV connections, most common

### RG-8: Thicknet--obsolete

### RG-58: Thinnet—also obsolete for data networks

### RG-59: Used for short spans in modern cable TV connections

# Coaxial Cable Connectors

Twisted Pair Cable

Figure 3-19 Twisted pair cable

Color-coded insulated copper wire pairs

### 0.4 to 0.8 mm diameter

### Encased in a plastic sheath

## More wire pair twists per foot

### More resistance to cross talk

### Higher-quality

### More expensive

## Twist ratio

### Twists per meter or foot

## High twist ratio

### Greater attenuation

## Hundreds of different designs

### Dependencies

#### Twist ratio, number of wire pairs, copper grade, shielding type, shielding materials

### 1 to 4200 wire pairs possible

## Wiring standard specification

### TIA/EIA 568

## Twisted pair wiring types

### Cat (category) 3, 4, 5, 5e, 6, and 6e, Cat 7

### CAT 5 most often used in modern LANs

## Advantages

### Relatively inexpensive

### Flexible

### Easy installation

### Spans significant distance before requiring repeater

### Accommodates several different topologies

### Handles current faster networking transmission rates

## Two categories

Figure 3-20 STP cable

STP (shielded twisted pair)

### UTP (unshielded twisted pair)

# STP (Shielded Twisted Pair)

## Individually insulated

## Surrounded by metallic substance shielding (foil)

### Barrier to external electromagnetic forces

### Contains electrical energy of signals inside

Figure 3-21 UTP cable

May be grounded

# UTP (Unshielded Twisted Pair)

## One or more insulated wire pairs

### Encased in plastic sheath

### No additional shielding

#### Less expensive, less noise resistance

## EIA/TIA standards

### Cat 3 (Category 3)

### Cat 4 (Category 4)

### Cat 5 (Category 5)

### Cat 5e (Enhanced Category 5)

### Cat 6 (Category 6)

### Cat 6e (Enhanced Category 6)

### Cat 7 (Category 7)

Figure 3-22 A Cat 5 UTP cable with pairs untwisted

# Comparing STP and UTP

Figure 3-23 RJ-45 and RJ-11 connectors

Throughput

### STP and UTP transmit the same rates

## Cost

### STP and UTP vary

## Noise immunity

### STP more noise resistant

### UTP subject to techniques to offset noise

## Size and scalability

### STP and UTP maximum segment length

#### 100 meters

## Connector

### STP and UTP use RJ-45 (Registered Jack 45)

### Telephone connections use RJ-11 (Registered Jack 11)

# Terminating Twisted Pair Cable

## Patch cable

### Relatively short cable

### Connectors at both ends

## Proper cable termination techniques

### Basic requirement for two nodes to communicate

## Poor terminations

### Lead to loss or noise

## TIA/EIA standards

### TIA/EIA 568A

### TIA/EIA 568B

## Straight-through cable

### Terminate RJ-45 plugs at both ends identically

Figure 3-26 RJ-45 terminations on a crossover cable

## Crossover cable

### Transmit and receive wires on one end reversed

## Termination tools

### Wire cutter

### Wire stripper

### Crimping tool

## After making cables

### Verify data transmit and receive

Fiber-Optic Cable

## Fiber-optic cable (fiber)

### One (or several) glass or plastic fibers at its center (core)

## Data transmission

### Pulsing light sent from laser

### LED (light-emitting diode) through central fibers

## Cladding

### Layer of glass or plastic surrounding fibers

### Different density from glass or plastic in strands

### Reflects light back to core

#### Allows fiber to bend

## Plastic buffer

### Outside cladding

### Protects cladding and core

### Opaque

#### Absorbs any escaping light

## Kevlar strands (polymeric fiber) surround plastic buffer

Figure 3-30 A fiber-optic cable

Plastic sheath covers Kevlar strands

## Different varieties

### Based on intended use and manufacturer

## Two categories

### Single-mode

### Multimode

# SMF (Single-Mode Fiber)

## Uses narrow core (< 10 microns in diameter)

### Laser generated light travels over one path

#### Little reflection

### Light does not disperse

## Accommodates

### Highest bandwidths, longest distances

### Connects carrier’s two facilities

## Costs prohibit typical LANs, WANs use

Figure 3-31 Transmission over single-mode fiber-optic cable

# MMF (Multimode Fiber)

## Uses core with larger diameter than single-mode fiber

### Common size: 62.5 microns

## Laser or LED generated light pulses travel at different angles

## Common uses

### Cables connecting router to a switch

### Cables connecting server on network backbone

Figure 3-32 Transmission over multimode fiber-optic cable

## Benefits

### Extremely high throughput

### Very high resistance to noise

### Excellent security

### Ability to carry signals for much longer distances before requiring repeaters than copper cable

### Industry standard for high-speed networking

## Drawback

### More expensive than twisted pair cable

### Requires special equipment to splice

## Throughput

### Reliable transmission rates

#### Can reach 100 gigabits (or 100,000 megabits) per second per channel (but only for singlemode, not multimode)

## Cost

### Most expensive transmission medium

## Connectors

### ST (straight tip)

### SC (subscriber connector or standard connector)

### LC (local connector)

### MT-RJ (mechanical transfer registered jack)

## Noise immunity

### Unaffected by EMI

## Size and scalability

### Segment lengths vary

#### 150 to 40,000 meters

#### Due primarily to optical loss

DTE (Data Terminal Equipment) and DCE (Data Circuit-Terminating Equipment) Connector Cables

## DTE (data terminal equipment)

### Any end-user device

## DCE (data circuit-terminating equipment)

### Device that processes signals

### Supplies synchronization clock signal

## DTE and DCE connections

### Serial

#### Pulses flow along single transmission line

#### Sequentially

### Serial cable

#### Carries serial transmissions

## RS-232 (Recommended Standard 232)

### EIA/TIA standard

### Physical layer specification

#### Signal voltage, timing, compatible interface characteristics

### Connector types

#### RJ-45 connectors, DB-9 connectors, DB-25 connectors

## RS-232 used between PC and router today

## RS-232 connections

### Straight-through, crossover, rollover

Structured Cabling

## Cable plant

### Hardware making up enterprise-wide cabling system

## Standard

### TIA/EIA joint 568 Commercial Building Wiring Standard

Figure 3-39 TIA/EIA structured cabling in an enterprise

## Components

### Entrance facilities

### MDF (main distribution frame)

### Cross-connect facilities

### IDF (intermediate distribution frame)

### Backbone wiring

### Telecommunications closet

### Horizontal wiring

### Work area

Figure 3-40 TIA/EIA structured cabling in a building

Table 3-2 TIA/EIA specifications for backbone cabling

Figure 3-45 A typical UTP cabling installation

Best Practices for Cable Installation and Management

## Choosing correct cabling

### Follow manufacturers’ installation guidelines

### Follow TIA/EIA standards

## Network problems

### Often traced to poor cable installation techniques

## Installation tips to prevent Physical layer failures

Last modified 12-14-09