# Objectives

## Identify and explain the functions of the core TCP/IP protocols

## Explain how the TCP/IP protocols correlate to layers of the OSI model

## Discuss addressing schemes for TCP/IP in IPv4 and IPv6 protocols

## Describe the purpose and implementation of DNS (Domain Name System) and DHCP (Dynamic Host Configuration Protocol)

## Identify the well-known ports for key TCP/IP services

## Describe common Application layer TCP/IP protocols

Characteristics of TCP/IP (Transmission Control Protocol/ Internet Protocol)

## Protocol Suite

### “TCP/IP”

### Subprotocols

## TCP, IP, UDP, ARP

## Developed by Department of Defense

### ARPANET (1960s)

## Internet precursor

## Popularity

### Low cost

### Communicates between dissimilar platforms

### Open nature

### Routable

## Spans more than one LAN (LAN segment)

### Flexible

## Runs on combinations of network operating systems or network media

## Disadvantage: requires more configuration

The TCP/IP Core Protocols

## TCP/IP suite subprotocols

## Operates in Transport or Network layers of OSI model

## Provide basic services to protocols in other layers

## Most significant protocols in TCP/IP

### TCP

### IP

# TCP (Transmission Control Protocol)

## Transport layer protocol

## Provides reliable data delivery services

### Connection-oriented subprotocol

## Establish connection before transmitting, with the TCP Handshake

### Sequencing and checksums

### Flow control

## Transmitter waits for ACK before sending more

## TCP segment format

### Encapsulated by IP datagram in Network layer

## Becomes IP datagram’s “data”

# TCP Segment

# Important TCP Header Fields

## Flags, especially SYN and ACK

### Indicates purpose of segment

## Source Port and Destination Port

### Guides data to the correct process on the destination computer

## SEQ number and ACK number

### Used to arrange segments in the correct order

# TCP Handshake

## Computer A sends SYN to Computer B

### SYN flag set

## SEQ field: Random initial sequence number (ISN)

## ACK field: Empty (zeroes)

## Computer B replies with SYN/ACK

### SYN and ACK flags set

## SEQ field: Computer B's random initial sequence number (ISN)

## ACK field: Computer A's ISN plus 1

## Computer A responds with ACK

### ACK flag set

## SEQ field: Computer A's ISN plus 1

## ACK field: Computer B's ISN plus 1

# Ending a TCP Session

## FIN flag indicates transmission end

# Wireshark Demonstration

## Relative SEQ and ACK numbers at top

## Absolute SEQ and ACK values at bottom, in hexadecimal

# UDP (User Datagram Protocol)

## Transport layer protocol

## Provides unreliable data delivery services

## Connectionless transport service

## No assurance packets received in correct sequence

## No guarantee packets received at all

## No error checking, sequencing

## Lacks sophistication

## More efficient than TCP

## Useful situations

## Great volume of data transferred quickly

# IP (Internet Protocol)

## Network layer protocol

### Routes packets using IP addresses

## Enables TCP/IP to internetwork

### Routers move IP packets move from one network to another

## Unreliable, connectionless protocol

### No guaranteed data delivery, no handshake

## Some higher level protocols provide reliability, like TCP

# Important IP Header Fields

## TTL (Time to Live)

### Decreases by one for each router the packet passes through (a "hop")

### When TTL reaches zero, the packet is discarded

## Source Destination IP Addresses

### Used to deliver packet and response

# ICMP (Internet Control Message Protocol)

## Network layer protocol

### Reports on data delivery success/failure

## Announces transmission failures to sender

### Network congestion

### Data fails to reach destination

### Data discarded: TTL expired

## ICMP cannot correct errors

### Provides critical network problem troubleshooting information

# IGMP (Internet Group Management Protocol)

## Network layer protocol

## Manages multicasting

### Allows one node to send data to defined group of nodes

## Uses

### Internet teleconferencing

### Routers sending traffic reports to each other

# ARP (Address Resolution Protocol)

## Network layer protocol

## Obtains a MAC address from an IP address

## ARP table (ARP cache)

### Computers store recently-used MAC-to-IP address mappings

### Increases efficiency

## Controlled by ARP command

# ARP Demonstration

## ARP -D *

### Clears the ARP cache

## ARP -A

### Shows the ARP cache

## S

# RARP (Reverse Address Resolution Protocol)

## Converts MAC address to IP Address

### Obsolete—replaced by DHCP

IPv4 Addressing

# IPv4 Addressing

## Networks recognize two addresses

### Logical (Network layer)

### Physical (MAC, hardware) addresses

## IP protocol handles logical addressing

## Specific parameters

### Unique 32-bit number

## Divided into four octets (sets of eight bits)

## Separated by periods

## Example: 144.92.43.178

## IP address information

### Network Class determined by first octet

## Class A, Class B, Class C

## Class D, Class E rarely used (never assign)

### Class D: value between 224 and 230

## Multicasting

### Class E: value between 240 and 254

## Experimental use

## Eight bits have 256 combinations

### Networks use 1 through 254

### 0: reserved as placeholder

## 10.0.0.0

### 255: reserved for broadcast transmission

## 255.255.255.255

## Class A devices

### Share same first octet (bits 0-7)

## Network ID

### Host: second through fourth octets (bits 8-31)

## Class B devices

### Share same first two octet (bits 0-15)

### Host: second through fourth octets (bits 16-31)

## Class C devices

### Share same first three octet (bits 0-23)

### Host: second through fourth octets (bits 24-31)

## Running out of addresses

### IPv6 incorporates new addressing scheme

## Loop back address

### First octet equals 127 (127.0.0.1)

## Loopback test

### Attempting to connect to own machine

### Powerful troubleshooting tool

## Windows XP, Vista

### ipconfig command

## Unix, Linux

### ifconfig command

# Binary and Dotted Decimal Notation

## Decimal number between 0 and 255 represents each binary octet

## Period (dot) separates each decimal

## Dotted decimal address has binary equivalent

### Converting each octet

### Remove decimal points

# Subnet Mask

## Identifies every device on TCP/IP-based network

## 32-bit number (net mask)

### Identifies device’s subnet

## Combines with device IP address

## Informs network about segment, network where device attached

## Four octets (32 bits)

### Expressed in binary or dotted decimal notation

## Assigned same way as IP addresses

### Manually, or automatically (via DHCP)

# Subnetting

### Subdividing network single class into multiple, smaller logical networks (segments)

## Control network traffic

## Make best use of limited number of IP addresses

### Subnet mask varies depending on subnetting

## Nonsubnetted networks use defaults

Assigning IP Addresses

## Government-sponsored organizations

### Dole out IP address blocks to companies

### IANA, ICANN, RIRs

## Companies, individuals

### Obtain IP addresses from ISPs

## Every network node must have unique IP address

### Otherwise it cannot send or receive Internet packets

# Static and Automatic IP Address Assignment

## Static IP address

### Manually typed into each device

### Modify client workstation TCP/IP properties

## Only way to change

### Human error cause duplicates

## Automatic IP addressing

### BOOTP and DHCP

### Reduce duplication error

# BOOTP (Bootstrap Protocol)

## Mid-1980s

## Application layer protocol

## Central list

### IP addresses, associated devices’ MAC addresses

### Assign client IP addresses dynamically

## Dynamic IP address

### Assigned to device upon request

### Changeable

## BOOTP process

### Client connects to network

### Sends broadcast message asking for IP address

## Includes client’s NIC MAC address

### BOOTP server looks up client’s MAC address in BOOTP table

### Responds to client

## Client’s IP address

## Server IP address

## Server host name

## Default router IP address

## Process resembles RARP

### Difference

## RARP requests, responses not routable

## RARP only capable of issuing IP address to client

## BOOTP may issue additional information (client’s subnet mask)

## BOOTP surpassed by DHCP (Dynamic Host Configuration Protocol)

### More sophisticated IP addressing utility

### DHCP requires little intervention

## BOOTP difficult to maintain on large networks

# DHCP (Dynamic Host Configuration Protocol)

## Assigns network device unique IP address

### Automatically

## Application layer protocol

## Developed by IETF (BOOTP replacement)

## Operation

### Similar to BOOTP

### Lower administrative burden

## Administrator does not maintain table

### Requires DHCP service on DHCP server

# Reasons to Use DHCP

## Saves time spent assigning IP addresses

## Prevents accidental duplicate IP addresses

## Allows users to move devices (like laptops) without having to change their TCP/IP configuration

# DHCP Leasing Process

## Device borrows (leases) IP address

### Devices use IP address temporarily

## Specified time limit

## Lease time

### Determine when client obtains IP address at log on

### User may force lease termination

## DHCP service configuration

### Specify leased address range

### Configure lease duration

## Several steps to negotiate client’s first lease

# Terminating a DHCP Lease

## Lease expiration

### Automatic

## Established in server configuration

### Manually terminated at any time

## Client’s TCP/IP configuration

## Server’s DHCP configuration

## Circumstances requiring lease termination

### DHCP server fails and replaced

## Windows: release of TCP/IP settings

## DHCP services run on several server types

### Installation and configurations vary

# APIPA (Automatic Private IP Addressing)

## Client cannot communicate without valid IP address

## What if DHCP server not running?

### Microsoft Windows offers Automatic Private IP Addressing

## Provides IP address automatically

## IANA (Internet Assigned Numbers Authority) reserved predefined pool of addresses

### 169.254.0.0 through 169.254.255.255

## APIPA

### Assigns a random IP address from the 169.254.y.x range

### Assigns default Class B subnet mask

## 255.255.0.0

## Disadvantage

### Computer only communicates with other nodes using addresses in APIPA range

### Cannot normally connect to the Internet with a 169.254.y.z address

## APIPA suitable use

### Small networks: no DHCP servers

## APIPA unsuitable use

### Networks communicating with other subnets, WAN

## APIPA enabled by default: OK

### First checks for DHCP server

## Allows DHCP server to assign addresses

### Does not reassign new address if static

### Works with DHCP clients

### Disabled in registry

IPv6 Addressing

## IP next generation (IPng)

### Replacing IPv4 (gradually)

## IPv6 support

### Most new applications, servers, network devices

## Delay in implementation

### Cost of upgrading infrastructure

## IPv6 advantages

### More efficient header, better security, better prioritization provisions, automatic IP address configuration

### Billions of additional IP addresses

## Difference between IPv4 and IPv6 addresses

### Size

## IPv4: 32 bits

## IPv6: eight 16-bit fields (128 bits)

## IPv6: 296 (4 billion times 4 billion times 4 billion) available IP addresses

### Representation

## IPv4: binary numbers separated by period

## IPv6: hexadecimal numbers separated by colon

## IPv6 shorthand: “::” any number of multiple, zero-value fields

# Demo: IPv6 Addresses in Windows 7

## US Government requires IPv6 compatibility on its devices now

### Links Ch 4f, 4g

## Difference between IPv4 and IPv6 addresses (cont’d.)

### Representation (cont’d.)

## IPv6 loopback address is 0:0:0:0:0:0:0:1

## Abbreviated loopback address ::1

### Scope

## IPv6 addresses can reflect scope of transmission’s recipients

## Unicast address represents single device interface

## Multicast address represents multiple interfaces (often on multiple devices)

## Difference between IPv4 and IPv6 addresses (cont’d.)

### Scope (cont’d.)

## Anycast address represents any one interface from a group of interfaces

## Any one can accept transmission

### Format Prefix (IPv6)

## Beginning of address

## Variable-length field

## Indicates address type: unicast, multicast, anycast

Sockets and Ports

## Processes assigned unique port numbers

## Process’s socket

### Port number plus host machine’s IP address

## Port numbers

### Simplify TCP/IP communications

### Ensures data transmitted correctly

## Example

### Telnet port number: 23

### IPv4 host address: 10.43.3.87

### Socket address: 10.43.3.87:23

## Port number range: 0 to 65535

## Three types

### Well Known Ports

## Range: 0 to 1023

## Operating system or administrator use

### Registered Ports

## Range: 1024 to 49151

## Network users, processes with no special privileges

### Dynamic and/or Private Ports

## Range: 49152 through 65535

## No restrictions

# Using Non-Standard Ports

## A server could be configured to use an unusual port, such as a Web server on port 8080

## Not good idea: standards violation

### Sometimes done for security or testing

Host Names and DNS (Domain Name System)

## TCP/IP addressing

### Long, complicated numbers

### Good for computers

## People remember words better

### Internet authorities established Internet node naming system

## Host

### Internet device

## Host name

### Name describing device

# Domain Names

## Domain

### Group of computers belonging to same organization

### Share common part of IP address

## Domain name

### Identifies domain (loc.gov)

### Associated with company, university, government organization

## Fully qualified host name (jasmine.loc.gov)

### Local host name plus domain name

## Label (character string)

### Separated by dots

### Represents level in domain naming hierarchy

## Example: www.google.com

### Top-level domain (TLD): com

### Second-level domain: google

### Third-level domain: www

## Second-level domain

### May contain multiple third-level domains

## ICANN established domain naming conventions

## ICANN approved over 240 country codes

## Host and domain names restrictions

### Any alphanumeric combination up to 63 characters

### Include hyphens, underscores, periods in name

### No other special characters

# Hosts Files

## ARPAnet used HOSTS.TXT file

### Associated host names with IP addresses

### Host matched by one line

## Identifies host’s name, IP address

## Alias provides nickname

## UNIX-/Linux-based computer

### Host file called hosts, located in the /etc directory

## Windows 9x, NT, 2000, XP, Vista computer

### Host file called hosts

### Located in %systemroot%\system32\drivers\etc folder

# Windows Hosts File

## Rarely used, but still present

# DNS (Domain Name System)

## Hierarchical Distributed Database

### Associates domain names with IP addresses

## DNS refers to:

### Application layer service accomplishing association

### Organized system of computers; databases making association possible

## DNS redundancy

### Many computers across globe related in hierarchical manner

### Root servers

## 13 computers (ultimate authorities)

# Name servers (DNS servers)

## Servers that contain databases of associated names, IP addresses

## Provide information on request

## To convert names like www.ccsf.edu into IP addresses like 147.144.1.212

## This process is called name resolution

## Resource record

### Describes one piece of DNS database information

### Many different types

## Dependent on function

### Contents

## Name field

## Type field

## Class field

## Time to Live field

## Data length field

## Actual data

# Demo: CCSF’s Name Servers

# Configuring DNS

## Large organizations

### Often maintain two name servers

## Primary and secondary

### Ensures Internet connectivity

## Each device must know how to find server

### Automatically by DHCP

### Manually configure workstation TCP/IP properties

# DDNS (Dynamic DNS)

## Allows a user to host a Web site on a computer with a dynamic IP address

## Process

### Service provider runs program on user’s computer

## Notifies service provider when IP address changes

### Service provider’s server launches routine to automatically update DNS record

## Effective throughout Internet in minutes

## Not as good as a real static IP address

## Larger organizations pay for statically assigned IP address

Application Layer Protocols

## Work over TCP or UDP plus IP

### Translate user requests

## Into format readable by network

## HTTP

### Application layer protocol central to using Web

## BOOTP and DHCP

### Automatic address assignment

# Telnet

## Terminal emulation protocol

### Log on to remote hosts

## Using TCP/IP protocol suite

### TCP connection established

## Keystrokes on user’s machine act like keystrokes on remotely connected machine

## Often connects two dissimilar systems

## Can control remote host

## Drawback

### Notoriously insecure

# FTP (File Transfer Protocol)

## Send and receive files via TCP/IP

## Host running FTP server portion

### Accepts commands from host running FTP client

## FTP commands

### Operating system’s command prompt

## No special client software required

## FTP hosts allow anonymous logons

## After connected to host

### Additional commands available

### Type help

## Graphical FTP clients

### MacFTP, WS_FTP, CuteFTP, SmartFTP

## Rendered command-line method less common

## FTP file transfers directly from modern Web browser

### Point browser to FTP host

### Move through directories, exchange files

## SFTP

### More secure

# TFTP (Trivial File Transfer Protocol)

## Enables file transfers between computers

### Simpler (more trivial) than FTP

## TFTP relies on Transport layer UDP

### Connectionless

### Does not guarantee reliable data delivery

## No ID and password required

### Security risk

## No directory browsing allowed

## Useful to load data, programs on diskless workstation

## Used to put software on IP phones and routers

# NTP (Network Time Protocol)

## Synchronizes network computer clocks

## Depends on UDP Transport layer services

### Benefits from UDP’s quick, connectionless nature

## Time sensitive

## Cannot wait for error checking

## Time synchronization importance

### Routing

### Time-stamped security methods

### Maintaining accuracy, consistency between multiple storage systems

# NNTP (Network News Transfer Protocol)

## Facilitates newsgroup messages exchange

### Between multiple servers, users

## Similar to e-mail

### Provides means of conveying messages

## Differs from e-mail

### Distributes messages to wide group of users at once

## User subscribes to newsgroup server host

## News servers

### Central collection, distribution point for newsgroup messages

# PING (Packet Internet Groper)

## Provides verification

### TCP/IP installed, bound to NIC, configured correctly, communicating with network

### Host responding

## Uses ICMP services

### Send echo request and echo reply messages

## Determine IP address validity

## Ping IP address or host name

## Ping loopback address: 127.0.0.1

### Determine if workstation’s TCP/IP services running

## Operating system determines Ping command options, switches, syntax

Last modified 9-14-09