# Objectives

## Describe the basic and hybrid LAN physical topologies, and their uses, advantages, and disadvantages

## Describe the backbone structures that form the foundation for most LANs

## Understand the transmission methods underlying Ethernet networks

## Compare the different types of switching used in data transmission

Topologies

Simple Physical Topologies

## Physical topology

### Physical network nodes layout

### Depicts broad scope

### Does not specify:

#### Device types

#### Connectivity methods

#### Addressing schemes

### Fundamental shapes

#### Bus, ring, star

#### Hybrid

# Bus topology

### Bus

#### Single cable

#### Connecting all network nodes

#### No intervening connectivity devices

### One shared communication channel

### Physical medium

#### Coaxial cable

### Passive topology

#### Node listens for, accepts data

#### Use broadcast to send

### Broadcast domain

#### Node communicates using broadcast transmission

### Terminators

#### 50-ohm resistors

#### Stops signal at end of wire

### Signal bounce

#### Signals echo between two network ends

### One end grounded

#### Removes static electricity

## Advantages

### Relatively inexpensive

## Disadvantage

### Does not scale well

#### All nodes share a fixed amount of total bandwidth

### Difficult to troubleshoot

### Not very fault tolerant

# Ring

## Ring topology

### Node connects to nearest two nodes

### Circular network

### Clockwise data transmission

#### One direction (unidirectional) around ring

### Active topology

#### Workstation participates in data delivery

#### Data stops at destination

### Physical medium

#### Twisted pair or fiber-optic cabling

## Drawback

### Malfunctioning workstation can disable network

### Not flexible or scalable

# Star

## Star topology

### Node connects through central device

#### Hub, router, or switch

### Physical medium

#### Twisted pair or fiber-optic cabling

### Single cable connects two devices

### Require more cabling, configuration

## Advantage

### Fault tolerance

#### Centralized connection point can be a single point of failure, however

### Scalable

## Most popular fundamental layout

### Ethernet networks based on star topology

## 1024 addressable logical network nodes maximum

Logical Topologies

## Describes data transmission between nodes

## Most common: bus, ring

## Bus logical topology

### Signals travel from one device to all other devices

### May or may not travel through intervening connectivity device

### Bus logical topology used by networks with:

#### Physical bus topology

#### Star, star-wired bus topology

### Ethernet

## Ring logical topology

### Signals follow circular path

### Ring logical topology used by networks with:

#### Pure ring topology

#### Star-wired ring hybrid physical topology

### Token ring

# Hybrid Physical Topologies

## Pure bus, ring, star topologies

### Rarely exist

#### Too restrictive

## Hybrid topology

### More likely

### Complex combination of pure topologies

### Several options

# Star-Wired Ring

## Star-wired ring topology

### Star physical topology

### Ring logical topology

## Benefit

### Star fault tolerance

## Network use

### Token Ring networks

#### IEEE 802.5

# Star-Wired Bus

## Star-wired bus topology

### Workstation groups

### Star-connected devices

### Networked via single bus

## Advantage

### Cover longer distances

### Easily interconnect, isolate different segments

## Drawback

### Cabling, connectivity device expense

## Basis for modern Ethernet networks

Backbone Networks

## Cabling connecting hubs, switches, routers

## More throughput

## Large organizations

### Fiber-optic backbone

### Cat 5 or better for hubs, switches

## Enterprise-wide network backbones

### Complex, difficult to plan

## Enterprise

### Entire organization

### Significant building block: backbone

# Serial Backbone

## Simplest backbone

### Two or more internetworking devices

### Connect using single daisy-chain cable

## Daisy-chain

### Linked series of devices

## Benefit

### Logical growth solution

#### Modular additions

### Low-cost LAN infrastructure expansion

#### Easily attach hubs

## Backbone components

### Hubs, gateways, routers, switches, bridges

## Serial connection of repeating devices

### Essential for distance communication

## Standards

### Define number of hubs allowed

### Exceed standards

#### Intermittent, unpredictable data transmission errors

# Distributed Backbone

## Connectivity devices

### Connected to a hierarchy of central connectivity devices

## Benefit

### Simple expansion, limited capital outlay

## More complicated distributed backbone

### Connects multiple LANs, LAN segments

#### Using routers

## More benefits

### Workgroup segregation

### May include daisy-chain linked hubs

#### Consider length

## Drawback

### Potential for single failure points

# Collapsed Backbone

## Uses router or switch

### Single central connection point for multiple subnetworks

## Highest layer

### Router with multiprocessors

## Central router failure risk

## Routers may slow data transmission

## Advantages

### Interconnect different subnetwork types

### Central management

# Parallel Backbone

## Most robust network backbone

## More than one central router, switch

### Connects to each network segment

## Requires duplicate connections between connectivity devices

## Advantage

### Redundant links

### Increased performance

### Better fault tolerance

## Disadvantage

### More cabling

## Used to connect most critical devices

Switching

# Switching

## Logical network topology component

## Determines connection creation between nodes

## Three methods

### Circuit switching

### Message switching

### Packet switching

# Circuit Switching

## Connection established between two network nodes

### Before transmitting data

## Dedicated bandwidth

## Data follows same initial path selected by switch

## Monopolizes bandwidth while connected

### Resources are wasted

## Uses

### Old analog phone calls

### Live audio, videoconferencing

### Home modem connecting to ISP

# Message Switching

## Connection established between two devices

### Data transferred then connection broken

### Information stored and forwarded in second device

## Repeat store and forward routine

### Until destination reached

## All information follows same physical path

## Connection not continuously maintained

## Device requirements

### Sufficient memory, processing power

## Not  commonly used in modern networks, but will be used in space

### See link Ch 5a, Delay-Tolerant Networking (DTN)

# Packet Switching

## Most popular, used by Ethernet and the Internet

## Breaks data into packets before transporting

## Packets

### Travel any network path to destination

### Find fastest circuit available at any instant

### Need not follow the same path

### Need not arrive in sequence

### Reassembled at destination

## Requires speedy connections for live audio, video transmission

## Advantages

### No wasted bandwidth

### Devices do not process information

# MPLS (Multiprotocol Label Switching)

## IETF

### Introduced in 1999

## Multiple layer 3 protocols

### Travel over any one of several connection-oriented layer 2 protocols

## Supports IP

## Common use

### Layer 2 WAN protocols

## Advantages

### Use packet-switched technologies over traditionally circuit switched networks

### Create end-to-end paths

#### Act like circuit-switched connections

### Addresses traditional packet switching limitations

### Better QoS (quality of service)

# Flow Routing ( not in textbook)

## New technique connects streams of related packets so they all travel along the same path

## No need to make routing decisions after the first packet

## Saves time and power (link Ch 5c)

Ethernet

# Ethernet

## Developed by Xerox: 1970s

### Improved by:

#### Digital Equipment Corporation (DEC), Intel, Xerox (DIX)

## Benefits

### Flexible

### Excellent throughput

#### Reasonable cost

## Popular network technology

## All variations share common access method

#### CSMA/CD

CSMA/CD (Carrier Sense Multiple Access with Collision Detection)

## Network access method

### Controls how nodes access communications channel

### Necessary to share finite bandwidth

## Carrier sense

### Ethernet NICs listen, wait until free channel detected

## Multiple access

### Ethernet nodes simultaneously monitor traffic, access media

## Collision

### Two nodes simultaneously:

#### Check channel, determine it is free, begin transmission

## Collision detection

### Nodes detect unusually high voltages

#### Perform collision detection routine

### Jamming

#### NIC issues 32-bit sequence

#### Indicates previous message faulty

### Retransmit data

## Heavily trafficked network segments

### Collisions common

## Segment growth

### Performance suffers

### “Critical mass” number dependencies

#### Data type and volume regularly transmitted

## Collisions corrupt data, truncate data frames

### Network must compensate for them

## Collision domain

### Portion of network where collisions occur

## Ethernet network design

### Repeaters repeat collisions

#### Result in larger collision domain

### Switches and routers

#### Separate collision domains

## Collision domains differ from broadcast domains

## Ethernet cabling distance limitations

### Affected by collision domains

## Data propagation delay

### Time for data to travel

#### From one segment point to another point

### Too long

#### Cannot identify collisions accurately

### 100 Mbps networks

#### Three segment maximum connected with two hubs

### 10 Mbps buses

#### Five segment maximum connected with four hubs

Ethernet Standards for Copper Cable

## IEEE Physical layer standards

### Specify how signals transmit to media

### Differ significantly in signal encoding

#### Affect maximum throughput, segment length, wiring requirements

# 10Base-T

### 10 represents maximum throughput: 10 Mbps

### Base indicates baseband transmission

### T stands for twisted pair

### Two pairs of wires: transmit and receive

#### Full-duplex transmission

### Follows 5-4-3 rule of networking

#### Five network segments

#### Four repeating devices

#### Three populated segments maximum

# 100Base-T (Fast Ethernet)

### IEEE 802.3u standard

### Similarities with 10Base-T

#### Baseband transmission, star topology, RJ-45 connectors

### Supports three network segments maximum

#### Connected with two repeating devices

#### 100 meter segment length limit between nodes

### 100Base-TX

#### 100-Mbps throughput over twisted pair

#### Full-duplex transmission: doubles effective bandwidth

# 1000Base-T (Gigabit Ethernet)

### IEEE 802.3ab standard

### 1000 represents 1000 Mbps

### Base indicates baseband transmission

### T indicates twisted pair wiring

### Four pairs of wires in Cat 5 or higher cable

#### Transmit and receive signals

### Data encoding scheme: different from 100Base-T

### Standards can be combined

### Maximum segment length: 100 meters, one repeater

# 10GBase-T

### IEEE 802.3an

### Pushing limits of twisted pair

#### Requires Cat 6 or Cat 7 cabling

#### Maximum segment length: 100 meters

### Benefit

#### Very fast data transmission, lower cost than fiber-optic

### Use

#### Connect network devices

#### Connect servers, workstations to LAN

Ethernet Standards for Fiber-Optic Cable

## 100Base-FX (Fast Ethernet)

### 100-Mbps throughput, broadband, fiber-optic cabling

#### Multimode fiber containing: at least two strands

### Half-duplex mode

#### One strand receives, one strand transmits

#### 412 meters segment length

### Full duplex-mode

#### Both strands send and receive

#### 2000 meters segment length

### One repeater maximum

### IEEE 802.3u standard

## 1000Base-LX (1-Gigabit Ethernet)

### IEEE 802.3z standard

### 1000: 1000-Mbps throughput

### Base: baseband transmission

### LX: reliance on 1300 nanometers wavelengths

### Longer reach than any other 1-gigabit technology

### Single-mode fiber: 5000 meters maximum segment

### Multimode fiber: 550 meters maximum segment

### One repeater between segments

### Excellent choice for long backbones

## 1000Base-SX (1-Gigabit Ethernet)

### IEEE 802.3z standard

### Differences over 1000Base-LX

#### Multimode fiber-optic cable (installation less expensive)

#### Uses short wavelengths (850 nanometers)

### Maximum segment length dependencies

#### Fiber diameter, modal bandwidth used to transmit signals

## 1000Base-SX (1-Gigabit Ethernet) (cont’d.)

### Modal bandwidth measurement

#### Highest frequency of multimode fiber signal (over specific distance)

#### MHz-km

#### Higher modal bandwidth, multimode fiber caries signal reliably longer

### 50 micron fibers: 550 meter maximum length

### 62.5 micron fibers: 275 meter maximum length

### One repeater between segments

### Best suited for shorter network runs

# 10-Gigabit Fiber-Optic Standards

## Extraordinary potential for fiber-optic cable

### Pushing limits

## 802.3ae standard

### Fiber-optic Ethernet networks

### Transmitting data at 10 Gbps

### Several variations

### Common characteristics

#### Star topology, allow one repeater, full-duplex mode

### Differences

#### Signal’s light wavelength, maximum allowable segment length

10-Gigabit Fiber-Optic Standards

# 10GBase-SR and 10GBase-SW

### 10G: 10 Gbps

### Base: baseband transmission

### S: short reach

### Physical layer encoding

#### R works with LAN fiber connections

#### W works with SONET fiber connections

### Multimode fiber: 850 nanometer signal transmission

### Maximum segment length

#### Depends on fiber diameter

# 10GBase-LR and 10GBase-LW

### 10G: 10 Gbps

### Base: baseband transmission

### L: long reach

### Single-mode fiber: 1319 nanometer signal transmission

### Maximum segment length

#### 10,000 meters

### 10GBase-LR: WAN or MAN

### 10GBase-LW: SONET WAN links

# 10GBase-ER and 10GBase-EW

### E: extended reach

### Single-mode fiber

#### Transmit signals with 1550 nanometer wavelengths

### Longest fiber-optic segment reach

#### 40,000 meters (25 miles)

### 10GBase-EW

#### Encoding for SONET

### Best suited for WAN use

Ethernet Frames

## Four types

### Ethernet_802.2 (Raw)

### Ethernet_802.3 (Novell proprietary)

### Ethernet_II (DIX)

### Ethernet_SNAP

## Frame types differ slightly

### Coding and decoding packets

## No relation to topology, cabling characteristics

## Framing

### Independent of higher-level layers

## Using and Configuring Frames

### Ensure all devices use same, correct frame type

#### Node communication

### Ethernet_II used today in almost all networks

### Frame type configuration

#### Through NIC configuration software

#### NIC autodetect, autosense

### Importance

#### Know frame type for troubleshooting

## Frame Fields

### Common fields

#### 7-byte preamble, 1-byte start-of-frame delimiter

#### SFD (start-of-frame delimiter) identifies where data field begins

#### 14-byte header

#### 4-byte FCS (Frame Check Sequence)

#### Frame size range: 64 to 1518 total bytes

### Larger frame sizes result in faster throughput

### Improve network performance

#### Properly manage frames

## Ethernet_II (DIX)

### Developed by DEC, Intel, Xerox (abbreviated DIX)

#### Before IEEE

### Contains 2-byte type field

#### Identifies the Network layer protocol

### Ethernet_SNAP frame type

#### Provides type field

#### Calls for additional control fields

#### Less room for data

### Most commonly used on contemporary Ethernet networks

# PoE (Power over Ethernet)

## IEEE 802.3af standard

### Supplying electrical power over Ethernet connections

## Two device types

### PSE (power sourcing equipment)

### PDs (powered devices)

## Requires Cat 5 or better copper cable

## Connectivity devices must support PoE

## Compatible with current 802.3 installations

Last modified 9-22-09