# Objectives

## Identify the functions of LAN connectivity hardware

## Install, configure, and differentiate between network devices such as, NICs, hubs, bridges, switches, routers, and gateways

## Explain the advanced features of a switch and understand popular switching techniques, including VLAN management

## Explain the purposes and properties of routing

## Describe common IPv4 and IPv6 routing protocols

NICs (Network Interface Cards)

# NICs (Network Interface Cards)

## Connectivity devices

### Enable device transmission

### Transceiver

#### Transmits and receives data

## Physical layer and Data Link layer functions

### Issue data signals

### Assemble and disassemble data frames

### Interpret physical addressing information

### Determine right to transmit data

## Smart hardware

### Perform prioritization (link Ch 6a)

### Network management

### Buffering

### Traffic-filtering (link Ch 6b)

## Do not analyze information at layers 3 through 7

## Importance

### Common to every networking device, network

# Types of NICs

## Before ordering or installing NIC

### Know device interface type

## Types of NICs

### Access method (Ethernet or Token Ring)

### Network transmission speed

### Connector interfaces

### Compatible motherboard or device type

### Manufacturer

## Bus

### Circuit, signaling pathway

### Motherboard uses to transmit data to computer’s components

#### Memory, processor, hard disk, NIC

### Differ according to capacity

#### Defined by data path width and clock speed

### Data path size

#### Parallel bits transmitting at any given time

#### Proportional to attached device’s speed

# Internal Bus Standards

## Expansion slots

### Multiple electrical contacts on motherboard

### Allows bus expansion

## Expansion card (expansion board)

Figure 6-1 PCI NIC

Circuit board for additional devices

### Inserts into expansion slot, establishes electrical connection

### Device connects to computer’s main circuit or bus

### Computer centrally controls device

## Multiple bus types

### PCI bus: most popular expansion board NIC

## PCI (Peripheral Component Interconnect)

### 32- or 64-bit bus

### Clock speeds rated at 33-, 66- or 133-MHz

### Maximum data transfer rate: 1 Gbps

### Introduced by Intel (1992)

Figure 6-2 PCIe NIC

Latest official version: 3.0 (2004)

## ISA (Industry Standard Architecture)

### Original PC bus type (early 1980s)

#### Support for 8-bit and 16-bit data path, 4.77-MHz clock

## PCI bus characteristics

### Shorter connector length, faster data transmission

#### Compared to previous bus types (ISA)

### PCs and Macintosh compatible

## PCIe (PCI Express)

### 32- or 64-bit bus

### Maximum 133-MHz clock speed

### Transfer rate

#### 500 Mbps per data path (full-duplex transmission)

## PCIe advantages over PCI

### More efficient data transfer

### Quality of service distinctions support

### Error reporting, handling

### Current PCI software compatible

## PCIe slots differ from conventional PCI

### Vary by lanes supported

### Lane offers full-duplex throughput of 500 Mbps

#### Support up to 16 lanes

#### x16 slot : 8 Gbps throughput

## Determining bus type

### Read documentation

### Look inside PC case

### If more than one expansion slot type:

#### Refer to NIC, PC manufacturers’ guidelines

#### Choose NIC matching most modern bus

Figure 6-3 A motherboard with multiple expansion slots

# Peripheral Bus Standards

## Attach peripheral devices externally

## External connection advantage

### Simple installation

## Personal Computer Memory Card International Association or PCMCIA

### Sets standards for externally attached cards

#### Connect virtually any external device type

## PC Card

### First standard PCMCIA-standard adapter

#### 16- bit interface running at 8 MHz

## CardBus standard (1990s)

### 32-bit interface running at 33 MHz

### Matches PCI expansion board standard

## ExpressCard standard

### Many different external devices  connect to portable computers

### 26-pin interface

### Data transfer rates: 250 Mbps in each direction

#### 500 Mbps total

### Same data transfer standards as PCIe specification

### Two sizes

#### 34 mm, 54 mm wide

## USB (universal serial bus) port

### Two USB standards

#### Difference: speed

#### USB 1.1: transfer rate of 12 Mbps

#### USB 2.0: transfer rate of 480 Mbps

### Future

#### USB 3.0 (SuperSpeed USB)

#### Transfer rate: 4.8 Gbps

## Firewire

### Apple Computer (1980s)

### IEEE 1394 standard (1995)

### Traditional Firewire connection: 400 Mbps (max)

### Newer version: 3 Gbps

### Connects most peripheral types

### Connects small network

#### Two or more computers using bus topology

## FireWire-connected peripherals

### Similar to USB- and PCMCIA-connected peripherals

#### Simple installation

#### Supported by most modern operating systems

### Two connector varieties: 4-pin and 6-pin

### 6-pin connector

#### Two pins supply power

#### Interconnect computers

## CompactFlash

### Designed by CompactFlash Association (CFA)

#### Ultrasmall

#### Removable data and input/output device

### Latest standard: 4.0

#### Data transfer rate: 133 Mbps

### Uses

#### Connects devices too small for PCMCIA slots

#### Wireless connections

# On-Board NICs

## Connect device directly to motherboard

### On-board ports: mouse, keyboard

## New computers, laptops

### Use onboard NICs integrated into motherboard

## Advantages

### Saves space

### Frees expansion slots

# Wireless NICs

## Contain antennas

### Send, receive signals

### All bus types supported

## Disadvantages over wire-bound NICs

### More expensive

### Bandwidth and security limitations

# Installing NICs

## Three general steps

### Install hardware

### Install NIC software

### Configure firmware (if necessary)

#### Set of data, instructions

#### Saved to NIC’s ROM (read-only memory) chip

#### Use configuration utility program

## EEPROM (electrically erasable programmable read-only memory)

### Apply electrical charges

#### ROM data erased, changed

# Installing and Configuring NIC Hardware

## Read manufacturer’s documentation

## Install expansion card NIC

### Verify toolkit contents

### Unplug computer

### Ground yourself

### Open computer case

#### Select slot, insert NIC, attach bracket, verify cables

### Replace cover, turn on computer

#### Configure NIC software

## Modern operating systems

### Do not require restart for PCMCIA-standard adapter

## Servers, other high-powered computers

### Install multiple NICs

### Repeat installation process for additional NIC

### Choose different slot

# Installing and Configuring NIC Software

## Device driver

### Software

#### Enables attached device to communicate with operating system

## Purchased computer

### Drivers installed

## Add hardware to computer

### Must install drivers

## Operating system built-in drivers

### Automatically recognize hardware, install drivers

### Computer startup

#### Device drivers loaded into RAM

#### Computer can communicate with devices

## Drivers not available from operating system

### Install and configure NIC software

#### Use operating system interface

# Interpreting LED Indicators

## After NIC is installed:

### Test by transmitting data

### Assess NIC LEDs for network communication

#### Vary by manufacturer

#### Read documentation

### Common lights

#### ACT, LNK, LED, TX, RX

# IRQ (Interrupt Request)

## Message to computer

### Stop and pay attention to something else

## Interrupt

### Circuit board wire

#### Device issues voltage to signal request

## IRQ number

### Uniquely identifies component to main bus

### NICs use IRQ 9, 10, or 11

## Two devices using same interrupt

### Resource conflicts, performance problems

#### Many symptoms

### Must reassign IRQ

#### Through operating system

#### Through adapter’s EEPROM configuration utility

#### Through computer’s CMOS configuration utility

## CMOS (complementary metal oxide semiconductor)

### Microchip requiring very little energy to operate

### Stores settings pertaining to computer’s devices

### Battery powered

#### Settings saved after computer turned off

### Information used by BIOS (basic input/output system)

## BIOS

### Simple instruction set

#### Enables computer to initially recognize hardware

# Memory Range

## Memory NIC, CPU use for exchanging, buffering data

## Some are reserved for specific devices

## NICS

### High memory area (A0000–FFFFF range)

### Manufacturers prefer certain ranges

## Resource conflicts less likely (than IRQ settings)

# Base I/O Port

## Memory area

### Channel for moving data between NIC and CPU

## Cannot be used by other devices

## NICs use two channel memory ranges

### Base I/O port settings identify beginning of each range

# Firmware Settings

## Contain NIC’s transmission characteristics

## Combination

### EEPROM chip on NIC and data it holds

## Change firmware

### Change EEPROM chip

### Requires bootable CD-ROM

#### Configuration, install utility shipped with NIC

## Configuration utility

### View IRQ, I/O port, base memory, node address

### Change settings

### Perform diagnostics

#### NIC’s physical components, connectivity

## Loopback plug (loopback adapter)

### Outgoing signals redirected into computer for testing

### Use with loopback test

# Choosing the Right NIC

## Considerations

### Compatibility with existing system

#### Network bus type, access method, connector types, transmission speed

### Drivers available

#### Operating system, hardware

### Subtle differences

#### Affecting network performance

#### Important for server

Repeaters and Hubs

## Repeater

### Simplest connectivity device regenerating signals

### Operates at Physical layer

#### Has no means to interpret data

### Limited scope

#### One input port, one output port

#### Receives and repeats single data stream

### Suitable for bus topology networks

### Extend network inexpensively

### Rarely used on modern networks

#### Limitations; other devices decreasing costs

## Hub

### Repeater with more than one output port

#### Multiple data ports, uplink port

### Repeats signal in broadcast fashion

### Operates at Physical layer

### Ethernet network hub

#### Star or star-based hybrid central connection point

### Connect workstations, print servers, switches, file servers, other devices

### Devices share same bandwidth amount, collision domain

#### More nodes leads to transmission errors, slow performance

### Placement in network varies

#### Simplest: stand-alone workgroup hub

#### Different hub to each small workgroup

#### Placement must adhering to maximum segment and length limitations

### Hubs vary according to:

#### Supported media type, data  transmission speeds

### Passive hubs, Intelligent hubs (managed hubs), Stand-alone hubs (workgroup hubs)

### Replaced by switches or routers

#### Limited features

#### Merely repeat signals

Bridges

# Bridges

## Connects two network segments

### Analyze incoming frames and decide where to send

#### Based on frame’s MAC address

## Operate at Data Link layer

## Single input port and single output port

## Interpret physical addressing information

## Advantages over repeaters and hubs

### Protocol independence

### Add length beyond maximum segments limits

### Improve network performance

## Disadvantage compared to repeaters and hubs

### Longer to transmit data

## Filtering database (forwarding table)

### Used in decision making

#### Filter or forward

## New bridge installation

### Bridge must learn network MAC addresses

### Fills its filtering database

#### Destination node’s MAC address

#### Associated port

### All network nodes discovered over time

## Today bridges nearly extinct

### Improved router and switch speed, functionality

### Lowered router and switch cost

Switches

Figure 6-16 Switches

Switches

## Subdivide network

### Smaller logical pieces, segments

## Operates at Data Link layer (traditional)

## Operate at layers 3 and 4 (advanced)

## Interpret MAC address information

## Components

### Internal processor, operating system, memory, several ports

## Multiport switch advantages over bridge

### Better bandwidth use, more cost-efficient

### Each port acts like a bridge

#### Each device effectively receives own dedicated channel

### Ethernet perspective

#### Dedicated channel represents collision domain

## Historically

### Switches replaced hubs, eased congestion, provided better security, performance

## Disadvantages

### Can become overwhelmed despite buffers

#### Cannot prevent data loss

#### UDP collisions mount: network traffic halts

## Switches replaced workgroup hubs

### Decreased cost, easy installation, configuration,

### Separate traffic according to port

# Installing a Switch

## Follow manufacturer’s guidelines

## General steps (assume Cat 5 or better UTP)

### Verify switch placement

### Turn on switch

Figure 6-18 A switch on a small network

Verify lights, self power tests

### Configure (if necessary)

### Connect NIC to a switch port (repeat for all nodes)

### After all nodes connected, turn on nodes

### Connect switch to larger network (optional)

# Switching Methods

## Difference in switches

### Incoming frames interpretation

### Frame forwarding decisions making

## Four switching modes exist

### Two basic methods discussed

#### Cut-Through Mode

#### Store-and-Forward Mode

# Cut-Through Mode

## Switch reads frame’s header

## Forwarding decision made before receiving entire packet

### Uses frame header: first 14 bytes contains destination MAC address

## Cannot verify data integrity using frame check sequence

## Can detect runts

### Erroneously shortened packets

## Runt detected: wait for integrity check

## Cannot detect corrupt packets

### May propagate flawed packets

## Advantage

### Speed

## Disadvantage

### Data buffering (switch flooded with traffic)

## Best use

### Small workgroups needing speed

### Low number of devices

# Store-and-Forward Mode

## Switch reads entire data frame into memory

### Checks for accuracy before transmitting information

## Advantage over cut-through mode

### Transmit data more accurately

## Disadvantage over cut-through mode

### More time consuming

## Best use

### Larger LAN environments; mixed environments

### Can transfer data between segments running different transmission speeds

# VLANs and Trunking

## VLANs (virtual local area networks)

### Logically separate networks within networks

#### Groups ports into broadcast domain

## Broadcast domain (subnet)

### Port combination making a Layer 2 segment

#### Ports rely on layer 2 device to forward broadcast frames

## Collision domain

### Remember, switches prevent collisions

### Each device is on a separate collision domain

Image from link Ch 6c

## Advantage of VLANs

### Flexible

#### Ports from multiple switches or segments

#### Use any end node type

### Reasons for using VLAN

#### Separating user groups who need special security

#### Isolating connections with heavy traffic

#### Identifying priority device groups

#### Grouping legacy protocol devices

## VLAN creation

### Configuring switch software

#### Manually through configuration utility

#### Automatically using VLAN software tool

### Critical step

#### Indicate to which VLAN each port belongs

### Additional specifications

#### Security parameters, filtering instructions, port performance requirements, network addressing and management options

## Maintain VLAN by switch software

## Potential VLAN issues

### Cutting off group from rest of network

#### Correct by using router

## Trunking

### Switch’s interface carries traffic of multiple VLANs

## Trunk

### Single physical connection between devices

#### Many logical VLANs transmit, receive data

## VLAN data separation

### Frame contains VLAN identifier in header

## Advantage of VLAN trunking

### Economical interface usage

### Switches make efficient use of processing capabilities

## VLAN configuration

### Can be complex

### Requires careful planning

#### Ensure users and devices can exchange data

#### Ensure VLAN switch properly interacts with other devices

# STP (Spanning Tree Protocol)

## IEEE standard 802.1D

## Operates in Data Link layer

## Prevents traffic loops

### Calculating paths avoiding potential loops

### Artificially blocking links completing loop

## Three steps

### Select root bridge based on Bridge ID

### Examine possible paths between network bridge and root bridge

### Disables links not part of shortest path

## History

### Introduced in 1980s

#### Original STP too slow

### RSTP (Rapid Spanning Tree Protocol)

#### Newer version

#### IEEE’s 802.1w standard

## Cisco and Extreme Networks

### Proprietary versions

## No enabling or configuration needed

### Included in switch operating software

Figure 6-21 Enterprise-wide switched network

May alter default priorities

# Content and Multilayer Switches

## Layer 3 switch (routing switch)

### Interprets Layer 3 data

## Layer 4 switch

### Interprets Layer 4 data

## Content switch (application switch)

### Interprets Layer 4 through Layer 7 data

## Advantages

### Advanced filtering, statistics keeping, security functions

## Disadvantages

### No agreed upon standard

#### Layer 3 and Layer 4 switch features vary widely

Figure 6-22 STP-selected paths on a switched network

## Distinguishing between Layer 3 and Layer 4 switch

### Manufacturer dependent

## Higher-layer switches

### Three times Layer 2 switches

### Used in backbone

Routers

## Multiport connectivity device

### Directs data between network nodes

### Integrates LANs and WANs

#### Different transmission speeds, protocols

## Operate at Network layer (Layer 3)

### Directs data from one segment or network to another

### Logical addressing

### Protocol dependent

## Slower than switches and bridges

### Need to interpret Layers 3 and higher information

## Traditional stand-alone LAN routers

### Being replaced by Layer 3 routing switches

## New niche for routers

### Specialized applications

#### Linking large Internet nodes

#### Completing digitized telephone calls

# Router Characteristics and Functions

## Intelligence

### Tracks node location

### Determine shortest, fastest path between two nodes

### Connects dissimilar network types

## Large LANs and WANs

### Routers indispensible

Figure 6-23 Routers

## Router components

### Internal processor, operating system, memory, input and output jacks, management control interface

## Modular router

### Multiple slots

#### Holding different interface cards, other devices

## Inexpensive routers

### Home, small office use

## Router tasks

### Connect dissimilar networks

### Interpret Layer 3 addressing

### Determine best data path

### Reroute traffic

Figure 6-24 The placement of routers on a LAN

## Optional functions

### Filter broadcast transmissions

### Enable custom segregation, security

### Provide fault tolerance

### Monitor network traffic, diagnose problems

## Directing network data

### Static routing

#### Administrator programs specific paths between nodes

### Dynamic routing

#### Router automatically calculates best path between two nodes

#### Routing table

## Installation

### Simple: small office, home LANs

### Challenging: sizeable networks

# Routing Protocols

## Best path

### Most efficient route from one node to another

### Dependent on:

#### Number of hops between nodes

#### Current network activity

#### Unavailable link

#### Network transmission speed

#### Topology

### Determined by routing protocol

## Routing protocol

### Router communication

### Collects current network status data

#### Contribute to best path selection

#### Routing table creation

## Router convergence time

### Time router takes to recognize best path

#### Change or network outage event

### Distinguishing feature

#### Overhead; burden on network to support routing protocol

# Distance-Vector: RIP, RIPv2, BGP

## Distance-vector routing protocols

### Determine best route based on distance to destination

### Factors

#### Hops, latency, network traffic conditions

## RIP (Routing Information Protocol)

### Only factors in number of hops between nodes

#### Limits 15 hops

### Interior routing protocol

### Slow and less secure

## RIPv2 (Routing Information Protocol Version 2)

### Generates less broadcast traffic, more secure

### Cannot exceed 15 hops

### Less commonly used

## BGP (Border Gateway Protocol)

### Communicates using BGP-specific messages

### Many factors determine best paths

### Configurable to follow policies

### Most complex (choice for Internet traffic)

# Link-State: OSPF, IS-IS

## Link-state routing protocol

### Routers share information

#### Each router independently maps network, determines best path

## OSPF (Open Shortest Path First)

### Interior or border router use

### No hop limit

### Complex algorithm for determining best paths

### Each OSPF router

#### Maintains database containing other routers’ links

## IS-IS (Intermediate System to Intermediate System)

### Codified by ISO

### Interior routers only

### Less common than OSPF

# Hybrid: EIGRP

## Hybrid

### Link-state and distance-vector characteristics

### EIGRP (Enhanced Interior Gateway Routing Protocol)

#### Cisco network routers only

### EIGRP benefits

#### Fast convergence time, low network overhead

#### Easier to configure and less CPU-intensive than OSPF

#### Supports multiple protocols

#### Accommodates very large, heterogeneous networks

Gateways and Other Multifunction Devices

## Gateway

### Combinations of networking hardware and software

#### Connecting two dissimilar networks

### Connect two systems using different formatting, communications protocols, architecture

### Repackages information

### Reside on servers, microcomputers, connectivity devices, mainframes

## Popular gateways

### E-mail gateway, Internet gateway, LAN gateway, Voice/data gateway, Firewall

Last modified 10-1-09