# Objectives

## Identify a variety of uses for WANs

## Explain different WAN topologies, including their advantages and disadvantages

## Compare the characteristics of WAN technologies, including their switching type, throughput, media, security, and reliability

## Describe several WAN transmission and connection methods, including PSTN, ISDN, T-carriers, DSL, broadband cable, ATM, and SONET

## Describe multiple methods for remotely connecting to a network

WAN Essentials

# WAN Essentials

## WAN

### Network traversing some distance, connecting LANs

### Transmission methods dependent on business needs

## WAN and LAN common properties

### Client-host resource sharing, Layer 3 protocols, packet-switched digitized data

## WAN and LAN differences

### Layers 1 and 2 access methods, topologies, media

### LAN wiring: private

### WAN wiring: public through NSPs (network service providers)

## WAN site

### Individual geographic locations

## WAN link

### WAN site to WAN site connection

WAN Topologies

## Differences from LAN topologies

### Distance covered, number of users, distance traveled

### Connect sites via dedicated links

#### Much slower than LAN connections

#### Use different connectivity devices

## WAN connections

### Require Layer 3 devices

#### Routers

### Not capable of nonroutable protocols

### Exception: Metro Ethernet (not in book here)

# Bus

## Each site connects to two sites maximum serially

### Similar LAN topology site dependency

#### Network site dependent on every other site to transmit and receive traffic

### Difference from LAN topology

#### Different locations connected to another through point-to-point links

## Best use

### Organizations requiring small WAN, dedicated circuits

## Drawback

### Not scalable

# Ring

## Each site connected to two other sites

### Forms ring pattern

#### Similar to LAN ring topology

## Differences from LAN ring topology

### Connects locations

### Relies on redundant rings

#### Data rerouted upon site failure

### Expansion

#### Difficult, expensive

## Best use

### Connecting four, five locations maximum

# Star

## Mimics star topology LAN

### Single site central connection point

### Separate data routes between any two sites

## Advantages

### Single connection failure affects one location

#### Different from bus, star topology

### Shorter data paths between any two sites

#### When all dedicated circuits functioning

### Expansion: simple, less costly

## Drawback

### Central site is a single point of failure

# Mesh

## Incorporates many directly interconnected sites

### Data travels directly from origin to destination

### Routers can redirect data easily, quickly

## Most fault-tolerant WAN type

## Full-mesh WAN

### Every WAN site directly connected to every other site

### Drawback: cost

## Partial-mesh WAN

### Reduce costs

# Tiered

## Sites connected in star or ring formations

### Interconnected at different levels

### Interconnection points organized into layers

#### Form hierarchical groupings

## Flexibility

### Allows many variations, practicality

### Requires careful considerations:

#### Geography, usage patterns, growth potential

#### Image from link Ch 7a

PSTN

## PSTN (Public Switched Telephone Network)

### Network of lines, carrier equipment providing telephone service

### POTS (plain old telephone service)

### Encompasses entire telephone system

### Originally: analog traffic

### Today: digital data, computer controlled switching

## Dial-up connection

### Used early on

### Modem connects computer to distant network

#### Not always on—you need to dial up to connect

# PSTN Elements

## Cannot handle digital transmission (older parts of the network)

### Requires modem to convert digital to analog and vice versa

## Signal travels path between modems

### Over carrier’s network

#### Includes CO (central office), remote switching facility

#### Signal converts back to digital pulses

## CO (central office)

### Where telephone company terminates lines

### Switches calls between different locations

## Local loop (last mile)

### Portion connecting residence, business to nearest CO

#### Most likely uses copper wire, carries analog signal

#### Some cities have fiber to the home (FTTH)

## Demarcation point

### Local loop endpoint

### Carriers responsibility ends

### Wires terminate at NIU (network interface unit)

## PSTN Internet connection advantages

### Ubiquity, ease of use, low cost

## PSTN disadvantages

### Some circuit switching used

### Marginal security

### Slow (56 kbps max.)

X.25 and Frame Relay

## X.25 ITU standard

### Analog, packet-switching technology

#### Designed for long distance

### Original standard: mid 1970s

#### Mainframe to remote computers: 64 Kbps throughput

### Update: 1992

#### 2.048 Mbps throughput

#### Client, servers over WANs

### Verifies transmission at every node

#### Excellent flow control, ensures data reliability

#### Slow and unsuitable for time-sensitive applications

### Never adopted widely in the USA

## Frame relay

### Updated X.25: digital, packet-switching

### Protocols operate at Data Link layer

#### Supports multiple Network, Transport layer protocols

## Both perform error checking

### Frame relay: no reliable data delivery guarantee

#### Checks for errors but does not fix them

### X.25: errors fixed or retransmitted

## Throughput

### Frame relay: 64 Kbps to 45 Mbps

### Customer chooses

## Both use virtual circuits

### Based on potentially disparate physical links

#### Logically appear direct

### Advantage: efficient bandwidth use

## Both configurable as SVCs (switched virtual circuits)

### Connection established for transmission, terminated when complete

## Both configurable as PVCs (permanent virtual circuits)

### Connection established before transmission, remains after transmission

## PVCs

### Not a dedicated line--you are sharing the wires with other people

### Path can change

## X.25 or frame relay lease contract

### Specify endpoints, bandwidth

### CIR (committed information rate)

#### Minimum bandwidth guaranteed by carrier

## PVC lease

### Share bandwidth with other users

## Frame relay lease advantage

### Pay for bandwidth required

### Less expensive technology

### Long-established worldwide standard

## Frame relay and X.25 disadvantage

### Throughput variability, due to shared lines

### Not as private or secure as dedicated lines

## Frame relay and X.25 easily upgrade to T-carrier dedicated lines

### Due to same connectivity equipment

ISDN

## Digital data transmitted over PSTN

## Gained popularity: 1990s

### Connecting WAN locations

#### Exchanges data, voice signals

## Protocols at Physical, Data Link, Network layers

### Signaling, framing, connection setup and termination, routing, flow control, error detection and correction

## Relies on PSTN for transmission medium

## Dial-up or dedicated connections

### Dial-up relies exclusively on digital transmission

# Error in Textbook

## Page 311, second paragraph

## “ISDN specifies protocols at the Physical, Data Link, and Transport layers”

### SHOULD BE

## “ISDN specifies protocols at the Physical, Data Link, and Network layers”

## Single line

### Simultaneously: two voice calls, one data connection

## Two channel types

### B channel: “bearer”

#### Circuit switching for voice, video, audio: 64 Kbps

### D channel: “data”

#### Packet-switching for call information: 16 or 64 Kbps

## BRI (Basic Rate Interface) connection

## PRI (Primary Rate Interface) connection

## BRI: two B channels, one D channel (2B+D)

### B channels treated as separate connections

#### Carry voice and data

## Bonding

### Two 64-Kbps B channels combined

#### Achieve 128 Kbps

Figure 7-10 A BRI link

## NT1: Network Termination 1

## TA: Terminal Adapter

Figure 7-11 A PRI link

## PRI: 23 B channels, one 64-Kbps D channel (23B+D)

### Separate B channels independently carry voice, data

### Maximum throughput: 1.544 Mbps

## PRI and BRI may interconnect

T-Carriers

## T1s, fractional T1s, T3s

## Physical layer operation

## Single channel divided into multiple channels

### Using TDM (time division multiplexing) over two wire pairs

## Medium

### Telephone wire, fiber-optic cable, wireless links

# Types of T-Carriers

## Many available

### Most common: T1 and T3

## T1: 24 voice or data channels

### Maximum data throughput: 1.544 Mbps

## T3: 672 voice or data channels

### Maximum data throughput: 44.736 Mbps (45 Mbps)

## T-carrier speed dependent on signal level

### Physical layer electrical signaling characteristics

### DS0 (digital signal, level 0)

#### One data, voice channel

## T1 use

### Connects branch offices, connects to carrier

### Connects telephone company COs, ISPs

## T3 use

### Data-intensive businesses

## T3 provides 28 times more throughput (expensive)

### Multiple T1’s may accommodate needs

## TI costs vary by region

## Fractional T1 lease

### Use some T1 channels, charged accordingly

# T-Carrier Cost

## Link Ch 7b

# T-Carrier Connectivity

## T-carrier line requires connectivity hardware

### Customer site, switching facility

### Purchased or leased

## T-carrier line requires different media

### Throughput dependent

Figure 7-12 A T1 smart jack

Wiring

### Plain telephone wire

#### UTP or STP copper wiring

#### STP preferred for clean connection

### Coaxial cable, microwave, fiber-optic cable

### T1s using STP require repeater every 6000 feet

### Multiple T1s

#### Coaxial cable, microwave, fiber-optic cabling

### T3s require microwave, fiber-optic cabling

## Smart Jack

### Terminate T-carrier wire pairs

#### Customer’s demarc (demarcation point)

#### Inside or outside building

### Connection monitoring point

## CSU/DSU (Channel Service Unit/Data Service Unit)

### Two separate devices

### Combined into single stand-alone device

#### Interface card

### T1 line connection point

#### At customer’s site

## CSU

### Provides digital signal termination

### Ensures connection integrity

## DSU

### Converts T-carrier frames into frames LAN can interpret (vice versa)

### Connects T-carrier lines with terminating equipment

### Incorporates multiplexer

## Incoming T-carrier line

### Multiplexer separates combined channels

## Outgoing T-carrier line

### Multiplexer combines multiple LAN signals

## Terminal Equipment

### Switches, routers, bridges

### Best option: router, Layer 3 or higher switch

#### Accepts incoming CSU/DSU signals

#### Translates Network layer protocols

#### Directs data to destination

## CSU/DSU may be integrated with router, switch

### Expansion card

### Faster signal processing, better performance

### Less expensive, lower maintenance solution

DSL

## DSL (digital subscriber line)

### Operates over PSTN

### Directly competes with ISDN, T1 services

### Not available in all areas: must be close to a telco central office

### Best suited for WAN local loop

### Supports multiple data, voice channels

#### Over single line

#### Higher, inaudible telephone line frequencies

### Uses advanced data modulation techniques

#### Data signal alters carrier signal properties

#### Amplitude or phase modulation

# Types of DSL

## xDSL refers to all DSL varieties

### ADSL, G.Lite, HDSL, SDSL, VDSL, SHDSL

## Two DSL categories

### Asymmetrical and symmetrical

## Downstream

### Data travels from carrier’s switching facility to customer

## Upstream

### Data travels from customer to carrier’s switching facility

## Downstream, upstream throughput rates may differ

### Asymmetrical

#### More throughput in one direction

#### Downstream throughput higher than upstream throughput

#### Best use: video conferencing, web surfing

### Symmetrical

#### Equal capacity for upstream, downstream data

#### Examples : HDSL, SDSL, SHDSL

#### Best use: uploading, downloading significant data amounts

## How DSL types vary

### Data modulation techniques

### Capacity

### Distance limitations

### PSTN use

# DSL Connectivity

## ADSL: common example on home computer

### Establish TCP connection

### Transmit through DSL modem

#### Internal or external

#### Splitter separates incoming voice, data signals

#### May connect to hub, switch, router

### DSL modem forwards modulated signal to local loop

#### Signal continues over four-pair UTP wire

#### Distance less than 18,000 feet: signal combined with other modulated signals in telephone switch

### Carrier’s remote switching facility

#### Splitter separates data signal from voice signals

#### Request sent to DSLAM (DSL access multiplexer) which aggregates many DSL lines together

#### Combined signal is sent to the Internet backbone

## DSL competition

### T1, ISDN, broadband cable

## DSL installation

### Hardware, monthly access costs

#### Slightly less than ISDN, significantly less than T1s

## DSL drawbacks

### Not available in all areas

### Upstream throughput lower than broadband cable

Broadband Cable

## Cable companies connectivity option

## Based on TV signals coaxial cable wiring

### Theoretical maximum speed

#### 150 Mbps downstream, 10 Mbps upstream

### Real transmission

#### 10 Mbps downstream, 2 Mbps upstream

#### Transmission limited (throttled)

#### Shared physical connections

## Best use

### Web surfing

### Network data download

## Requires cable modem

### Modulates, demodulates transmission, reception signals via cable wiring

### Operates at Physical and Data Link layer

### May connect to connectivity device, like a hub, switch, or router to allow several computers to share the bandwidth

## Infrastructure required

### HFC (hybrid fiber-coax)

#### Expensive fiber-optic link supporting high frequencies

#### connects cable company’s offices to node

#### Location near customer

### Cable drop

#### Connects node to customer’s business or residence

#### Fiber-optic or coaxial cable

#### Connects to head end

## Provides dedicated connection

## Many subscribers share same local line, throughput

ATM (Asynchronous Transfer Mode)

## Functions in Data Link layer

## Asynchronous communications method

### Each frame transmitted with start and stop bits

## Specifies Data Link layer framing techniques

## Fixed packet size

### Sets ATM apart from Ethernet

### Packet (cell)

#### 48 data bytes plus 5-byte header

## Smaller packet size requires more overhead

### Decrease potential throughput

### Cell efficiency compensates for loss

## ATM relies on virtual circuits

### ATM considered packet-switching technology

### Virtual circuits provide circuit switching advantage

#### Reliably available point-to-point connection

### Reliable connection

## Allows specific QoS (quality of service) guarantee

### Important for time-sensitive applications

## Compatible with other leading network technologies

### Cells support multiple higher-layer protocols

### LANE (LAN Emulation)

#### Allows integration with Ethernet, token ring network

#### Encapsulates incoming Ethernet or token ring frames

#### Converts to ATM cells for transmission

## Throughput

### 25 Mbps to 622 Mbps

## Cost

### Relatively expensive

### Gigabit Ethernet is replacing ATM on many networks

# ATM Service Costs

### 256 Kbps	$ 600 /month

### 3 Mbps  	$ 1,200 /month

### 45 Mbps	$ 8,000 /month

### From links Ch 6c, 6d

SONET (Synchronous Optical Network)

## Four key strengths

### It can integrate many other WAN technologies

### Fast data transfer rates

### Simple link additions, removals

### High degree of fault tolerance

## Synchronous

### Data transmitted, received by nodes conforms to timing scheme

## Advantage

### Interoperability

## Fault tolerance

### Double-ring topology over fiber-optic cable

## SONET Ring

### Begins, ends at telecommunications carrier’s facility

### Connects organization’s multiple WAN sites in ring fashion

### Connect with multiple carrier facilities

#### Additional fault tolerance

### Terminates at multiplexer on carrier and customer premises

#### Easy SONET ring connection additions, removals

## Data rate

### Indicated by OC (Optical Carrier) level

## Implementation

### Large companies

### Long-distance companies

#### Linking metropolitan areas and countries

### ISPs

#### Guarantying fast, reliable Internet access

### Telephone companies

#### Connecting Cos

## COST

### Expensive

# SONET Prices

### OC1	51.84 Mbps 		$ 10,000- $20,000 /month

### OC3  	155.52 Mbps 		$ 30,000- $50,000 /month

### OC24 	1.244  Gbps 		over $100,000 /month

### OC255 	13.21 Gbps 		costs are extremely high

### From Link Ch 6e

WAN Technologies Compared

Remote Connectivity

## Remote access

### Service allowing client connection, log on capability

#### LAN or WAN in different geographical location

## Remote client

### Access files, applications, shared resources

## Remote access communication requirement

### Client, host transmission path

### Appropriate software

### Dial-up networking, Microsoft’s RAS or RRAS, VPNs

# Dial-Up Networking

## Dialing directly into private network’s or ISP’s remote access server

### Log on to network

## Transmission methods

### PSTN, X.25, ISDN

## Advantages

### Technology well understood

### Software availability

## Disadvantages

### Throughput

### Quality

### Administrative maintenance

## Microsoft software

### RAS (Remote Access Service) (Early Windows versions)

### RRAS (Routing and Remote Access Service) (Windows 2000 Server, XP, and later versions)

# Remote Access Servers

## Server requirements

### Accept client connection

#### Grant privileges to network’s resources

## Device types

### Dedicated devices: Cisco’s AS5800 access servers

### Computers installed with special software

## Microsoft remote access software

### RRAS (Routing and Remote Access Service)

#### Computer accepts multiple remote client connections

#### Server acts as router

#### Multiple security provisions

# Remote Access Protocols

## SLIP and PPP

### Workstations connect using serial connection

#### Encapsulate higher-layer networking protocols, in lower-layer data frames

### SLIP carries IP packets only

#### Harder to set up

#### Supports only asynchronous data

### PPP carries many different Network layer packets

#### Automatic set up

#### Performs error correction, data compression, supports encryption

#### Supports asynchronous and synchronous transmission

## PPPoE (PPP over Ethernet) standard

### Connects home computers to ISP

#### Via DSL, broadband cable

# Remote Virtual Computing

## Computer client controls computer host (server)

### Across network connection

## Dedicated WAN link, Internet connection, dial-up

### Established directly between client, host modems

## Host allows client access

### User name or computer name, password credentials

## Thin client

### Remote virtual computing software requires little bandwidth

## Advantage

### Simple configuration

### Runs on any connection type

### Single host

#### Accept simultaneous connections from multiple clients

## Remote virtual computing software

### Differences

#### Capabilities, security mechanisms, supported platforms

### Examples

#### Microsoft’s Remote Desktop, VNC, Citrix’s ICA

## Remote desktop

### Windows client and server operating systems

### Relies on RDP (Remote Desktop Protocol)

#### Application layer protocol

#### Uses TCP/IP to transmit graphics, text quickly

#### Carries session, licensing, encryption information

#### Exists for other operating systems

### Not included in Windows home editions

## VNC (Virtual Network Computing)

### Open source system

#### One workstation remotely manipulates, receives screen updates from another workstation

#### Free, anyone can modify

### Protocols operate in Application layer

### Advantages

#### Multiple computer platform operation

#### Open source

#### Single computer supports multiple sessions

### Drawback: screen refresh rate

## ICA (Independent Computing Architecture)

### Citrix System’s Presentation Server

#### Proprietary software

### Advantages

#### Ease of use

#### Broad compatibility

### Disadvantages

#### High cost of Citrix products

#### Server software configuration complexity

VPNs (Virtual Private Networks)

## Wide area networks

### Logically defined over public transmission systems

## Isolated from other public line traffic

## Software

### Inexpensive

### Sometimes included with other widely used software

## Tailored to customer’s distance, bandwidth needs

## Two important design considerations

### Interoperability and security

## Tunneling

### Ensures VPN carries all data types privately

## Tunnel

### Virtual connection between two VPN nodes

## PPTP (Point-to-Point Tunneling Protocol)

### Microsoft

#### Encryption, authentication, access services

### Dial directly into RRAS access server

### Dial into ISP’s remote access server first

## L2TP (Layer 2 Tunneling Protocol)

### Cisco

#### Connects VPN using equipment mix

#### Connect two routers

#### Tunnel endpoints not on same packet-switched network

Last modified 10-12-09