# Objectives

## Explain how nodes exchange wireless signals

## Identify potential obstacles to successful wireless transmission and their repercussions, such as interference and reflection

## Understand WLAN (wireless LAN) architecture

# Objectives (cont’d.)

## Specify the characteristics of popular WLAN transmission methods, including 802.11 a/b/g/n

## Install and configure wireless access points and their clients

## Describe wireless MAN and WAN technologies, including 802.16 and satellite communications

The Wireless Spectrum

## Continuum of electromagnetic waves

### Data, voice communication

### Arranged by frequencies

#### Lowest to highest

### Spans 9 KHz and 300 GHz

## Wireless services associated with one area

## FCC oversees United States frequencies

## ITU oversees international frequencies

### Air signals propagate across borders

Characteristics of Wireless Transmission

## Similarities with wired

### Layer 3 and higher protocols

### Signal origination

#### From electrical current, travel along conductor

## Differences from wired

### Signal transmission

#### No fixed path, guidance

## Antenna

### Signal transmission and reception

### Same frequency required on each antenna

#### Share same channel

# Antennas

## Radiation pattern

### Relative strength over three-dimensional area

#### All electromagnetic energy antenna sends, receives

## Directional antenna

### Issues wireless signals along single direction

## Omnidirectional antenna

### Issues, receives wireless signals

#### Equal strength, clarity

#### All directions

## Range

### Reachable geographical area

# Signal Propagation

## LOS (line-of-sight)

### Signal travels

#### In straight line, directly from transmitter to receiver

## Obstacles affect signal travel

### Pass through them

### Absorb into them

### Subject signal to three phenomena

#### Reflection: bounce back to source

#### Diffraction: splits into secondary waves

#### Scattering: diffusion in multiple different directions

## Multipath signals

### Wireless signals follow different paths to destination

### Caused by reflection, diffraction, scattering

### Advantage

#### Better chance of reaching destination

### Disadvantage

#### Signal delay

# Signal Degradation

## Fading

### Change in signal strength

#### Electromagnetic energy scattered, reflected, diffracted

## Attenuation

### Signal weakens

#### Moving away from transmission antenna

### Correcting signal attenuation

#### Amplify (analog), repeat (digital)

## Noise

### Usually the worst problem

#### No wireless conduit, shielding

# Frequency Ranges

## 2.4-GHz band (older)

### Frequency range: 2.4–2.4835 GHz

### 11 unlicensed communications channels

### Susceptible to interference

## Unlicensed

### No FCC registration required

## 5-GHz band (newer)

### Frequency bands

#### 5.1 GHz, 5.3 GHz, 5.4 GHz, 5.8 GHz

### 24 unlicensed bands, each 20 MHz wide

### Used by weather, military radar communications

# Narrowband, Broadband, and Spread Spectrum Signals

## Defines wireless spectrum use:

### Narrowband

#### Transmitter concentrates signal energy at single frequency, very small frequency range

### Broadband

#### Relatively wide wireless spectrum band

#### Higher throughputs than narrowband

### Spread-spectrum

#### Multiple frequencies used to transmit signal

#### Offers security

## FHSS (frequency hopping spread spectrum)

### Signal jumps between several different frequencies within band

### Synchronization pattern known only to channel’s receiver, transmitter

## DSSS (direct-sequence spread spectrum)

### Signal’s bits distributed over entire frequency band at once

### Each bit coded

#### Receiver reassembles original signal upon receiving bits

# Fixed versus Mobile

## Fixed communications wireless systems

### Transmitter, receiver locations do not move

### Transmitting antenna focuses energy directly toward receiving antenna

#### Point-to-point link results

### Advantage

#### No wasted energy issuing signals

#### More energy used for signal itself

## Mobile communications wireless systems

### Receiver located anywhere within transmitter’s range

#### Receiver can roam

WLAN (Wireless LAN) Architecture

## Ad hoc WLAN

### Wireless nodes transmit directly to each other

### Use wireless NICs

#### No intervening connectivity device

### Poor performance

#### Many spread out users, obstacles block signals

## Access point (AP)

### Accepts wireless signals from multiple nodes

#### Retransmits signals to network

### Base stations, wireless routers, wireless gateways

## Infrastructure WLAN

### Stations communicate with access point

#### Not directly with each other

### Access point requires sufficient power, strategic placement

## WLAN may include several access points

### Dependent upon number of stations

### Stations per access point varies: 10-100

## Mobile networking allows roaming wireless nodes

### Range dependent upon wireless access method, equipment manufacturer, office environment

#### Access point range: 300 feet maximum

## Point-to-point link

### Can connect two separate LANs

### Fixed link, directional antennas between two access points

#### Allows access points 1000 feet apart

## Support for same protocols, operating systems as wired LANs

### Ensures compatibility

802.11 WLANs

## Wireless technology standard

### Describes unique functions

#### Physical and Data Link layers

### Differences

#### Specified signaling methods, geographic ranges, frequency usages

### Developed by IEEE’s 802.11 committee

## Wi-Fi (wireless fidelity) standards

### 802.11b, 802.11a, 802.11g, 802.11n (draft)

### Share characteristics

#### Half-duplexing, access method, frame format

# 802.11n Approved

## Approved on 9-11-09

### See link Ch 8b

# Access Method

## 802.11 MAC services

### Append 48-bit (6-byte) physical addresses to frame

#### Identifies source, destination

## Same physical addressing scheme as 802.3

### Allows easy combination

## Wireless devices

### Not designed for simultaneous transmit, receive

### Cannot quickly detect collisions

### Use different access method

## CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance)

### Minimizes collision potential

### Uses ACK packets to verify every transmission

#### Requires more overhead than 802.3

#### Real throughput less than theoretical maximum

## RTS/CTS (Request to Send/Clear to Send) protocol

### Optional

### Ensure packets not inhibited by other transmissions

### Efficient for large transmission packets

### Further decreases overall 802.11 efficiency

# Association

## Several packet exchanged between computer, access point

### Gain Internet access

## Scanning

### Surveying surroundings for access point

### Active scanning transmits special frame

#### Probe

### Passive scanning listens for special signal

#### Beacon frame

## SSID (service set identifier)

### As shown, names like NETGEAR or 2WIRE619

### Unique character string identifying access point

#### In beacon fame information

### Configured in access point

### Better security, easier network management

## BSS (basic service set)

### Station groups sharing Access Point

### BSSID (basic service set identifier)

#### Station group identifier

### The MAC address of the Access Point

## ESS (extended service set)

### Access point group connecting same LAN

#### Share ESSID (extended service set identifier)

### Allows roaming

#### Station moving from one BSS to another without losing connectivity

### Example: "CCSF Wi-Fi" is the ESSID of our ESS with 100 Access Points

## Several access points detected

### Select strongest signal, lowest error rate

### Poses security risk

#### Powerful, rogue access point can perform a man-in-the-middle attack

## ESS with several authorized access points

### Must allow station association with any access point

#### While maintaining network connectivity

## Reassociation

### Mobile user moves from one access point’s range into another’s range

### Occurs by simply moving, high error rate

## Stations’ scanning feature

### Used to automatically balance transmission loads

#### Between access points

# Frames

## 802.11 specifies MAC sublayer frame type

## Three categories of frames

### Management: association and reassociation

#### Probe, beacon frames

### Control: medium access, data delivery

#### ACK and RTS/CTS frames

### Data: carry data sent between stations

## 802.11 data frame overhead

### Four address fields

#### Source address and destination address are the same as in Ethernet

#### Transmitter address and receiver address refer to an intermediate access point in large WLANs

### Sequence Control field

#### Labels fragmented frames so they can be reassembled

### Frame Control field

#### Type of frame, encryption, retry, etc.

## All forms of Wi-Fi share the same MAC sublayer characteristics

### 802.11a, b, g, n

## They differ in modulation methods, frequency, usage, ranges

# 802.11b

## DSSS (direct-sequence spread spectrum) signaling

## 2.4-GHz band

### Separated into 22-MHz channels

## Throughput

### 11 Mbps theoretical

### 5 Mbps actual throughput

## 100 meters distance limit

### Node to Access Point

## Oldest, least expensive

## Being replaced by 802.11g

# 802.11a

## Released after 802.11b

## 5-GHz band

### Not congested like 2.4-GHz band

#### Lower interference, requires more transmit power

## Throughput

### 54 Mbps theoretical

### 11 and 18 Mbps effective

#### Attributable to higher frequencies, unique modulating data method, more available bandwidth

## 20 meter distance limit

## More expensive, least popular

# Orthogonal Frequency Division Multiplexing (OFDM)

## Uses each frequency to carry data in parallel

## Faster than DSSS

## Used by 802.11a, g

# 802.11g

## Affordable as 802.11b

## Throughput

### 54 Mbps theoretical

### 20 to 25 Mbps effective

## 100 meter node range

## 2.4-GHz frequency band

### Compatible with 802.11b networks

# 802.11n

## Draft: expected ratification in late 2009

## Manufacturers

### Selling 802.11n-compatible transceivers

## Primary goal

### Wireless standard providing much higher effective throughput

## Maximum throughput: 600 Mbps

### Threat to Fast Ethernet

## Backward compatible with 802.11a, b, g standards

## 2.4-GHz or 5-GHz frequency range

## Compared with 802.11a, 802.11g

### Same data modulation techniques

## Compared with three 802.11 standards

### Manages frames, channels, encoding differently

#### Allows high throughput

## MIMO (multiple input-multiple output)

### Multiple access point antennas may issue signal to one or more receivers

### Receivers combine signal together

### Increases network’s throughput, access point’s range

## Channel bonding

### Two adjacent 20-MHz channels bonded to make 40-MHz channel

#### More than doubles the bandwidth available in single 20-MHz channel

#### Because less bandwidth is used to buffer between channels

## Higher modulation rates

### More efficient use of channels

## Frame aggregation

### Combine multiple frames into one larger frame

### Advantage: reduces overhead

## Maximum throughput depends on the strategies used

### 2.4-GHz or 5-GHz band

### Actual throughput: 65 to 600 Mbps

## Backward compatible, can be mixed with 802.11a, 802.11b, or 802.11 g

### Not all 802.11n features work in mixed-mode WLANs

## Recommendation

### Use 802.11n-compatible devices

Bluetooth Networks

## Ericson’s original goals

### Wireless technology compatible with multiple devices

### Require little power

### Cover short ranges

## Aim of Bluetooth Special Interest Group (SIG)

### Refine and standardize technology

### Result: Bluetooth

#### Mobile wireless networking standard using FHSS (frequency hopping spread spectrum) RF signaling in 2.4-GHz band

## Version 1.1

### Maximum theoretical throughput: 1 Mbps

### Effective throughput: 723 Kbps

### 10 meter node distance

### Designed for PANs (personal area networks)

## Version 2.0 (2004)

### Different encoding schemes

#### 2.1-Mbps throughput

### 30 meters node distance

### Usage: cellular telephones, phone headsets, computer peripherals, PDAs

# Summary of WLAN Standards

Implementing a WLAN

## Designing a small WLAN

### Home, small office

## Formation of larger, enterprise-wide WANs

## Installing and configuring access points and clients

## Implementation pitfalls

### Avoidance

## Material applies to 802.11b and 802.11g

### Most popular

# Determining the Design

## One access point

### Combine with switching, routing functions

### Connects wireless clients to LAN

### Acts as Internet gateway

## Access point WLAN placement considerations

### Typical distances between access point and client

### Obstacles

#### Type, number between access point and clients

## Larger WLANs

### Systematic approach to access point placement

## Site survey

### Assesses client requirements, facility characteristics, coverage areas

### Determines access point arrangement ensuring reliable wireless connectivity

#### Within given area

### Proposes access point testing

#### Testing wireless access from farthest corners

## Install access points

### Must belong to same ESS, share ESSID

## Enterprise-wide WLAN design considerations

### How wireless LAN portions will integrate with wired portions

# Configuring Wireless Connectivity Devices

## Netgear WGR614 (v7)

### Popular, low-cost access point

### Four switch ports, routing capabilities

### Supports 802.11b, 802.11g transmission

## Configuration steps on other small wireless connectivity devices

### Differ somewhat

### Follow similar process, modify same variables

# Configuring Wireless Clients

## Configuration varies from one client type to another

## Windows XP client WLAN configuration

### Use graphical interface

## Linux and UNIX clients wireless interface configuration

### Use graphical interface

### iwconfig command-line function

#### View, set wireless interface parameters

# Avoiding Pitfalls

## Access point versus client configurations

### SSID mismatch

### Incorrect encryption

### Incorrect channel, frequency

### Standard mismatch (802.11 a/b/g/n)

## Incorrect antenna placement

### Verify client within 330 feet

## Interference

### Check for EMI sources

Wireless WANs and Internet Access

## Wireless broadband

### Latest wireless WAN technologies

### Specifically designed for:

#### High-throughput, long-distance digital data exchange

# 802.11 Internet Access

## Access points: 802.11b or 802.11g access methods

## Hot spots

### Places with publicly available wireless Internet access

### Free or subscription

## Hot spot subscription Internet access

### Log on via Web page

### Client software managing client’s connection

#### Network log on, secure data exchange

#### Added security: accept connection based on MAC address

## Accept user’s connection based on MAC address

# 802.16 (WiMAX) Internet Access

## WiMAX (Worldwide Interoperability for Microwave Access)

### Current version: 802.16e (2005)

#### Improved mobility, QoS characteristics

#### Digital voice signals, mobile phone users

## Functions in 2 and 66 GHz range

### Licensed, nonlicensed frequencies

## Line-of-sight paths between antennas

### Throughput potential maximized

## Non-line-of-sight paths

### Exchange signals with multiple stations at once

## Two distinct advantages over Wi-Fi

### Much greater throughput (70 Mbps)

### Much farther range (30 miles)

## Appropriate for MANs and WANs

## Highest throughput achieved over shortest distances between transceivers

## Possible uses

### Alternative to DSL, broadband cable

### Well suited to rural users

### Internet access to mobile computerized devices

### Residential homes

## Metropolitan area installation

### No need for home antenna

## WiMAX MANs

### Extensive connectivity

### Download data rates faster than home broadband connection

### Shared service

#### Apportioned bandwidth

## Drawback

### Expensive

# Clear

## WIMAX provider

## Available in ten cities so far, including Portland and Las Vegas, but not San Francisco yet

### Links: Ch 8f, 8g

# Satellite Internet Access

# Satellite Orbits

## Geosynchronous orbit

### Satellites orbit the Earth at the same rate as the Earth turns

### Downlink

#### Satellite transponder transmits signal to Earth-based receiver

### Typical satellite

#### 24 to 32 transponders

#### Unique downlink frequencies

## LEO (low Earth orbiting) satellites

### Orbit Earth with altitude 100 miles to 1240 miles

### Not positioned over equator

## MEO (medium Earth orbiting) satellites

### Orbit Earth 6000 to 12,000 miles above surface

### Not positioned over equator

#### Latitude between equator and poles

### Advantage

#### Cover larger Earth surface area than LEO satellites

#### Less power, less signal delay than GEO satellites

## Geosynchronous orbiting satellites most popular for satellite Internet access

# Satellite Frequencies

## Five frequency bands

### L-band—1.5–2.7 GHz

### S-band—2.7–3.5 GHz

### C-band—3.4–6.7 GHz

### Ku-band—12–18 GHz

### Ka-band—18–40 GHz

## Within bands

### Uplink, downlink transmissions differ

## Satellite Internet access providers

### Use C- or Ku-bands and Ka-band (future)

# Satellite Internet Services

## Subscriber

### Small satellite dish antenna, receiver

### Exchanges signals with provider’s satellite network

## Satellite Internet access service

### Dial return arrangement (asymmetrical)

#### Receives Internet data via downlink transmission

#### Sends data to satellite via analog modem connection

### Satellite return arrangement (symmetrical)

#### Send, receive data to and from Internet using satellite uplink and downlink

Last modified 10-15-09