# Objectives

## Understand methods of network design unique to TCP/IP networks, including subnetting, CIDR, and address translation

## Explain the differences between public and private TCP/IP networks

## Describe protocols used between mail clients and mail servers, including SMTP, POP3, and IMAP4

## Employ multiple TCP/IP utilities for network discovery and troubleshooting

Designing TCP/IP-Based Networks

## TCP/IP protocol suite use

### Public Internet connectivity

### Private connection data transmission

## TCP/IP fundamentals

### IP: routable protocol

#### Interfaces requires unique IP address

#### Node may use multiple IP addresses

### Two IP versions: IPv4 and IPv6

#### IPv4: older; more common

## IPv4 addresses

### Four 8-bit octets

#### Binary or dotted decimal

## Network host name assignment

### Dynamic using DHCP

### Static

## Network classes: A, B, C, D, E

### Class D, E addresses reserved

### Node’s network class provides information about segment network node belongs to

# Subnetting

## Separates network

### Multiple logically defined segments (subnets)

#### Geographic locations, departmental boundaries, technology types

## Subnet traffic separated from other subnet traffic

## Reasons to separate traffic

### Enhance security

### Improve performance

### Simplify troubleshooting

# Classful Addressing in IPv4

# First, simplest IPv4 addressing type

## Adheres to network class distinctions

## Recognizes Class A, B, C addresses

## Network information (network ID)

### First 8 bits in Class A address

### First 16 bits in Class B address

### First 24 bits in a Class C address

## Host information

### Last 24 bits in Class A address

### Last 16 bits in Class B address

### Last 8 bits in Class C address

## Drawbacks

### Fixed network ID size limits number of network hosts

### Difficult to separate traffic from various parts of a network

| # Class | # Subnet Mask in Binary # Subnet Mask in Decimal | # # Network Bits | # CIDR Notation |
| --- | --- | --- | --- |
| # A | # 11111111 00000000 00000000 00000000 # 255 0 0 0 | # 8 | # /8 |
| # B | # 11111111 11111111 00000000 00000000 # 255 255 0 0 | # 16 | # /16 |
| # C | # 11111111 11111111 11111111 00000000 # 255 255 255 0 | # 24 | # /24 |

# IPv4 Subnet Masks

## Identifies how network subdivided

## Indicates where network information located

## Subnet mask bits

### 1: corresponding IPv4 address bits contain network information

### 0: corresponding IPv4 address bits contain host information

## Network class

### Associated with subnet mask

## ANDing

### Combining bits

#### Bit value of 1 AND another bit value of 1 results in 1

#### Bit value of 0 AND any other bit results in 0

### ANDing logic

#### 1: “true”, 0: “false

## ANDing example

### Address’s fourth octet

#### Any combination of 1s and 0s

#### Results in network ID fourth octet of 0s

# Reserved Addresses

## Cannot be assigned to node network interface; used as subnet masks

## Network ID

### Bits available for host information set to 0

### Classful IPv4 addressing network ID ends with 0 octet

### Subnetting allows network ID with other decimal values in last octet(s)

## Broadcast address

### Octet(s) representing host information equal all 1s

### Decimal notation: 255 (for class  C)

# IPv4 Subnetting Techniques

## Subnetting breaks classful IPv4 addressing rules

### IP address bits representing host information change to represent network information

#### Reduce usable host addresses per subnet

#### Hosts, subnets available after subnetting related to host information bits borrowed

# Subnetting a Class C Address

| # Subnet Mask in Binary # Subnet Mask in Decimal | # # Usable Addr | # CIDR Notation |
| --- | --- | --- |
| # 11111111 11111111 11111111 10000000 # 255 255 255 128 | # 254 | # /25 |
| # 11111111 11111111 11111111 11000000 # 255 255 255 192 | # 62 | # /26 |
| # 11111111 11111111 11111111 11100000 # 255 255 255 224 | # 30 | # /27 |
| # 11111111 11111111 11111111 11110000 # 255 255 255 240 | # 14 | # /28 |
| # 11111111 11111111 11111111 11111000 # 255 255 255 248 | # 6 | # /29 |
| # 11111111 11111111 11111111 11111100 # 255 255 255 252 | # 2 | # /30 |

# Calculating IPv4 Subnets

## Formula: # Usable Addresses =  2n −2

### n: number of subnet mask bits that are  zero

## Example

### Class C network

#### Network ID: 199.34.89.0

#### Want to divide into six subnets

## Class A, Class B, and Class C networks

### Can be subnetted

#### Each class has different number of host information bits usable for subnet information

#### Varies depending on network class and the way subnetting is used

## LAN subnetting

### LAN’s devices interpret device subnetting information

### External routers

#### Need network portion of device IP address

Figure 10-3 A router connecting several subnets

# CIDR (Classless Interdomain Routing)

## Also called classless routing or supernetting

## Not exclusive of subnetting

### Provides additional ways of arranging network and host information in an IP address

### Conventional network class distinctions do not exist

## Example: subdividing Class C network into six subnets of 30 addressable hosts each

## Supernet

### Subnet created by moving subnet boundary left

# Subnet and Supernet Masks

## Suppose you are starting with a class C address like 192.168.0.0/24

## This would be a subnet:

### Subnet Address: 192.168.0.0 / 26

### Subnet Mask:	11111111 11111111 11111111 11000000

### 255      255      255      192

### Usable address range: 192.168.0.1 - 192.168.0.62

## This would be a supernet:

### Subnet Address: 192.168.0.0 / 22

### Subnet Mask:	11111111 11111111 11111100 00000000

### 255      255      252        0

### Usable address range: 192.168.0.1 - 192.168.3.254

## Example: class C range of IPv4 addresses sharing network ID 199.34.89.0

### Need to greatly increase number of default host addresses

## CIDR notation (or slash notation)

### Shorthand denoting subnet boundary position

### Form

#### Network ID followed by forward slash ( / ), followed by number of bits used for network portion of the address

#### Like 147.144.51.0/24

# Internet Gateways

## Gateway

### A device that connects two networks or subnets

## Default gateway

### Sends LAN traffic to other subnets, such as the Internet

### Usually a router

## Network nodes

### Have one default gateway address

#### Assigned manually or automatically (DHCP)

# Address Translation

## Public network

### Any user may access

#### Little or no restrictions

## Private network

### Access restricted

#### Clients, machines with proper credentials

### Hiding IP addresses

#### Provides more flexibility in assigning addresses

NAT (Network Address Translation)

SNAT

### Gateway replaces client’s private IP address with Internet-recognized IP address

## Reasons for using address translation

### Overcome IPv4 address quantity limitations

### Add marginal security to private network when connected to public network

### Develop network addressing scheme

## SNAT (Static Network Address Translation)

### Client associated with one private IP address, one public IP address

#### Never changes

### Useful when operating mail server

## DNAT (Dynamic Network Address Translation)

### Also called IP masquerading

### Internet-valid IP address might be assigned to any client’s outgoing transmission

## PAT (Port Address Translation)

### Each client session with server on Internet assigned separate TCP port number

#### Client server request datagram contains port number

### Internet server responds with datagram’s destination address including same port number

## NAT

### Separates private, public transmissions on TCP/IP network

## Gateways conduct network translation

### Most networks use router

## Gateway might operate on network host

### Windows operating systems

#### ICS (Internet Connection Sharing)

TCP/IP Mail Services

## E-mail

### Most frequently used Internet services

### Functions

#### Mail delivery, storage, pickup

## Mail servers

### Communicate with other mail servers

### Deliver messages, send, receive, store messages

## Mail clients

### Send messages to; retrieve messages from mail servers

# SMTP (Simple Mail Transfer Protocol)

## Used to send email

### From one mail server to another

## Operates at Application layer

### Relies on TCP at Transport layer

### Uses TCP port 25

## Email client software

### Administrator must specify SMTP server name

#### Such as smtp.gmail.com

# MIME (Multipurpose Internet Mail Extensions)

## SMPT drawback: ASCII characters only

## MIME standard encodes, interprets binary files, images, video, non-ASCII character sets within e-mail message

### Identifies each mail message element according to content type

#### Text, graphics, audio, video, multipart

## Does not replace SMTP

### Works in conjunction with it

## Encodes different content types

### Fools SMTP

# POP (Post Office Protocol)

## Application layer protocol

### Retrieve messages from mail server

## POP3 (Post Office Protocol, version 3)

### Current, popular version

### Relies on TCP, operates over port 110

### Store-and-forward type of service

## Advantages

### Minimizes server resources

#### Mail deleted from server after retrieval

#### Disadvantage for mobile users

### Mail server, client applications support POP3

# IMAP (Internet Message Access Protocol)

## More sophisticated alternative to POP3

## IMAP4: current version

## Advantages

### Replace POP3 without having to change e-mail programs

### E-mail stays on server after retrieval

#### Good for mobile users

## Features

### Users can retrieve all or portion of mail message

### Users can review messages and delete them

#### While messages remain on server

### Users can create sophisticated methods of organizing messages on server

### Users can share mailbox in central location

## Disadvantages

### Requires more storage space, processing resources than POP servers

### Network managers must watch user allocations closely

### If the IMAP4 server fails, users cannot access mail

Additional TCP/IP Utilities

## TCP/IP transmission process

### Many points of failure

#### Increase with network size, distance

## Utilities

### Help track down most TCP/IP-related problems

### Help discover information about node, network

## Nearly all TCP/IP utilities

### Accessible from command prompt

### Syntax differs per operating system

# Ipconfig

## Command-line utility providing network adapter information

### IP address, subnet mask, default gateway

## Windows operating system tool

### Command prompt window

#### Type ipconfig and press Enter

### Switches manage TCP/IP settings

#### Forward slash ( / ) precedes command switches

## Requires administrator rights

### To change workstation’s IP configuration

# IPCONFIG | MORE

## Useful in Vista and Windows 7

## Because they have a lot of extra virtual network adapters

# Renewing a IP Address

## IPCONFIG /RELEASE

### Ends the current IP address lease

## IPCONFIG /RENEW

### Asks DHCP for a new IP address

## These commands are often required when moving from one LAN to another

# Ifconfig

## Utility used on UNIX and Linux systems

### Modify TCP/IP network interface settings, release and renew DHCP-assigned addresses, check TCP/IP setting status

### Runs at UNIX, Linux system starts

#### Establishes computer TCP/IP configuration

## Used alone or with switches

### Uses hyphen ( - ) before some switches

### No  preceding character for other switches

# Netstat

## Displays TCP/IP statistics, component details, host connections

## Used without switches

### Displays active TCP/IP connections on machine

## Can be used with switches

### -a   shows all connections including listening ones

### -n   display raw port numbers instead of letters like http

## For help, use NETSTAT /?

# Nbtstat

## NetBIOS

### Used on Windows LANs

### Protocol runs in Session and Transport layers

### Associates NetBIOS names with workstations

### Not routable

#### Can be made routable by encapsulation

## Nbtstat utility

### Provides information about NetBIOS statistics

### Resolves NetBIOS names to IP addresses

#### Limited use as TCP/IP diagnostic utility

# Hostname and Nslookup

## Hostname utility

### Provides client’s host name

#### Administrator may change

## Nslookup

### Query DNS database from any network computer

#### Find the device host name by specifying its IP address

### Verify host configured correctly; troubleshoot DNS resolution problems

# Dig

## Domain information groper

## Similar to nslookup

### Query DNS database

### Find specific IP address host name

## Useful for diagnosing DNS problems

## Dig utility provides more detailed information than nslookup

## Flexible: two dozen switches

## Included with UNIX, Linux operating systems

## Windows system: must obtain third party code

# Whois

## Query DNS registration database

### Obtain domain information

## Troubleshoot network problems

## Syntax on Linux or Unix

### whois xxx.yy

#### xxx.yy is second-level domain name

## Windows system

### Requires additional utilities

## Web sites provide simple, Web-based interfaces

# Traceroute

## Windows-based systems: tracert

## Linux systems: tracepath

## ICMP ECHO requests

### Trace path from one networked node to another

### Identifying all intermediate hops between two nodes

## Transmits UDP datagrams to specified destination

### Using either IP address or host name

#### To identify destination

## Command used a number of switches

# Mtr (my traceroute)

## UNIX, Linux operating systems

### Route discovery, analysis utility

### Combines ping, traceroute functions

#### Output: easy-to-read chart

## Simplest form

### mtr ip_address or mtr host_name

#### Run continuously

#### Stop with Ctrl+C or add limiting option to command

## Number of switches refine functioning, output

## Results misleading

### If devices prevented from responding to ICMP traffic

## Windows XP, Vista, Server 2003, Server 2008

### Pathping program as command-line utility

### Similar switches as mtr

### Pathping output differs slightly

#### Displays path first

#### Then issues hundreds of ICMP ECHO requests before revealing reply, packet loss statistics

# Route

## Route utility

### Allows viewing of host’s routing table

## UNIX or Linux system

### Type route and press Enter

## Windows-based system

### Type route print and press Enter

## Cisco-brand router

### Type show ip route and press Enter

## Route command

### Add, delete, modify routes

## Route command help

### UNIX or Linux system

#### Type man route and press Enter

### Windows system

#### Type route ? and press Enter

Figure 10-16 Example routing table

Table 10-6 Fields in routing table on a UNIX host

Last modified 10-28-09