# Objectives

## Use terminology specific to converged networks

## Explain VoIP (voice over IP) services and their user interfaces

## Explain video-over-IP services and their user interfaces

## Describe VoIP and video-over-IP signaling and transport protocols, including SIP, H.323, and RTP

## Understand QoS (quality of service) assurance methods critical to converged networks, including RSVP and DiffServ

Terminology

# IP Telephony

## IP telephony (VoIP)

### Any network carrying voice signals using TCP/IP

#### Public or private

### Runs over any packet-switched network

## VoATM (voice over ATM)

## VoDSL (voice over DSL)

## Data connection types carrying VoIP signals

### T-carriers, ISDN, broadband cable, satellite connections, WiFi, WiMAX, cellular telephone networks

# IP Telephony

## Internet telephony

### VoIP relies on Internet

### Advantage: breadth and low cost

## Private lines can also carry VoIP calls

### Effective and economical

### Network congestion control capabilities

#### Better sound quality

# Non-Data Applications

## Other non-data applications on converged networks

### FoIP (Fax over IP)

### IPTV (IP television)

### Videoconferencing

#### Multiple participants communicate and collaborate via audiovisual means

### Streaming video

#### Compressed video delivered in continuous stream

#### Hulu.com for free Internet TV shows

### Webcasts

#### Streaming videos supplied via the Web

# Terminology

## Multicasting

### One node transmits same content to every client in group

## Video over IP

### IPTV, videoconferencing, streaming video, IP multicasting

## Unified communications (unified messaging) service

### Several communication forms available from single user interface

VoIP (Voice over IP) Applications and Interfaces

# VoIP (Voice over IP) Applications and Interfaces

## Significant VoIP implementation reasons

### Lower voice call costs (toll bypass)

### New, enhanced features and applications

### Centralize voice and data network management

## Voice and data configurations

### Traditional telephone (sends, receives analog signals)

### Telephone specially designed for TCP/IP transmission

### Computer with microphone, speaker, VoIP client software

### Mixture

# Analog Telephones

## Using traditional telephone, digital VoIP connection

### Must convert voice signals into bits

## Analog-to-digital conversion

### Codec

#### Method to accomplish analog signal compressing, encoding

### ATA (analog telephone adapter)

#### Card within computer workstation, externally attached device

#### Telephone line connects to RJ-11 adapter port

#### Converts analog voice signals to IP packets

# VoIP-Enabled Router

## Connect

### Analog telephone line to VoIP-enabled device (switch, router, gateway)

## Convert

### Analog voice signals into packets

# Digital PBX (IP-PBX)

### PBX (private branch exchange): telephone switch connecting calls within a private organization

## IP-PBX

### Private switch

### Accepts, interprets both analog and digital voice signals

### Connects with traditional PSTN lines, data networks

### Transmits, receives IP-based voice signals to and from other network connectivity devices

### Packaged with sophisticated software

# Analog PBX with Gateway

## Traditional telephone connects to analog PBX

### Then connects to voice-data gateway

## Gateway connects traditional telephone circuits with TCP/IP network

### Internet or private WAN

## Gateway

### Digitizes incoming analog voice signal

### Compresses data

### Assembles data into packets

### Issues packets to packet-switched network

# IP Telephones

## IP telephones (IP phones)

### Transmit, receive only digital signals

### Voice immediately digitized, issued to network in packet form

### Requires unique IP address

### Looks like traditional touch-tone phone

#### Connects to RJ-45 wall jack

#### Connection may pass through connectivity device before reaching IP-PBX

## IP telephones: popular and unique features

### Web browser screens

### Connect to user’s PDA

### Speech recognition capabilities

### Busy line instant message option

### Accept emergency messages

## IP telephones: mobility

### Move phone to new office; connect to wall jack; accept and make calls

### Faster than traditional PBX extension reprogramming

## Conventional analog telephone

### Obtains current from local loop

#### Signaling (ring, dial tone)

## IP telephones: issues

### Need electric current

### Not directly connected to local loop

### Require separate power supply

#### Susceptible to power outages

#### Requires assured backup power sources

## IP telephones may obtain current using PoE (power over Ethernet)

# Softphones

## Computer programmed to act like IP telephone

### Softphones and IP telephones

#### Provide same calling functions

#### Connect to network; deliver services differently

### Prerequisites

#### Computer minimum hardware requirements

#### IP telephony client installed

#### Digital telephone switch communication

#### Full-duplex sound card

#### Microphone, speakers

### Example: Skype

## Graphical interface

### Presented after user starts softphone client software

### Customizable

## Difference between IP telephones and softphones

### Softphones have versatile connectivity

#### VoIP solution for traveling employees and telecommuters

### Softphones have convenient and localized call management

#### Call tracking: date, time, duration, originating number, caller names

#### Simplifies recordkeeping and billing

Video-over-IP Applications and Interfaces

# Video-over-IP Applications and Interfaces

## Cisco Systems estimate

### By 2011, 60 percent of Internet traffic will be video traffic

## Requirements for prediction to come true

### Continual price drop in video-over-IP hardware

### Networks must further augment capacities, reliability

## Video-over-IP services categories

### Streaming video, IPTV, videoconferencing

## Video-over-IP transmission

### Digitize audio, visual signals

#### Use video codecs

# Streaming Video

## Simplest among all video-over-IP applications

### Basic computer hardware, software requirements

## Video-on-demand

### Files stored on video streaming server

#### Popular

#### Viewer chooses video when convenient: Web browser

## Streaming video

### Video issued live

#### Directly: source to user

## Streaming video

### Video issued live

#### Directly from source to user

### Drawbacks

#### Content may not be edited before distribution

#### Viewers must connect with stream when issued

## Video-on-demand benefits

### Content viewed at user’s convenience

### Viewers control viewing experience

#### Pausing, rewinding, fast-forwarding

## Considerations

### Number of clients receiving each service

#### Point-to-point video over IP

#### Point-to-multipoint video over IP (not necessarily multicast transmission)

### Network type classification

#### Private

#### Public

## Most streaming video

### Takes place over public networks

# IPTV (IP Television)

## Telecommunications carriers, cable companies are upgrading networks to deliver high-bandwidth Internet connections

### IPTV digital television signals is a valuable added service

## Elements

### Telco accepts video content at a head end

### Telco’s CO (central office)

#### Servers provide management services

#### Video channel assigned to multicast group

## Advantages of IPTV multicasting

### Simple to manage content delivery

#### Issue one multicast transmission to entire group

### Local loop capacity issues

#### Most rely on copper to home (limits throughput)

#### Sending all channels could overwhelming local loop

#### Solution: Telco transmits only content ordered

## IGMP (Internet Group Management Protocol)

### Manages multicasting

### Routers communicate using multicast routing protocol

## Compressed, digital video signal travels like data signal

### Over technologies like DSL or WIMAX

## Advantage

### Telecommunications carrier, cable company

#### Control connection end to end

#### Better monitor and adjust QoS

## Set top box

### Decodes video signal, issues to television

### Manages delivery

#### Communicating with content servers

### Manage services

#### Pay per-view, video-on-demand programming

# Videoconferencing

## Unidirectional video-over-IP services

### Video delivered to user who only watches content

## Videoconferencing

### Full-duplex connections

#### Participants send and receive audiovisual signals

### Real time

### Benefits

#### Cost savings, convenience

#### Replace face-to-face business meetings

#### Allows collaboration

## Videoconferencing uses

### Telemedicine

### Tele-education

### Judicial proceedings

### Surveillance (usually unidirectional)

## Hardware, software requirements

### Computer workstation

#### Means to generate, send, receive audiovisual signals

### Video terminal, video phone

# Video Bridge

## Video bridge

### Manages multiple audiovisual sessions

#### Participants can see, hear each other

### Conference server

#### Hardware or software

## Leased Internet-accessible video bridging services

### Occasional videoconference use

## Video bridge depends on signaling protocols

Signaling Protocols

# Signaling Protocols

## Signaling

### Information exchange

#### Between network components, system

#### Establishing, monitoring, releasing connections

#### Controlling system operations

### Set up, manage client sessions

### Perform several functions

## Early VoIP

### Proprietary signaling protocols

## Today

### Standardized signaling protocols

# SS7 (not VoIP)

## Signaling System 7

### The protocol used to set up most of the world's PSTN calls (not VoIP calls)

### Sets up connections between phone switches to route calls

### Handles ringing, answering, billing, SMS, and other services

# H.323 (used for VoIP)

## Describes architecture and a group of protocols

### Establishing, managing packet-switched network multimedia sessions

## Supports voice, video-over-IP services

## Terms

### H.323 terminal -- a node, such as an IP Phone

### H.323 gateway -- connects different signaling protocols, such as H.323 and SS7

### H.323 gatekeeper -- nerve center, handles billing, accounting, bandwidth allocation, etc.

### MCU (multipoint control unit) -- serves several H.323 terminals, like a video bridge

### H.323 zone -- the devices managed by a single H.323 gatekeeper

# H.323 Zone
(gatekeeper is not shown)

## H.225 and H.245 signaling protocols

### Specified in H.323 standard

### Operate at Session layer

## H.225 handles call or videoconference signaling

## H.245 ensures correct information type formatting

### Uses logical channels

## H.323 standard specifies

### Protocol interoperability

#### Presentation layer: coding, decoding signals

#### Transport layer

## 1996 ITU codified

### Multiservice signaling open protocol

## Early version

### Suffered slow call setup

## Second version (H.323.2)

### VoIP networks popular call signaling protocol

## Third version

### Not widely accepted yet

#### SIP attracted network administrators attention

# SIP (Session Initiation Protocol) (Used for VoIP)

## Performs functions similar to H.323

## Version 2.0 (RFC 2543)

### Codified by the IETF in 1999

#### Application layer signaling, multiservice control protocol, packet-based networks

## Goals

### Modeled on HTTP protocol

### Reuse existing TCP/IP protocols

#### Session management, enhanced services

### Modular and specific

# SIP Terms

## User agent

### Any node sending or receiving SIP messages

## User agent client

### End-user device, like an IP Phone

## User agent server

### Servers that provide SIP signals to many clients

## Registrar server

### Knows network addresses of its user agents

## Proxy server

### Makes connections within the SIP domain

## Redirect server

### Connects to external SIP domains

# Common SIP Servers

## Asterisk -- open-source and free

## Proprietary SIP systems available from

### 3Com

### Avaya

### Cisco

### Nortel

### others

# SIP is Replacing H.323

## VoIP vendors prefer SIP over H.323

### Simplicity

### Fewer instructions to control call

### Consumes fewer processing resources

### More flexible

## SIP and H.323

### Regulate call signaling, control for VoIP or video-over-IP clients and servers

### Not used for communication between media gateways

# MGCP (Media Gateway Control Protocol) and MEGACO (H.248)

## Gateways

### Integral to converged networks

#### Media gateway, fax gateway

## “Payload” use

### Different channels from control signals

### Different logical and physical paths than signals

## Expedites information handling

### Use separate physical paths

### Gateways still need to exchange and translate

#### Use signaling and control information

# Media Gateway Controller

## MGC (media gateway controller)

### Computer managing multiple media gateways

### Selects the gateway and physical media to use for payload and control packets

### Also called a softswitch

### Advantageous on large VoIP networks

## MGCs can communicate with media gateways with either the MGCP or MEGACO protocols

# MGCP (Media Gateway Control Protocol)

## MGCP (Media Gateway Control Protocol): RFC 3435

### Multiservice networks supporting many media gateways

### Operate with H.323 or SIP

# MEGACO

## MEGACO

### Newer than MGCP

### Performs same functions as MGCP

### Different commands and processes

### Operates with H.323 or SIP

### Superior to MGCP

### Supports ATM

### Developed by ITU and IETF

Transport Protocols

# Transport Protocols

## Signaling protocols like MEGACO, MGCP, H.323, and SIP only set up and tear down connections

## Different protocols deliver voice or video payload

## Review of Transport layer protocols

### TCP: connection oriented

#### Delivery guaranteed, but slower

### UDP: connectionless

#### No accountability; preferred in real-time applications

#### Packet loss tolerable if additional protocols overcome UDP shortcomings

# RTP (Real-time Transport Protocol)

## RFC 1889

### Operates at Application layer

### Relies on UDP

## Applies sequence numbers to indicate:

### Destination packet assembly order

### Packet loss during transmission

## Assigns packet timestamp

### Receiving node

#### Compensates for network delay, synchronize signals

## No mechanism to detect success--for that, it relies on RTCP

# RTCP (Real-time Transport Control Protocol)

## RFC 3550

## Provides quality feedback to participants

### Packets transmitted periodically

### Measures # packets lost or delayed

### Cannot correct those problems--QoS does that

## RTCP is optional

QoS (Quality of Service) Assurance

# QoS (Quality of Service) Assurance

## QoS: measures network performance

### High QoS: uninterrupted, accurate, faithful reproduction

### Low (poor) QoS: distorted VoIP, video over IP

## To ensure consistent QoS

### Requires more dedicated bandwidth

### Requires techniques ensuring high QoS

#### RSVP

#### DiffServ

#### MPLS

# RSVP (Resource Reservation Protocol)

## RFC 2205

## Transport layer protocol

## Before transmission

### Reserves network resources

## Creates a path between sender, receiver

### Provides sufficient bandwidth

#### Signal arrives without delay

## Issues PATH statement via RSVP to receiving node

### Indicates required bandwidth, expected service level

## Two service types

### Guaranteed service

### Controlled-load service

## Router marks transmission’s path

### Routers issue PATH message

### Destination router issues Reservation Request (RESV) message

#### Follows same path in reverse

#### Reiterates information

#### Routers allocated requested bandwidth

#### Sending node transmits data

## RSVP messaging

### Separate from data transmission

### Does not modify packets

### Specifies and manages unidirectional transmission

#### Resource reservation process takes place in both directions

## RSVP Emulates circuit-switched path

### Provides excellent QoS

## Drawback: high overhead--consumes a lot of network resources

### Acceptable on small networks

### Larger networks use DiffServ

# DiffServ

## Addresses traffic prioritization QoS issues

## Differs from RSVP

### Modifies actual IP datagrams that contain payload

### Assigns priority to all network traffic

## Time-sensitive voice and video services

### Offers more protection

## To prioritize traffic

### IPv4 datagram: DiffServ field

### IPv6 datagram: Traffic Class field

# IP Header

## From Wikipedia (link Ch 4c)

## Two forwarding types

### EF (Expedited Forwarding)

#### Data stream assigned minimum departure rate

#### Circumvents delays

### AF (Assured Forwarding)

#### Data streams assigned different router resource levels

#### Prioritizes data handling

#### On time, in sequence packet arrival: no guarantee

# MPLS (Multiprotocol Label Switching)

## Modifies data streams at Network layer

## A first router data stream encounters

### Prefixes IP datagram header with label

#### Packet forwarding information

### Router’s data stream path revises label

#### Indicates next hop

### Considers network congestion

## Very fast forwarding: no delay

### Destination IP address compared to routing tables

#### Forward data to closest matching node

Last modified 11-4-09