# Objectives

## Describe the steps involved in an effective troubleshooting methodology

## Follow a systematic troubleshooting process to identify and resolve networking problems

## Document symptoms, solutions, and results when troubleshooting network problems

## Use a variety of software and hardware tools to diagnose problems

Troubleshooting Methodology

# Troubleshooting Methodology

## Proceed logically and methodically

## Follow recommended steps

## Use experience when necessary

## Logical approach benefits

### Prevents wasteful, time-consuming efforts

#### Unnecessary software, hardware replacements

## Troubleshooting steps

### Identify symptoms, problems

### Identify affected area

### Determine what has changed

### Establish most probable cause

### Determine if escalation necessary

### Create action plan, solution

### Implement solution, test result

### Identify results, effects

### Document solution, process

# Identify the Symptoms and Problems

## Ask questions

### Answers help identify network problem symptoms

## Avoid jumping to conclusions about symptoms

## Pay attention

### Users, system and network behaviors, and error messages

### Treat each symptom uniquely

# Identify the Affected Area

## Determine affected area and group

### One user or workstation?

### A workgroup?

### A department?

### One location within an organization?

### An entire organization?

## Narrow down time frame

### When did the problem begin?

### Has the network, server, or workstation ever worked properly?

### Did the symptoms appear in the last hour or day?

### Have the symptoms appeared intermittently for a long time?

### Do the symptoms appear only at certain times of the day, week, month, or year?

## Benefits

### Eliminates some causes; points to other

## Take time to troubleshoot correctly

### Ask specific questions

#### Identify problem scope

### Filter unrelated user information

## Discover time and frequency of problem

### May reveal more subtle network problems

## Identify affected problem area

### Leads to next troubleshooting steps

# Determine What Has Changed

## Become aware of recent network changes

### Ask questions pinpointing problem resulting from network change

## Possible actions if change generated problem

### Correct problem

### Reverse change

#### Less risky, less time consuming

## Network change records

### Track what has changed

## Make records available to staff members

# Establish the Most Probable Cause

## Close to determining problem cause

## Verify user competency

### Ensure human error is not problem source

### Verify user is performing network tasks correctly

#### Watch

#### Connect using remote desktop software

#### Talk over phone

## Results

### Catch user-generated mistakes

### Gain clues for further troubleshooting

## Re-create the problem

### Learn more about problem causes

### Reproduce symptoms

#### Log on as user; log on under privileged account

#### Follow same steps

### Ask questions

#### Determine whether problem symptoms reproducible, to what extent

#### Ask user precisely what was done before error

## Verify Physical layer connectivity

### Half of all network problems occur at Physical layer

### Symptoms of Physical layer problems

#### Segment, network lengths exceed standards

#### Noise

#### Improper terminations, faulty connectors, loose connectors, poorly crimped connections

#### Damaged cables

#### Faulty NICs

### Software errors may point to physical connectivity problem

### Diagnosing Physical layer Problems

#### Ask questions

#### Verify connections between devices

#### Verify soundness of connection hardware

### Swapping equipment: tests theories

#### Exchange faulty component for functional one

#### Change patch cable

#### Better option: build in redundancy

## Verify logical connectivity

### Review questions identifying logical connectivity problem

### Logical connectivity problems more difficult to isolate, resolve than physical connectivity problems

### Software-based causes

#### Resource conflicts with NICs configuration

#### Improperly configured NIC

#### Improperly installed, configured client software

#### Improperly installed, configured network protocols, services

# Determine Whether Escalation is Necessary

## Help desks

### Help desk analysts

#### Proficient in basic workstation, network troubleshooting

#### First-level support

### Network specialist

#### Second-level support

### Help desk coordinator

#### Third-level support personnel

## Know when and how to escalate

### Follow given procedure

# Create an Action Plan and Solution Including Potential Effects

## Consider how solution affects users, network functionality

## Scope

### Assess solution’s scope

#### Before implementing solution

### Wait if not an emergency

## Trade-offs

### Restore functionality for one user group

#### May remove functionality for others

## Security

### Be aware of inadvertent security implications

#### Network access, resource privileges addition or removal consequences

### Before upgrade, patch installation

#### Understand access changes for authorized, unauthorized users

## Scalability

### Solution types

#### Position network for future additions, enhancements

#### Temporary fix organization will outgrow

## Cost

### Weigh options carefully

### Consider poor network performance identification

#### Address those areas separately

## Use vendor information

### Manufacturer documentation

### Free online troubleshooting information

#### Searchable databases

#### Sophisticated web interfaces for troubleshooting their equipment

#### Vendor’s technical phone support

#### Consult with others, within, outside your organization

# Implement and Test the Solution

## Implement only after proposed solution’s effects researched

## Implementation

### Brief or long

### Requires foresight, patience

### Use methodical and logical approach

#### Leads to efficient correction process

#### Unless major problem – fix as quickly as possible

### Follow series of steps

#### Implement a safe, reliable solution

## Roll out solution in stages

### Especially in large-scale fixes

## After solution implementation

### Verify problem solved properly

### Testing dependencies

#### Solution itself

#### Area affected by problem

#### Inability to test immediately

# Identify the Results and Effects of the Solution

## After testing solution implementation

### Determine how and why solution was successful

### Determine solution’s effect on users and functionality

### Avoid creating unintended, negative consequences

### Communicate solution to colleagues

# Document the Solution and Process

## Always record:

### Problem symptoms and cause (or causes)

### Solution

## Justification for recording

### Impossible to remember each incident’s circumstances

### Job changes

## Use centrally located database

### Accessible to all networking personnel

## Call tracking system (help desk software)

### User-friendly, graphical

#### Prompts for problem information

### Assigns unique problem number

### Highly customizable

### If not available, use simple electronic form

## Supported services list document

### Lists all supported service, software and contacts

## Follow-up with user

# Help Desk Software Demo

## Notify others of changes

### Record resolution in call tracking system

### Communication

#### Alerts others about problem, solution

#### Notifies others of network changes made

### Many types of changes to record

#### Not necessary to record minor modifications

# Help to Prevent Future Problems

## Avert network problems with

### Network maintenance

### Documentation

### Security

### Upgrades

Troubleshooting Tools

# Troubleshooting Tools

## Utilities help troubleshoot network problems

### Ping

### Specialized tools

#### Specifically designed to analyze, isolate network problems

#### Simple continuity testers

#### Protocol analyzers

### Tool selection dependencies

#### Problem being investigated

#### Network characteristics

# Crossover Cable

## Crossover cable

### Reverses transmit and receive wire pairs in one connector

#### Directly interconnect two nodes without intervening connectivity device (hub or switch)

### Use

#### Verify NIC transmitting, receiving signals properly

# Tone Generator and Tone Locator

## Ideal situation

### Telecommunications closet ports, wire terminations all labeled properly

## Reality

### Telecommunications closet disorganized, poorly documented

## Tone generator (toner)

### Issues signal on wire pair

## Tone locator (probe)

### Emits tone when electrical activity detected

## Probe kit

### Generator and locator combination

### Fox and hound

## Testing requires trial and error

## Use

### Determine where wire pair terminates

## Not used to determine cable characteristics

# Multimeter

## Tests cables for faults

### Isolates problems with network cables

## Multimeter

### Measures electric circuit characteristics

#### Resistance and voltage

## Voltmeter

### Measures electric circuit pressure, voltage

#### Voltage creates signals over network wire

## Resistance

### Fundamental wire property

#### Dependent on wire’s molecular structure, size

### Measured in ohms

#### Using ohmmeter

## Impedance

### Resistance contributing to controlling signal

### Measured in ohms

### Telltale factor for ascertaining where cable faults lie

### Some required for proper signal transmission and interpretation

## Multimeter

### Separate instruments for measuring wires impedance, resistance, voltage

### Has several uses

### Sophistication, features, and costs vary

# Cable Continuity Testers

## Cable checkers (continuity testers, cable testers)

### Tests whether cable carrying signal to destination

### Tests cable continuity

## Copper-based cable tester

### Consists of two parts

#### Base unit generates voltage

#### Remote unit detects voltage

## Series of lights, audible tone

### Signal pass/fail

## Verify UTP, STP wires paired correctly

### Not shorted, exposed, crossed

## Fiber optic continuity tester

### Issues light pulses on fiber

### Determines whether pulses reached other end

### Tests the cable

## Test all cables to ensure meeting network’s required standards

### Homemade or purchased

## Offer convenience: portable, lightweight, low cost

# Cable Performance Testers

## Determines if cable carrying current

## Continuity testers versus performance testers

### Sophistication and price

### Performance tester accomplishes same continuity, fault tests

### Continuity testers perform additional tasks

## Sophisticated performance testers

### Include TDR (time domain reflectometer)

#### Issue signal, measures signal bounce back

#### Indicate proper terminator installation, function, node distance

## Fiber-optic connections testers

### Use OTDRs (optical time domain reflectometers)

#### Transmit light-based signals of different wavelengths over fiber

### OTDRs

#### Measure fiber length

#### Determine faulty splice locations, breaks, connectors, bends

#### Measure attenuation over cable

## Expensive

# Voltage Event Recorders

## Voltage event

### Any condition where voltage exceeds or drops below predefined levels

## Voltage event recorder

### Collects data about power quality

#### Downloaded to workstation

#### Analyzed by software

### Cost up to $5000

# Butt Set

## Lineman’s handset, telephone test set

### Butt into telephone conversation

### Rugged, sophisticated telephone

## Uses

### Determine if line functioning

#### Receiving signal, picking up noise affecting signal

### Sophisticated butt sets

#### Perform rudimentary cable testing

### Detecting dial tone on line

# Network Monitors

## Software-based tool

### Continually monitors network traffic from server, workstation attached to network

### Interprets up to Layer 3

### Determine protocols passed by each frame

### Cannot interpret frame data

## Included in NOS

### Microsoft’s Network Monitor

## Purchase or free download tools developed by other software companies

### All use similar graphical interfaces

## Network adapter

### Must support promiscuous mode

## Common terms for abnormal data patterns, packets

### Local collisions

### Late collisions

### Runts

### Giants

### Jabber

### Negative frame sequence checks

### Ghosts

# Protocol Analyzers

## Protocol analyzer (network analyzer)

### Captures traffic, analyzes frames

#### Typically to Layer 7

## Variety of protocol analyzer software available

### Wireshark

## Offers versatility in information type, depth

### Could collect more information than can be reasonably processed

### Set filters to avoid

## Before using network monitor or protocol analyzer

### Learn what network traffic normally looks like

#### Capture data for time period on regular basis

# Wireless Network Testers

## Tools containing wireless NICs, running wireless protocols

## Workstation wireless network connection properties

### Shows a little about wireless environment

#### Connection duration, signal speed and strength, number of packets exchanged

### Only applies to one workstation

## Programs scan for wireless signals

### Discover access points, wireless stations transmitting

### Free

# InSSIDer

## Commercial tools advantages

### Portability

### Installed with wireless network analysis tools

### Contain more powerful antennas

Last modified 11-18-09