# Objectives

## Identify the characteristics of a network that keep data safe from loss or damage

## Protect an enterprise-wide network from viruses

## Explain network- and system-level fault-tolerance techniques

## Discuss issues related to network backup and recovery strategies

## Describe the components of a useful disaster recovery plan and the options for disaster contingencies

What Are Integrity and Availability?

# What Are Integrity and Availability?

## Integrity

### Network’s programs, data, services, devices, connections soundness

## Availability

### How consistently, reliably a file or system can be accessed

#### By authorized personnel

## Both are compromised by:

### Security breaches, natural disasters, malicious intruders, power flaws, human error

## User error

### Unintentional

#### Harm data, applications, software configurations, hardware

### Intentional

#### Administrators must take precautionary measures to protect network

## Cannot predict every vulnerability

## Follow general guidelines for protecting network

# General Guidelines for Protecting your Network

## Allow only network administrators to change NOS and application system files

## Monitor the network for unauthorized access or changes

## Record authorized changes in a change management system

## Install redundant components

## Perform regular health checks

# General Guidelines for Protecting your Network

## Check system performance, error logs, and system log book

## Keep backups, boot disks, and emergency repair disks current and available

## Implement and enforce security and disaster recovery policies

Malware

# Malware

## Program or code

### Designed to intrude upon or harm system and resources

## Examples: viruses, Trojan horses, worms, bots

## Virus

### Replicating program intent to infect more computers

#### Through network connections, exchange of external storage devices

## Many destructive programs often called viruses

### Do not meet strict criteria of virus

### Example: Trojan horse

# Types of Malware

## Boot sector viruses

### Infects the boot sector of hard disks or floppies

### Runs when the computer is started

## Macro Virus

### Infects Microsoft Office files

## File-infector virus

### Infects executable files

## Worm

### Runs independently, not attached to a file

### Spreads through attachments to email, instant messages,  or other file transfer

# Types of Malware

## Trojan horse

### Claims to be something desirable, but hides something harmful

## Network Virus

### Propagates through network protocols, like FTP

## Bot

### Makes your computer a zombie--remotely controlled by a criminal, through IRC or some other network channel

### A group of those computers form a botnet, controlled by a botmaster (link Ch 14a)

# Malware Characteristics

## Making malware harder to detect and eliminate

### Encryption

#### Used by viruses, worms, Trojan horses

#### Thwart antivirus program’s attempts to detect it

### Stealth

#### Malware hides itself to prevent detection

#### Disguise themselves as legitimate programs, code

### Polymorphism

#### Change characteristics every time they transfer to new system

#### Use complicated algorithms, incorporate nonsensical commands

### Time dependence

#### Programmed to activate on particular date

#### Can remain dormant, harmless until date arrives

#### Logic bombs: programs designed to start when certain conditions met

## Malware can exhibit more than one of these characteristics

# Malware Protection

## Not just installing any virus-scanning program or anti-malware software

## Requires:

### Choosing appropriate anti-malware program

### Monitoring network

### Continually updating anti-malware program

### Educating users

# Anti-Malware Software

## Malware leaves evidence

### Some detectable only by anti-malware software

### User viewable symptoms

#### Unexplained file size increases

#### Significant, unexplained system performance decline

#### Unusual error messages

#### Significant, unexpected system memory loss

#### Periodic, unexpected rebooting

#### Display quality fluctuations

## Malware often discovered after damage done

## Minimal anti-malware functions

### Detect malware through signature scanning

#### Comparing file’s content with known malware signatures

### Detect malware through integrity checking

#### Comparing current file characteristics against archived version

### Detect malware by monitoring unexpected file changes

### Receive regular updates and modifications

### Consistently report only valid instances of malware--not false positives

#### Heuristic scanning: identifying malware by discovering “malware-like” behavior, is prone to false positives

# Anti-Malware Policies

## Dependent upon environment’s needs

## Decide whether to install anti-malware software on

### Every desktop

### Servers

## Policies provide rules for:

### Using anti-malware software

### Installing programs, sharing files, using external disks

## Management should authorize and support policy

## Anti-malware policy guidelines

### Protect network from damage, downtime

# Common Anti-Malware Software

## Free products

### Microsoft Security Essentials (probably the most logical choice for home users) (link Ch 14d)

### Avira

### Clamwin (no realtime protection, link Ch 14e)

### AVG (link Ch 14f)

## Commercial Products (for companies):

### McAfee, Norton Antivirus, F-Secure, eTrust

### Students get a free copy of McAfee Enterprise to use at home

# Hoaxes

## False alert email about:

### Dangerous, new virus

### Other malware causing workstation damage

## Ignore

### No realistic basis

### Attempt to create panic

### Do not pass on

## Verification

### Use reliable Web page listing virus hoaxes

## Watch for attached files

Fault Tolerance

# Fault Tolerance

## Capacity for system to continue performing

### Despite unexpected hardware, software malfunction

## Failure

### Deviation from specified system performance level

#### Given time period

## Fault

### Malfunction of one system component

### Can result in failure

## Fault-tolerant system goal

### Prevent faults from progressing to failures

# Levels of Fault Tolerance

## How critical are the network services to the company?

## Highest level of fault tolerance

### System remains unaffected by most drastic problem

### Power failure, flood, fire, etc.

## Lower level of fault tolerance

### System remains unaffected by more common problem

### Failure of a NIC or hard drive

# Environment

## Protect devices from:

### Excessive heat, moisture

#### Purchase temperature, humidity monitors

### Break-ins

### Natural disasters

# Power

## Blackout

### Complete power loss

## Brownout

### Temporary dimming of lights

## Causes

### Forces of nature

### Utility company maintenance, construction

## Solution

### Alternate power sources

# Power Flaws

## Not tolerated by networks

## Types:

### Surge

#### Momentary increase in voltage

### Noise

#### Fluctuation in voltage levels

### Brownout

#### Momentary voltage decrease

### Blackout

#### Complete power loss

# UPSs (Uninterruptible Power Supplies)

## Battery-operated power source

## Directly attached to one or more devices

## Attached to a power supply

## Prevents

### Harm to device, service interruption

## Variances

### Power aberrations rectified

### Time providing power

### Number of supported devices

### Price

# Standby UPS (Offline UPS)

### Switches quickly to battery upon power loss

### Problems

#### Time to detect power loss

#### Does not provide continuous power

# Online UPS

## Devices are always powered by the battery

## A/C power continuously charges battery

## No momentary service loss risk

## Handles noise, surges, sags

### Before power reaches attached device

## More expensive than standby UPSs

## Cost depends on power capacity

# Generators

## Powered by diesel, liquid propane, gas, natural gas, or steam

## Do not provide surge protection

## Provide electricity free from noise

## Used in highly available environments

## Generator choice

### Calculate organization’s crucial electrical demands

#### Determine generator’s optimal size

# Topology and Connectivity

## Before designing data links

### Assess network’s needs

## Fault tolerance in network design

### Supply multiple paths for data

#### Avoid single points of failure

### LAN: star topology and parallel backbone

### WAN: full-mesh or partial-mesh topology

### SONET technology

#### Relies on dual, fiber-optic ring

# Redundant Internet Connections

## Supply duplicate connection

### Use different service carriers

### Use two different routes

#### Critical data transactions must follow more than one possible path

## Network redundancy advantages

### Reduces network fault risk

#### Lost functionality

#### Lost profits

# Fault Tolerance at the WAN

## Consider PayNTime, a company that needs to get data from two clients to print checks

## One solution: lease two T1s to each client

### Expensive

## Better solution

### Partner with ISP

### Establishing secure VPNs

### Outsources the network redundancy and design

# Failures in the Data Room

## Many single points of failure

### T1 connection could incur fault

### Firewall, router, CSU/DSU, multiplexer, or switch might suffer faults in power supplies, NICs, or circuit boards

# Fault Tolerance in the Data Room

## Solution

### Redundant devices with automatic failover

#### Immediately assume identical component duties

### Use hot swappable devices

### Desired for switches or routers supporting critical links

### Adds to device cost

### Does not address all faults occurring on connection

#### Faults might affect connecting links

# Load Balancing

## Uses all redundant paths to move data faster

## That way the fault tolerance is not just wasted money when nothing fails

# Servers

## Critical servers

### Contain redundant components

#### Provide fault tolerance, load balancing

# Server Mirroring

## Mirroring

### Fault-tolerance technique

### One device, component duplicates another's activities

## Server mirroring

### One server continually duplicates another's transactions, data storage

### Uses identical servers, components

### High-speed link between servers

### Synchronization software

### Form of replication

#### Dynamic copying of data from one location to another

## Advantage

### Flexibility in server location

## Disadvantages

### Time delay for mirrored server to assume functionality

### Toll on network as data copied between sites

## Hardware and software costs

### May be justifiable

# Clustering

## Links multiple servers together

### Act as single server

## Clustered servers share processing duties

### Appear as single server to users

## Failure of one server

### Others take over

## For large networks

### More cost-effective than mirroring

## Many advantages over mirroring

### Each clustered server

#### Performs data processing

#### Always ready to take over

### Reduces ownership costs

### Improves performance

# Google Server Cluster

## Ch 14g

# Storage

## Data storage: also has issues of availability and fault tolerance

### Different methods are available for making sure shared data and applications are never lost or irretrievable

# RAID (Redundant Array of Independent [or Inexpensive] Disks)

## Collection of disks

### Provide shared data, application fault tolerance

## Disk array (drive)

### Group of hard disks

## RAID drive (RAID array)

### Collection of disks working in a RAID configuration

### Single logical drive

## Hardware RAID

### Set of disks, separate disk controller

### RAID array managed exclusively by RAID disk controller

#### Attached to server through server’s controller interface

## Software RAID

### Software implements, controls RAID techniques

#### Any hard disk type

### Less expensive (no controller, disk array)

### Performance rivals hardware RAID

# RAID Level 0 - Disk Striping

### Simple RAID implementation

### Data written in 64-KB blocks equally across all disks

### Not fault-tolerant

### Does not provide true redundancy

### Best RAID performance (in this chapter)

#### Uses multiple disk controllers

# RAID Level 1- Disk Mirroring

### Disk mirroring provides redundancy

#### Data from one disk copied automatically to another disk

### Advantages

#### Simplicity, automatic and complete data redundancy

### Disadvantages

#### Cost of two disks

#### CPU usage because software does the mirroring

## Disk duplexing

### Similar to disk mirroring

### Data continually copied from one disk to another

### Separate disk controller used for each disk

#### Provides added fault tolerance

# RAID Level 3 - Disk Striping with Parity ECC

### ECC (error correction code)

#### Algorithm to detect, correct errors

#### Known as parity error correction code

### Parity

#### Mechanism to verify data integrity

#### Number of bits in byte sum to odd, even number

#### Use either even parity, odd parity, not both

### Parity tracks data integrity

#### Not data type, protocol, transmission method, file size

### Parity error checking

#### Process of comparing data parity

### Advantage

#### High data transfer rate

### Disadvantage

#### Parity information appears on single disk

# RAID Level 5 - Disk Striping with Distributed Parity

### Most popular data storage technique

### Data written in small blocks across several disks

### Parity error checking information distributed among disks

### Advantages over RAID level 3

#### Writes data more rapidly

#### Uses several disks for parity information

#### Disk replacement causes little interruption

#### Controlling software regenerates failed file parts

### Hot spare (failover component)

#### Array disk, partition used only when one RAID disk fails

#### See link Ch 14i

### Cold spare (hot swappable component)

#### Duplicate component

#### Not installed

# NAS (Network Attached Storage)

## Specialized storage device, storage device group

### Provides centralized fault-tolerant data storage

## Difference from RAID

### Maintains own interface to LAN

## Advantages

### NAS device contains own file system

#### Optimized for saving, serving files

#### Reads, writes fast

### Easily expandable

#### No service interruption

## Disadvantage

### No direct communication with network clients

## Use

### Enterprises requiring fault tolerance, fast data access

# SANs (Storage Area Networks)

## Distinct networks of storage devices

## Communicate directly

### With each other, other networks

## Multiple storage devices

### Connected to multiple, identical servers

## Advantages

### Fault tolerant

### Fast

#### Special transmission method

#### Special protocols, like Fibre Channel

#### Despite the name, Fibre Channel can run over both copper and fiber media (link Ch 14k, 14l)

### SAN can be installed in location separate from LAN served

#### Provides added fault tolerance

### Highly scalable

### Faster, more efficient method of writing data

## Drawbacks

### High cost

#### Small SAN: $100,000

#### Large SAN: several million dollars

### More complex than NAS, RAID

#### Training, administration efforts required

## Use

### Environments with huge data quantities requiring quick availability

Data Backup

# Data Backup

## Backup

### Copies of data or program files

#### Created for archiving, safekeeping

### Store off site

## Without backup

### You risk losing everything

## Many backup options available

### Performed by different software and hardware

### Use different storage media types

## Can be controlled by NOS utilities, third-party software

# Backup Media and Methods

## Selecting backup media, methods

### Several approaches

#### Each has advantages and disadvantages

### Ask questions to select appropriate solution

# Optical Media

## Media storing digitized data

## Uses laser to write data, read data

## Examples

### CDs, DVDs

## Backup requirements

### Recordable CD or DVD drive, software utility

## CD-R (compact disc-recordable)

### Written to once, stores 650 MB data

## CD-RW (compact disc-rewriteable)

### Used more than once, stores 650 MB data

## CD backups

### Simple to restore from

#### Standard format

### Relatively low storage capacity

## Recordable DVD

### 4.7 GB on one single-layered side

### Double-layered, two-sided DVD

#### Store up to 17 GB of data

### Several different formats

## Disadvantage

### Writing data takes longer than other media

### Requires more human intervention

# Tape Backups

## Copying data to magnetic tape

## Relatively simple

## Stores very large data amounts

## Requirements

### Tape drive connected to network

### Management software

### Backup media

## Small network

### Stand-alone tape drives attached to each server

## Large network

### One large, centralized tape backup device

#### Manages all subsystems’ backups

## Extremely large environments

### Robots retrieve, circulate tapes from vault

#### Tape storage library

# External Disk Drives

## Removable disk drives

## Attached temporarily to computer

### USB, PCMCIA, FireWire, CompactFlash port

## Simple to use

### Save, share data

## Temporary drive appears like any other drive

## Large data capacity

### Backup control features, higher storage capacity, faster read-write access

#### Example: Iomega REV drive

# Network Backups

## Save data to another place on network

### Different server, another WAN location

### SAN, NAS storage device

## Online backup

### Saves data across Internet

#### To another company’s storage array

### Implement strict security measures

### Automated backup, restoration processes

### Online back up provider evaluation

#### Test speed, accuracy, security, recovery

# Online Backup Examples

## Iron Mountain

### Normal online backup on their servers

### Link Ch 14m

## Symform

### A cooperative service--your data is stored on other members' servers

### $5 a month, no data storage limit

### Ch 14n

# Backup Strategy

## Goal

### Perform reliable backups providing maximum data protection

## Documented in common area

### Accessible by all IT staff

## Address various questions

## Archive bit

### File attribute

#### Checked to set on or off

#### On indicates file must be archived

# Backup Methods

## Full backup

### All data copied

### Uncheck archive bits

### Uses the most tape and time

### Easiest restoration (play one tape)

## Incremental backup

### Copy data changed since last full or incremental backup

### Uncheck archive bits

### Uses the least tape and time

### Most complex recovery--must play many tapes

## Differential backup

### Copy only data changed since last backup

### All data marked for subsequent backup

### Does not uncheck archive bits

### Uses less tape and time than a Full Backup, but more than an Incremental Backup

### Data recovery requires two tapes

## Determine best backup rotation scheme

### Plan specifying when and how often backups occur

### Goal

#### Provide excellent data reliability without overtaxing network, requiring intervention

## Grandfather-Father-Son

### Uses backup sets

#### Daily (son)

#### Weekly (father)

#### Monthly (grandfather)

### Three backup types performed each month:

#### Daily incremental (every Monday through Thursday)

#### Weekly full (every Friday)

#### Monthly full (last day of the month)

## Ensure backup activity recorded in backup log

### Backup date

### Tape identification

### Type of data backed up,

### Type of backup

### Files backed up

### Site where tape stored

## Establish regular verification schedule

Disaster Recovery

# Disaster Recovery

## Disaster recovery

### Restoring critical functionality, data

#### After enterprise-wide outage

#### Affecting more than single system, limited group

## Consider possible extremes

### Hurricane, fire, etc.

### Not relatively minor outages, failures, security breaches, data corruption

# Disaster Recovery Planning

## Accounts for worst-case scenarios

## Identifies disaster recovery team

## Provides contingency plans

### Restore and replace:

#### Computer systems

#### Power

#### Telephony systems

#### Paper-based files

## Contains various sections

### Related to computer systems

## Lessens critical data loss risk

# Disaster Recovery Contingencies

## Cold site

### Components necessary to rebuild network exist

#### Not appropriately configured, updated, or connected

## Warm site

### Components necessary to rebuild network exist

#### Some appropriately configured, updated, and connected

## Hot site

### Components necessary to rebuild network exist

#### All are appropriately configured, updated, and connected

#### Match network’s current state

Link Ch 14o

Last modified 11-23-09