Application

Presentation

Session

Transport

Network

Data Link

Physical

Objectives

## What is the purpose of a Network model?

## What are the layers of the OSI model?

## What are the characteristics of each layer of the OSI model?

## How does the TCP/IP stack compare to the OSI model?

## What are the well-known TCP and/or UDP port numbers for a given collection of common applications

# The Purpose of Reference Models

## It breaks network communication into smaller, simpler parts that are easier to develop.

## It facilitates standardization of network components to allow multiple-vendor development and support.

## It allows different types of network hardware and software to communicate with each other.

## It prevents changes in one layer from affecting the other layers so that they can develop more quickly.

## It breaks network communication into smaller parts to make learning it easier to understand.

# Warriors of the Net video

\Quick Summary of Layers 1-4

# OSI Layers in Wireshark

1. Physical Layer

# Wiring Standards

# Asynchronous and Synchronous Communications

## Synchronizing Bits

### Two devices must agree on when one bit stops and another bit starts

## Asynchronous

### Uses start and stop bits

## Synchronous

### Internal clocks are synchronized at each end of the cable

# Bandwidth Usage

## Broadband

### Multiple channels share the same medium

### Ex: cable TV uses frequency division multiplexing (each channel uses a different frequency range)

## Baseband

### The whole medium is used for one transmission

### Example: Ethernet

# Time-Division Multiplexing (TDM)

## Each channel gets the same amount of time on the wire

# Statistical Time-Division Multiplexing (StatTDM)

## Busy channels get more time on the wire

# Frequency Division Multiplexing (FDM)

## Example: signals sent with different colors through the same fiber optic cable

# Layer 1 Devices

## Cables

## Wireless access points

## Hubs

### Because they don’t pay any attention to addresses, they just deliver signals to every connected device like a crossover cable

2. Data Link Layer

# MAC Addresses

## IPCONFIG /ALL

## Physical Address

## Built into the network interface

# Connection Services

## Flow control

### Prevents sender from sending data faster than the client can accept it

## Error control

### When a frame is received, a checksum is used to detect errors

#### Usually a Cyclic Redundancy Check (CRC)

### If the receiver's checksum does not match the sender's checksum, the frame is discarded and resent

# Layer 2 Devices

## Switches

## Bridges

## Network Interface Cards (NICs)

3. Network Layer

# IP Address

## Logical address

## Changes when the device is moved

# Switching

## Packet switching

### Data is broken into packets

### Many packets travel along network connections like cars on a freeway

## Circuit switching

### A physical line is dedicated to each connection

### Ex: old copper landline phone systems

## Message switching

### Store-and-forward, like email

# Layer 3 Devices

## Routers

## Multilayer Switches

4. Transport Layer

# TCP and UDP

## Transmission Control Protocol (TCP)

### Connection-oriented and reliable

### Handshake makes sure both ends are ready

### Segments are acknowledged and resent if necessary

## User Datagram Protocol (UDP)

### Connectionless and unreliable

### No handshake

### Best-effort delivery, no acknowledgements

# ICMP (Internet Control Message Protocol)

## At layer 4

## Used by ping and traceroute, and to indicate errors such as dropped packets

5. Session Layer

# Example of a Session

## User logs in with a username & password

## All data now has a special significance until that user logs off, or the session times out, or is terminated some other way

## Layer 6 Protocol

### H.323 (voice or video)

### NetBIOS (file sharing)

6. Presentation Layer

7. Application Layer

# Application Layer

## Closest to the user

## Hands data to an application in the format it expects, with no addresses or other transmission artifacts

## Examples: a downloaded file, an email message

The TCP/IP Stack

# The TCP/IP and OSI Models Compared

# IP Ver4 Header

# TTL (Time-to-Live)

## TTL decreases by one each time the packet is forwarded by a router

## If TTL reaches zero, the packet is discarded

## This eliminates packets trapped in routing loops

# Demo: Routing Loop

# TCP Header

# TCP Header Fields

## Port numbers

### Indicate which program on the end device should receive the data

### Examples: Port 25 for email, 80 for HTTP

## Window size

### Number of bytes that can be sent before waiting for an ACK

# TCP Header Fields

## Sequence and Acknowledgement numbers

### Used to put packets in order to reassemble files and other large messages

## Flags like SYN and ACK are used for the TCP handshake and to acknowledge data received

# UDP Header

## No handshake, acknowledgements, sequencing, or flow control

# Common Port Numbers

| DNS (Domain Name System) | TCP/UDP 53 |
| --- | --- |
| HTTP (Hypertext Transfer Protocol) | TCP 80 |
| SMTP (Simple Mail Transfer Protocol) | TCP 25 |
| POP (Post Office Protocol) | TCP 110 |
| Telnet (DHCP) | TCP 23 |
| DHCP (Dynamic Host Configuration Protocol) | UDP 67(IPv4 client) and 68(IPv4 server); |
| FTP (File Transfer Protocol) | TCP 20(data) and 21(control) |
| TFTP (Trivial File Transfer Protocol) | UDP 69 |
| NBNS (NetBIOS Name Service) | UDP/TCP 137 |
| IMAP4 (Internet Message Access Protocol) | TCP 143 |
| SNMP (Simple Network Management Protocol) | TCP/UDP 161 |
| HTTPS (Hypertext Transfer Protocol Secure) | TCP 443 |
| NTP (Network Time Protocol) | UDP 123 |
| SSL (Secure Sockets Layer) | TCP 443 |
| SSH (Secure Shell) | TCP 22 |

# Link Ch 2d for flash cards

# Port Types

## Port numbers are assigned in various ways, based on three ranges:

### System Ports (0-1023), System Ports are assigned by IETF process for standards-track protocols, as per RFC6335.  Also known as well-known-ports

### User Ports (1024-49151) ,User Ports are assigned by IANA using the "Expert Review" process, as per RFC6335

### Dynamic and/or Private Ports (49152-65535), Dynamic Ports are not assigned, they are dynamically created as your computer need them.  Also known as ephemeral ports.

# Communication Between Two Systems

### Last modified 4-21-13