# TJX Data Breach

## TJX used WEP security

## They lost 45 million customer records

## They settled the lawsuits for $40.9 million

### Link Ch 6a

# Objectives

## Describe the basic IEEE 802.11 wireless security protections

## Define the vulnerabilities of open system authentication, WEP, and device authentication

## Describe the WPA and WPA2 personal security models

## Explain how enterprises can implement wireless security

IEEE 802.11 Wireless Security Protections

# Institute of Electrical and Electronics Engineers (IEEE)

## In the early 1980s, the IEEE began work on developing computer network architecture standards

### This work was called Project 802

## In 1990, the IEEE formed a committee to develop a standard for WLANs (Wireless Local Area Networks)

### At that time WLANs operated at a speed of 1 to 2 million bits per second (Mbps)

# IEEE 802.11 WLAN Standard

## In 1997, the IEEE approved the IEEE 802.11 WLAN standard

## Revisions

### IEEE 802.11a

### IEEE 802.11b

### IEEE 802.11g

### IEEE 802.11n

# Controlling Access to a WLAN

## Access is controlled by limiting a device’s access to the access point (AP)

## Only devices that are authorized can connect to the AP

### One way: Media Access Control (MAC) address filtering

### CCSF uses this technique (unfortunately)

### See www.ccsf.edu/wifi

# MAC Address Filtering

# MAC Address Filtering

## Usually implemented by permitting instead of preventing

## CCSF does this

### www.ccsf.edu/wifi

# Wired Equivalent Privacy (WEP)

## Designed to ensure that only authorized parties can view transmitted wireless information

## Uses encryption to protect traffic

## WEP was designed to be:

### Efficient and reasonably strong

# WEP Keys

## WEP secret keys can be 64 or 128 bits long

## The AP and devices can hold up to four shared secret keys

### One of which must be designated as the default key

# WEP Encryption Process

# Transmitting with WEP

# Device Authentication

## Before a computer can connect to a WLAN, it must be authenticated

## Types of authentication in 802.11

### Open system authentication

#### Lets everyone in

### Shared key authentication

#### Only lets computers in if they know the shared key

Vulnerabilities of IEEE 802.11 Security

## Open system authentication

## MAC address filtering

## WEP

# Open System Authentication

## To connect, a computer needs the SSID (network name)

## Routers normally send out beacon frames announcing the SSID

## Passive scanning

### A wireless device listens for a beacon frame

# Turning Off Beaconing

## For "security" some people turn off beacons

### This annoys your legitimate users, who must now type in the SSID to connect

### It doesn't stop intruders, because the SSID is sent out in management frames anyway

### It can also affect roaming

### Windows XP prefers networks that broadcast

# MAC Address Filtering Weaknesses

## MAC addresses are transmitted in the clear

### An attacker can just sniff for MACs

## Managing a large number of MAC addresses is difficult

## MAC address filtering does not provide a means to temporarily allow a guest user to access the network

### Other than manually entering the user’s MAC address into the access point

# WEP

## To encrypt packets WEP can use only a 64-bit or 128-bit number

### Which is made up of a 24-bit initialization vector (IV) and a 40-bit or 104-bit default key

## The 24-bit IV is too short, and repeats before long

## In addition, packets can be replayed to force the access point to pump out IVs

# Cracking WEP

## With the right equipment, WEP can be cracked in just a few minutes

### You need a special wireless card

### We do it in CNIT 123: Ethical Hacking and Network Defense

Personal Wireless Security

### WPA Personal Security

### WPA2 Personal Security

# WPA Personal Security

## Wireless Ethernet Compatibility Alliance (WECA)

### A consortium of wireless equipment manufacturers and software providers

## WECA goals:

### To encourage wireless manufacturers to use the IEEE 802.11 technologies

### To promote and market these technologies

### To test and certify that wireless products adhere to the IEEE 802.11 standards to ensure product interoperability

## In 2002, the WECA organization changed its name to Wi-Fi (Wireless Fidelity) Alliance

## In October 2003 the Wi-Fi Alliance introduced Wi-Fi Protected Access (WPA)

### WPA had the design goal to protect both present and future wireless devices, addresses both wireless authentication and encryption

## PSK addresses authentication and TKIP addresses encryption

## Preshared key (PSK) authentication

### Uses a passphrase to generate the encryption key

## Key must be entered into both the access point and all wireless devices

### Prior to the devices communicating with the AP

## The PSK is not used for encryption

### Instead, it serves as the starting point (seed) for mathematically generating the encryption keys

# Temporal Key Integrity Protocol (TKIP)

## WPA replaces WEP with TKIP

## TKIP advantages:

### TKIP uses a longer 128-bit key

### TKIP uses a new key for each packet

# Message Integrity Check (MIC)

## WPA also replaces the (CRC) function in WEP with the Message Integrity Check (MIC)

### Designed to prevent an attacker from capturing, altering, and resending data packets

### See link Ch 6b

# WPA2 Personal Security

## Wi-Fi Protected Access 2 (WPA2)

### Introduced by the Wi-Fi Alliance in September 2004

### The second generation of WPA security

### Still uses PSK (Pre-Shared Key) authentication

### But instead of TKIP encryption it uses a stronger data encryption method called AES-CCMP

## PSK Authentication

### Intended for personal and small office home office users who do not have advanced server capabilities

### PSK keys are automatically changed and authenticated between devices after a specified period of time known as the rekey interval

# PSK Key Management Weaknesses

## People may send the key by e-mail or another insecure method

## Changing the PSK key is difficult

### Must type new key on every wireless device and on all access points

### In order to allow a guest user to have access to a PSK WLAN, the key must be given to that guest

# Pre-Shared Key Weakness

## A PSK is a 64-bit hexadecimal number

### Usually generated from a passphrase

#### Consisting of letters, digits, punctuation, etc. that is between 8 and 63 characters in length

## If the passphrase is a common word, it can be found with a dictionary attack

# Cracking WPA

# WPA2 Personal Security (continued)

## AES-CCMP Encryption

### Encryption under the WPA2 personal security model is accomplished by AES-CCMP

### This encryption is so complex that it requires special hardware to be added to the access points to perform it

# WPA and WPA2 Compared

Enterprise Wireless Security

## Two models:

## IEEE 802.11i

## WPA and WPA2 models

# IEEE 802.11i

## Improves encryption and authentication

## Encryption

### Replaces WEP’s original PRNG RC4 algorithm

### With a stronger cipher that performs three steps on every block (128 bits) of plaintext

# IEEE 802.11i

## IEEE 802.11i authentication and key management is accomplished by the IEEE 802.1x standard

# 802.1x Authentication

# IEEE 802.11i (continued)

## Key-caching

### Remembers a client, so if a user roams away from a wireless access point and later returns, she does not need to re-enter her credentials

## Pre-authentication

### Allows a device to become authenticated to an AP before moving into range of the AP

### Authentication packet is sent ahead

# WPA Enterprise Security

## Designed for medium to large-size organizations

## Improved authentication and encryption

## The authentication used is IEEE 802.1x and the encryption is TKIP

# WPA Enterprise Security (continued)

## IEEE 802.1x Authentication

### Provides an authentication framework for all IEEE 802-based LANs

### Does not perform any encryption

## TKIP Encryption

### An improvement on WEP encryption

### Designed to fit into the existing WEP procedure

# WPA2 Enterprise Security

## The most secure method

## Authentication uses IEEE 802.1x

## Encryption is AES-CCMP

# Enterprise  Wireless Security Models

# Enterprise Wireless Security Devices

## Thin Access Point

### An access point without the authentication and encryption functions

#### These features reside on the wireless switch

## Advantages

### The APs can be managed from one central location

### All authentication is performed in the wireless switch

# Wireless VLANs

### Can segment traffic and increase security

### The flexibility of a wireless VLAN depends on which device separates the packets and directs them to different networks

# Enterprise Wireless Security Devices (continued)

## For enhanced security, set up two wireless VLANs

### One for employee access

### One for guest access

# Rogue Access Point Discovery Tools

## Wireless protocol analyzer

### Auditors carry it around sniffing for rogue access points

## For more security, set up wireless probes to monitor the RF frequency

# Types of Wireless Probes

## Wireless device probe

## Desktop probe

## Access point probe

## Dedicated probe

### Last modified 2-27-09