# Objectives

## Define environmental controls

## Describe the components of redundancy planning

## List disaster recovery procedures

## Describe incident response procedures

Environmental Controls

# Environmental Controls

## Fire suppression

## Electromagnetic shielding

## Configuring HVAC (Heating, Ventilating, and Air Conditioning) systems

# Fire Suppression

## Damage inflicted as a result of a fire continues to remain high

## Fire suppression is an important concern for the safety of employees and business continuity

## In order for a fire to occur, four entities must be present at the same time:

### A type of fuel or combustible material

### Sufficient oxygen to sustain the combustion

### Enough heat to raise the material to its ignition temperature

### A chemical reaction that is the fire itself

# Fire Triangle

# Fire Suppression

## In a server closet or room that contains computer equipment

### Using a handheld fire extinguisher is not recommended because the chemical contents can contaminate electrical equipment

## Instead, stationary fire suppression systems should be:

### Integrated into the building’s infrastructure

### Release the suppressant in the room

# Electromagnetic Shielding

## Computer systems, cathode ray tube monitors, printers, and similar devices all emit electromagnetic fields

### That are produced by signals or the movement of data

## Van Eck phreaking

### A form of eavesdropping in which special equipment is used to pick up telecommunication signals or data within a computer device

#### By monitoring the electromagnetic fields

## Faraday cage

### A metallic enclosure that prevents the entry or escape of an electromagnetic field

### Consists of a grounded fine-mesh copper screening

## Telecommunications Electronics Material Protected from Emanating Spurious Transmissions (TEMPEST)

### A secret US government standard

### Intended to “reduce the conducted and radiated emissions from within the sensitive environment to an undetectable level outside the shielded enclosure in uncontrolled areas”

# Compromising Electromagnetic Emanations of Keyboards

## Link Ch 13e

# HVAC

## Heating, ventilation, and air conditioning (HVAC) systems

### Provide and regulate heating and cooling for data centers

## Controlling environmental factors can reduce electrostatic discharge (ESD)

### The sudden flow of electric current between two objects, which can destroy electronic equipment

## Don't put electronics above ceiling tiles

### Smoke in the plenum can spread rapidly

Redundancy Planning

## Building excess capacity in order to protect against failures

## Redundancy for

### Servers

### Storage

### Networks

### Power

### Sites

# Servers

## A crash of a single server that supports a critical application can have a significant impact

## Single point of failure

## Redundant Servers

### Can replace failed server, but takes time to deploy

# Server Clusters

## Server cluster

### The combination of two or more servers that are interconnected to appear as one

## There are two types of server clusters:

### Asymmetric server cluster

#### A standby server exists only to take over for another server in the event of its failure

#### Failover cluster

### Symmetric server cluster

#### Every server in the cluster performs useful work

#### Load-balancing cluster

# Google's Server Cluster

## Over 450,000 servers

### Links Ch 13g, 13h

# Storage

## Mean time between failures (MTBF)

### Average time until a component fails

## Fault tolerance

### The ability to endure failures

### Prevents a single problem from escalating into a major failure

### Can often be achieved by maintaining redundancy

# RAID (Redundant Array of Independent Drives)

### Uses multiple hard disk drives for increased reliability and performance

## The most common levels of RAID are:

### RAID Level 0 (striped disk array without fault tolerance)

### RAID Level 1 (mirroring)

### RAID Level 5 (independent disks with distributed parity)

### RAID 0+1 (high data transfer)

# Networks

## Redundant network

### “Waits” in the background during normal operations

### Uses a replication scheme to keep its copy of the live network information current

## Virtually all network components can be duplicated to provide a redundant network

## In addition, some organizations contract with more than one Internet Service Provider (ISP) for remote connectivity

# Power

## Uninterruptible power supply  (UPS)

### Maintains power to equipment if the line power fails

## Off-line UPS

### Switches to battery power rapidly when it detects a power outage

## On-line UPS

### Always uses battery power, charges when the line power is available

# UPS Tasks

## When the power fails, A UPS can:

### Send a message to the network administrator’s computer, or page or telephone the network manager

### Notify all users that they must finish their work immediately and log off

### Prevent any new users from logging on

### Disconnect users and shut down the server

## Because a UPS can only supply power for a limited amount of time, some organizations turn to using a backup generator to create power

# Sites

## Redundancy can also be planned for the entire site itself

## Hot site

### Generally run by a commercial disaster recovery service

### A whole duplicated office ready to go (link Ch 13j)

## Cold site

### Provides office space but the customer must provide and install all the equipment needed to continue operations

## Warm site

### Has all of the equipment installed but does not have active Internet or telecommunications facilities, and does not have current backups of data

Disaster Recovery Procedures

# AT&T's Network Disaster Recovery Team

## Trucks and vans with enough equipment and personnel to set up a main telephone central office in a vacant lot in a few hours

### Link Ch 13i

## Disaster recovery

### Procedures and processes for restoring an organization’s IT operations following a disaster

### Focuses on restoring computing and technology resources to their former state

## Disaster recovery procedures include

### Planning

### Disaster exercises

### Data backups

# Planning

## Disaster recovery plan (DRP)

### A written document that details the process for restoring IT resources

#### Following an event that causes a significant disruption in service

## Comprehensive in its scope, a DRP is intended to be a detailed document that is updated regularly

## All disaster recovery plans are different

# Sample DRP

## Most disaster recovery plans address the common features included in the following typical outline:

### Unit 1: Purpose and Scope

### Unit 2: Recovery Team

### Unit 3: Preparing for a Disaster

### Unit 4: Emergency Procedures

### Unit 5: Restoration Procedures

## It is important that a good DRP contains sufficient detail

# Sample DRP Excerpt

# Disaster Exercises

## Disaster exercises are designed to test the effectiveness of the DRP

## Objectives of these disaster exercises:

### Test the efficiency of interdepartmental planning and coordination in managing a disaster

### Test current procedures of the DRP

### Determine the strengths and weaknesses in responses

# Data Backups

### Information copied to a different medium and stored at an offsite location so that it can be used in the event of a disaster

## Five basic questions that should be answered:

### What information should be backed up?

### How often should it be backed up?

### What media should be used?

### Where should the backup be stored?

### What hardware or software should be used?

# Fault Tolerance v. Backup

## RAID is not a backup!

## RAIDs provide fault tolerance

### Data remains available even when a hard drive fails

## Backups provide disaster recovery

### Data can be restored even if the building burns down

## Backup software can internally designate which files have already been backed up

### By setting an archive bit in the properties of the file

## Backing up to magnetic tape has been the mainstay of data backups for over 30 years

## Grandfather-father-son backup system

### Divides backups into three sets: a daily backup (son), a weekly backup (father), and a monthly backup (grandfather)

# Types of Data Backups

## Recovery point objective (RPO)

### The maximum length of time that an organization can tolerate between backups

## Recovery time objective (RTO)

### The length of time it will take to recover the data that has been backed up

## An alternative to using magnetic tape is to back up to magnetic disk

### Such as a large hard drive or RAID configuration

### This is known as disk to disk (D2D)

## D2D offers better RPO than tape

### However, as with any hard drive, the D2D drive may be subject to failure or data corruption

## Disk to disk to tape (D2D2T)

### Combines the best of magnetic tape and magnetic disk

### Uses the magnetic disk as a temporary storage area

## Continuous data protection (CDP)

### Performs continuous data backups that can be restored immediately

# Web-based Backup

## Ch 13k

Incident Response Procedures

# Incident Response Procedures

## Incident response procedures include using forensic science and properly responding to a computer forensics event

# What Is Forensics?

## Forensics, also known as forensic science

### The application of science to questions that are of interest to the legal profession

## Computer forensics

### Can attempt to retrieve information—even if it has been altered or erased—that can be used in the pursuit of the attacker or criminal

### Also used to limit damage and loss of control of data

## The importance of computer forensics is due in part to the following:

### High amount of digital evidence

### Increased scrutiny by the legal profession

### Higher level of computer skill by criminals

# Responding to a Computer Forensics Incident

## Secure the crime scene

### The computer forensics response team should be contacted whenever digital evidence needs to be preserved and serve as first responders

### The physical surroundings of the computer should be clearly documented

### Photographs of the area should be taken before anything is touched

### The computer should be photographed from several angles

### Cables connected to the computer should be labeled

## Preserve the evidence

### Team should first capture any volatile data

### The team should next focus on the hard drive

#### Mirror image backups replicate all sectors of a computer hard drive, including all files and any hidden data storage areas

## Chain of custody

### Documents that the evidence was under strict control at all times and no unauthorized person was given the opportunity to corrupt the evidence

## Examine for evidence

### After a computer forensics expert creates a mirror image of a system:

#### Original system is secured

#### Mirror image is examined to reveal evidence

### Includes searching word processing documents, e-mail files, spreadsheets, and other documents for evidence

### Hidden clues can also be mined and exposed

#### Including the Windows page file

### Another source of hidden data is called slack

## Examine for evidence (continued)

### RAM slack

#### Can contain any information that has been created, viewed, modified, downloaded, or copied since the computer was last booted

### Drive file slack (sometimes called drive slack)

#### Contains remnants of previously deleted files or data from the format pattern associated with disk storage space that has yet to be used by the computer

### An additional source of hidden clues can be gleaned from metadata, or data about data

Last modified 4-30-09