Reviewing Basic Networking Concepts

# Basic Connectivity Protocols

## TCP (Transmission Control Protocol)

## UDP (User Datagram Protocol)

## IP (Internet Protocol)

## ARP (Address Resolution Protocol)

## ICMP (Internet Control Message Protocol)

# TCP

## Connection-oriented: guaranteed delivery

## Three-way handshake

### SYN

### SYN/ACK

### ACK

## SYN Flood Attack

### Consumes server resources, creating a Denial of Service (DoS)

# UDP

## Connectionless

## No handshake

## No guarantee of delivery

## Often used for DoS attacks

# IP

## Delivers packets to specified computer by IP Address

## IPv4: 32-bit address

### 192.168.1.1

## IPv6: 128-bit address

### fe80:0:0:0:462a:60ff:fef6:278a

# ARP

## Finds MAC address from IP address

## ARP Poisoning

### Sends false ARP messages

### Redirects traffic on a LAN

### Commonly used for Man-In-The-Middle Attacks

# ICMP

## Connectivity tests

### Ping

### Pathping

### Tracert

## Used in DoS attacks

## Blocked by default by Windows XP SP2 and later firewalls

# Encryption Protocols

## SSH (Secure Shell)

## SSL (Secure Sockets Layer)

## TLS (Transaction Layer Security)

## IPSec (Internet Protocol Security)

# SSH

## Used to encrypt Telnet

### Telnet lacks encryption and uses port TCP 23

## Secure Copy Protocol (SCP)

## Secure File Transfer Protocol (SFTP)

## Runs on port TCP 22

# SSL

## Can be used to encrypt HTTP traffic, as HTTPS

### Port TCP 443

## Can also secure LDAP as LDAPS

### Port TCP 636

## SSL is old and has security weaknesses

# TLS

## Replacement for SSL

## Runs on the same ports

### HTTPS on TCP 443

### LDAPS on TCP 636

# IPSec

## Native to IPv6 but back-ported to IPv4

## Encapsulates and encrypts IP packets

## Two components

### AH (Authentication Header)

#### Protocol ID 51 (neither TCP nor UDP)

### ESP (Encapsulating Security Payload)

#### Protocol ID 50

# Application Protocols

## HTTP (Hypertext Transfer Protocol)

## HTTPS (HTTP Secure)

## FTP (File Transfer Protocol)

## SFTP (Secure FTP)

## FTPS (FTP Secure)

## TFTP (Trivial File Transfer Protocol)

# Application Protocols

## Telnet

## SNMP (Simple Network Management Protocol)

## DNS (Domain Name System)

## NetBIOS (Network Basic Input/Output System)

## LDAP (Lightweight Directory Access Protocol)

# Application Protocols

## Kerberos

## SQL Server (Structured Query Language)

## RDP (Remote Desktop Protocol)

### Used by Terminal Services

### Also called Remote Desktop Service or Remote Administration

# HTTP

## Normal Web browser traffic

## Port TCP 80

## Not encrypted

# HTTPS

## Encrypts traffic

## Guarantees identity of server

## Displays padlock in Web browser and HTTPS at start of URL

## Uses SSL or TLS, port TCP 443

# FTP

## Upload or download files

## Data in cleartext, including passwords

## Active mode

### Ports TCP 20 for data and TCP 21 for control

## Passive mode

### Random port for data and TCP 21 for control

# SFTP and FTPS

## SFTP

### FTP over SSH

### Port TCP 22

## FTPS

### FTP over SSL or TLS

### Ports TCP 989 and 990

# TFTP

## Uses UDP port 69

## No authentication at all

## Used for IP phone and router firmware updates

## Many attacks used it

# Telnet

## Used to send command lines to remote systems

## Uses no encryption, not even for passwords

## Port TCP 23

# SNMP

## Used to monitor and manage network devices like routers, switches, and firewalls

## Sends traps – signals notifying management systems of their status

## Port UDP 161

## SNMPv1 and v2 sent "community strings" (passwords) in cleartext

## SNMPv3 encrypts passwords

# DNS

## Resolves host names like www.ccsf.edu into IP addresses like 147.144.1.212

## Ports UDP 53 and TCP 53

## Many security problems, which will be improved by switching to DNSSEC

# Dan Kaminsky

## World-famous DNS expert

## Found a serious flaw that enabled him to redirect Internet traffic

## Kept it secret till Microsoft and other vendors patched it

## Testified before Congress

### Link Ch 3a

# NetBIOS

## Used to resolve Windows computer names like SERVER1 to IP addresses on Local Area Networks

## A legacy protocol, replaced by DNS on most modern networks

## Still used by Windows

## Ports 137-139, both TCP and UDP

# LDAP

## Used for directories of users and objects on networks, including

### Microsoft Active Directory

### Novell Netware Directory Services

## Port TCP 389 (unencrypted)

## Port TCP 636 (LDAPS, encrypted)

# Kerberos

## Uses tickets for authentication

## Used in Windows domains and some Unix environments

## Port 88, both TCP and UDP

# SQL Server

## Manages databases

## Often has SSNs, email addresses, account numbers, and other PII (Personally Identifiable Information)

## Commonly hacked via SQL Injection

## Port TCP 1433 (Also UDP 1434)

# RDP

## Remotely control a Windows computer

## Service is called "Remote Administration", "Terminal Services", or "Remote Desktop"

## Port TCP 3389

## Also used by Remote Assistance

# Email Protocols

## SMTP (Simple Mail Transfer Protocol)

### Sends mail to other email servers

### Port TCP 25

## POP3 (Post Office Protocol v3)

### Moves incoming email to your local Inbox in clients like Outlook

### Port TCP 110

## IMAP4 (Internet Message Access Protocol v4)

### Moves incoming email to your local Inbox in clients like Outlook, or lets you view them on the server

### Port TCP 143

# Remote Access Protocols

## PPP (Point-to-Point Protocol)

## IPSec (Internet Protocol Security)

## PPTP (Point-to-Point Tunneling Protocol)

## L2TP (Layer 2 Tunneling Protocol)

## RADIUS (Remote Authentication Dial-in User Service)

## TACACS (Terminal Access Controller Access-Control System)

## TACACS+

# PPP

## Used to create dial-up connections to a server

## Commonly used by clients to connect to an ISP

# IPSec

## Can be used as a remote access tunneling protocol

## To encrypt traffic, forming secure connections over the Internet

## Uses IKE (Internet Key Exchange) over port UDP 500

# PPTP

## Old VPN (Virtual Private Network) protocol

## Included in Microsoft Windows

## Has serious security flaws

## Still commonly used

## Port TCP 1723

# L2TP

## Combines Microsoft's PPTP with Cisco's L2F

## Often combined with IPSec for encryption

## Port UDP 1701

# RADIUS

## Central authentication for remote access clients

## Encrypts passwords only

# TACACS / XTACACS

## Older network authentication protocols

## TACACS is generic

## XTACACS is Cisco proprietary

## Port UDP 49 for both TACACS and XTACACS

# TACACS+

## Used by Cisco VPN concentrators

## Encrypts entire authentication process

## Multiple challenge responses for Authentication, Authorization, and Accounting (AAA)

## Port TCP 49

# Subnetting

## See Binary Games in Projects (Extra Credit)

# Ports

## 0-1023: 	Well-known ports

## 1024-49151: 	Registered ports

### Registered by IANA for convenience

### Example: SQL Server on 1433

## 49152-65535: 	Dynamic and private ports

### "Ephemeral" ports for temporary use by any application

# Demo: Telnet to 147.144.1.2

# Firewalls

## Block ports by protocol and number

## For example, allowing TCP 80 but blocking UDP 69

# Port Scanners

## Find open , closed, or filtered ports

## Nmap

Understanding Basic Network Devices

# IP Address Types

## Unicast

### One sender, one receiver

### The most common type

## Broadcast

### One sender to all devices on a LAN

### IP 255.255.255.255 sends to all devices on a LAN

### 147.144.255.255 sends to all devices in the 147.144.0.0 network

# Hub

## Common on old 10 Mbps LANs

## Zero intelligence

## Whatever comes in on a port goes out all other ports

## Each user can sniff traffic intended for others

# Switch

## Replaces hubs in almost all LANs now

## Learns which devices are connected to each port

## Sends traffic only to the correct port, after learning where the devices are

### At first, it acts like a hub while learning

# Security Benefits of Switches

## Reduces the threat of sniffing attacks

### Because devices don't get other devices' traffic

## Can be defeated by flooding with random MAC addresses

### Switch runs out of RAM for switching table and acts like a hub instead

## Can also be defeated by ARP poisoning

# Security Benefits of Switches (continued)

## Port Security

### Only allow a device with the approved MAC address to connect to each port

## BUT: MAC addresses can be spoofed

# Physical Security of a Switch

## Put the switch in a locked wiring closet

## Prevents attacker from accessing:

### Console port used to manage the switch

### Monitor port used to sniff all traffic

# STP (Spanning Tree Protocol)

## If wires allow traffic to flow in loops, this can lead to a broadcast storm

## To prevent this, switches use

### STP (Spanning Tree Protocol) or

### RSTP (Rapid Spanning Tree Protocol)

## Blocks unneeded ports to prevent loops

## Included in all switches and on by default

# VLAN (Virtual Local Area Network)

## At CCSF, the CNIT Dept. computers are in several different rooms and buildings

### SCIE 37, CLOU 218, SCIE 214, etc.

## But they are all in the same subnet and see one another as on the same LAN

## Switches sort traffic by adding a VLAN Tag to each ethernet frame

# Router

## Connect network segments together

### For example, a LAN to the Internet

## Don't forward broadcasts

### Reduce "noise" traffic on segments

## Computers can act as routers

### But most networks use hardware routers

## Image from Cisco

# ACLs (Access Control Lists)

## Packet filtering

## Traffic that is not allowed is usually discarded

# Routers and Firewalls

## Routers can filter traffic in simple ways

### By protocol, port, or address

## Early firewalls filtered the same way

## Firewalls are much more advanced now

# Home Router

## You can also use a router or residential gateway, which typically adds network address translation (NAT) capabilities and security features

# Firewall

## Filters traffic, both inbound and outbound

## Host-based Firewall

### Protects a single host from intrusion

### Example: Windows Firewall

## Network-based Firewall

### Protects a whole network

## Image from Palo Alto Networks

# Firewall Rules

## For simple packet-filtering, they are similar to router access lists

# Web Application Firewall

## Specifically designed to stop SQL Injection and other Web App attacks

### Example: modsecurity for Apache

# Web Security Gateway and Appliances

## Unified security solution

## Content inspection—attachments

## Content inspection—spam

## Content inspection—streaming data

## URL filtering

## Certificate inspection

# Spam Filters

## Google’s Postini is very good too

# Firewall Logs and Log Analysis

## Firewalls log all blocked traffic, all allowed traffic, or both

## Splunk (Link Ch 3b)

## AlienVault OSSIM (Link Ch 3c)

# Load Balancers

## Distributes traffic to a cluster of servers

## High availability

Exploring the Network Perimeter

# DMZ (Demilitarized Zone)

## A semi-trusted zone between a private network and the Internet

## Provides defense in depth for internal network

## Image from Wikipedia

# Public and Private IPv4 Addresses

## Public IP addresses are used to send and receive Internet traffic

## They aren't free, but leased from Internet Service Providers

## Private addresses can't be used on the Internet, but are free for use on private networks

# RFC 1918 Private Addresses

## 10.0.0.1 – 10.255.255.254

## 172.16.0.1 – 172.21.255.254

## 192.168.0.1 – 192.168.255.254

# NAT 
(Network Address Translation)

## NAT allows many clients to share a single public IP address

### Cost savings

## Hides local IP addresses

### Provides some protection

### Users can't run unauthorized servers

## NAT breaks some network services

### IPSec and many others

# Proxy Server

## No direct connections from clients to Internet allowed

## Proxy fetches content from the Internet and clients look at cached content

## Makes Web pages load faster

## Can filter content and log visited sites

## Last modified 9-9-12