Understanding IDSs and IPSs

# IDS v. IPS

## IDS detects attacks but does not stop them

### Detective technical control

### Passive IDS merely logs attacks, and/or sends alerts

### Active IDS may send alerts and change environment

## IPS stops attacks in progress

### Preventive technical control

### Similar to Active IDS

# HIDS v. NIDS

## HIDS (Host-based IDS)

### Installed on a server or workstation

## NIDS (Network-based IDS)

### Installed on a network device, such as a router or switch

# Signatures v. Anomalies

## Signature-based monitoring recognizes known attack patterns

### Also called definition-based

## Anomaly-based monitoring detects abnormal behavior based on a baseline

### Also valled behavior-based or heuristics-based

# Active IDS v. IPS

## Both react to an attack in some manner

## IPS must be placed in line with the traffic to prevent the attack

# HIDS

## Additional software on a workstation or server

## Can detect attacks on the local system

## Can monitor changes to operating system files

## Protects only one host

# NIDS

## Can only see network traffic, not OS files

## Unable to decrypt encrypted traffic

# Detection Methods

## Signature-based

### Uses a database of predefined traffic patterns

### Database requires frequent updates

## Anomaly-based

### Needs to measure a performance baseline

### Baseline must be updated if network is changed

# Data Sources and Trends

## IDS collects data from various sources

### Firewall logs

### System logs

### Application logs

### May monitor logs in real time

# Alarms, Alerts, and Trends

## Alarms

### Also called Alerts

### Indicates that an interesting event was detected

### Does not always indicate a real attack

## Goal

### Set threshold low enough to detect all real attacks, but

### High enough to avoid too may false positives

# IDS Threshold

## Number of events required to cause an alert

### Example: 50 incomplete TCP handshakes per minute from the same IP

## There are no established rules for thresholds

### Must be "tuned" by administrators

## Untuned security devices tend to produce many false positives

# IDS Responses

## Passive (Alerts personnel)

### Pop-up window

### Central monitor

### E-mail

### Page or text message

## Active

### Alerts personnel

### Modify ACL on Firewall

### Divert attack to a honeypot or other safe environment

# Honeypot

## Appears to be a server worth hacking into

## Has no valuable data

## Often used to collect knowledge about attackers

# Zero-Day Vulnerability

## A vulnerability that is not publicly known

### Or a vulnerability for which there is no patch available

# Link Ch 4d

## Demo at link Ch 4e

# Honeynet

## A group of virtual servers appearing to be a live network

# Counterattacks

## Some active IDS systems attack the attacker back

## Legal problems

## Likely that you are attacking another innocent victim

Securing Wireless Networks

# Wireless Footprint

## High power makes a large footprint

### Easier for users to connect

### Easier for attackers to snoop on

## Careful antenna placement

## Metal in walls

## Directional antennas increase range of reception

# Security Protocols

## WEP – Broken and unsafe to use

## WPA – Much safer, stronger with AES than TKIP

## WPA2 – Best security currently available, especially in Enterprise mode with 802.1x or RADIUS server

# WEP (Wired Equivalent Privacy)

## Mathematically insecure

## Can be broken with no knowledge of the key 100% of the time

## Attacker needs 50,000 packets or so

# WPA (Wi-Fi Protected Access)

## Designed to run on hardware designed for WEP with only a software upgrade

## Weakest form of WPA uses TKIP (Temporal Key Integrity Protocol) and RC4 encryption

## Stronger form of WPA uses AES encryption

# WPA-2

## Stronger cryptography than WEP or WPA

## Uses CCMP mode of AES

# Personal and Enterprise Modes

## Both WPA and WPA-2 have Personal and Enterprise modes

## Personal

### Pre-Shared Key must be entered in each device

### Same for all users

# Personal and Enterprise Modes

## Enterprise mode

### Each user has individual credentials

#### Username and password

#### Extensible protocols like LEAP and PEAP which can use certificates

### Credentials stored on an 802.1x server

# Attacking WPS (Wi-Fi Protected Setup)

## Link Ch 4g

# EAP, LEAP, and PEAP

## EAP (Extensible Authentication Protocol)

### A framework that provides general guidance for authentication

## LEAP (Lightweight EAP)

### Cisco's attempt to improve WEP

### Uses CHAP

### Not secure, subject to offline dictionary attack

### Cisco recommends using PEAP instead

# ASLEAP

## Lightweight Extensible Authentication Protocol (LEAP)

### A Cisco product

### Vulnerable, but Cisco didn’t care

### Joshua Wright wrote the ASLEAP hacking tool to crack LEAP, and forced Cisco to develop a better protocol

#### See link Ch 4f

# Reaction to ASLEAP

## “Within months, some "helpful" person invested their time into generating a cracker tool. Publicizing the threat was a service to everyone, but I leave it as an exercise for readers to determine what satisfaction is obtained by the authors of tools that turn threat into reality and lay waste to millions of dollars of investments.”

## --"Real 802.11 Security", William Arbaugh and Jon Edney, as quoted in link Ch 4f

# PEAP (Protected EAP)

## Encapsulates EAP conversation in a TLS tunnel

## Requires a digital certificate for the server

## Requires a Certificate Authority

# WTLS and ECC

## WTLS (Wireless Transport Layer Security)

### Used by many smaller wireless devices

## ECC (Ellipt Curve Cryptography)

### A more efficient cryptography method than the RSA algorithm used on the Internet

### Used on small wireless devices to save power

# Other Security Concerns

## Change default administrator password

## MAC filtering

### Allows only approved MAC addresses to connect

### Easily sniffed & spoofed

# Disable SSID (Service Set Identifier) Broadcasts

## SSID is the name that identifies your network, such as "Linksys" or "CCSF Wifi"

## Only stops sloppy attackers who use NetStumbler

## SSID is in every packet sent anyway

# Change Default SSID

## Conceals the real model of your router

## Makes attackers work harder

# CCSF Wardriving

## Tue, nov. 13, 6 PM SCIE 200

# Wardriving Results

# WIGLE.NET

# Wardriving

## Part of a wireless audit

## Finds "Rogue" access points

### Unauthorized routers added to network

## Finds "Evil Twins"

### Access points spoofing the authorized ones to perform MITM attacks

# Interference

## Attackers can use radio transmissions to interfere with Wi-Fi

## Changing channel or power levels may help to resist the attack

## Disassociation Frames are the most effective DoS attack

# Isolation Mode

## Each client is on a separate VLAN, in effect

## Isolates clients better

## Protects clients from each other

## Done at Starbuck's

# Ad-Hoc Mode

## Infrastructure Mode

### The usual situation

### A wireless router connects all clients to the Internet

## Ad-Hoc Mode

### Wlreless clients connect to each other in a peer-to-peer manner

### Similar to a crossover cable in wired Ethernet

# Bluetooth Wireless

## Sh  ort-range connections

## In 'Discovery Mode", a Bluetooth device broadcasts its MAC address, to allow "pairing"

## PINs are often set to 0000

# Bluesnarfing and Bluejacking

## Bluesnarfing

### Access to or theft of data from a Bluetooth connection

## Bluejacking

### Unauthorized sending of text messages from a Bluetooth device

# Smartphone Security

## Data encryption

### Protects data if phone is lost

## Remote wipe

## Password protection

### Bluesnarfing attack bypasses password protection and data encryption

## Location systems via GPS

Exploring Remote Access

# Remote Access

## Allows users who are travelling to connect to internal network services

## Remote Access Service

### Through dial-up or VPN (Virtual Private Network)

# Components

## Access Method

### Dial-up or VPN

## Authentication

### Require secure authentication because users may use insecure lines

## Access Control

### User authorization

### Client health

# Dial-Up RAS

## Client and sever need phone lines and modems

## Requires POTS line (Plain Old Telephone Service)

## Requires a long-distance phone call

## Uses PPP (Point-to-Point Protocol)

# VPN

## Uses the Internet

## Faster and cheaper than Dial-up

## Uses tunneling to move LAN packets over the Internet

# VPN Concentrator

## Used at large companies

## Includes

### Strong encryption and authentication

### Handles many clients

# Tunneling Protocols

## IPSec (port UDP 500 for IKE)

### Problems with NAT

## PPTP (port TCP 1723)

### Old and vulnerable

## L2TP (port UDP 1701)

### No encryption, often used with IPSec

## SSTP (port TCP 443)

### Works through NAT

## For more details, see link Ch 4i

# IPsec as a Tunneling Protocol

## IPsec has two modes

### Transport Mode

#### Only the payload is encrypted

### Tunnel Mode

#### The entire IP packet is encrypted

## Uses IKE over UDP port 500 to authenticate clients

## Creates Security Associations (SAs) for the VPN

# AH and ESP

## IPsec can use

### AH (Authentication Header) alone

#### for Authentication and Integrity

#### no encryption

#### Protocol 51

### or ESP (Encapsulating Security Payload)

#### to encrypt the data

#### providing Confidentiality, Integrity, and Authentication

#### Protocol 50

## Link Ch 4j

# L2TP and IPsec

## L2TP creates a tunnel, but without encryption

### No confidentiality

## L2TP/IPsec used together provide

### Authentication

#### from Authentication Header

#### on Protocol 51

### Encryption

#### from Encapsulating Security Payload (ESP)

#### on Protocol 50

# NAT and IPsec

## NAT changes the IP addresses of packets

## IPsec detects this change as packet damage and discards it

## To use IPsec through NAT, use

### NAT-T (NAT Traversal) (link Ch 4k), or

### Another tunneling protocol such as SSTP

# Site-to-Site VPNs

## Easier for users; just appears as a normal network connection

# RAS Authentication

## CHAP (Challenge Handshake Authentication Protocol)

### Old, rarely used anymore

## MS-CHAPv2

### Much more secure than CHAP

### Mutual authentication

### Can be integrated with other authentication methods such as EAP, PEAP, and more

# RAS Authentication

## RADIUS (Remote Authentication Dial-In User Service)

### Central server for authentication

### Encrypts only passwords

## TACACS+

### Used by Cisco VPN concentrators

### Replaces RADIUS

### Encrypts entire client-server negotiation process

### Uses multiple challenges and responses

# Network Access Control

## Checks health of client before allowing it to connect to the company LAN

### Up-to-date patches and antivirus, firewall on

### Health agent runs on client

## Authentication

### NAC can deny access if clients don't provide valid credentials

## Image from Microsoft

# MAC Filtering

## A form of network access control

## Only approved MAC addresses can connect

## Easy for attackers to circumvent on a wireless network

### By sniffing and spoofing MAC addresses

## Last modified 9-17-12