# Overview

## Explain the components and essential operations of virtual private networks (VPNs)

## Describe the different types of VPNs

## Discuss VPN setups, such as mesh or hub-and-spoke configurations

## Select the right VPN tunneling protocol for a specific user need

## Define the process of setting up secure remote access for individual users via a VPN

## Discuss best practices for effective configuration and maintenance of VPNs

# Introduction

## Private leased lines

### Give the user dedicated use of a predefined bandwidth or data rate

### Often used to connect remote users or branch offices to a central administrative site

### Don’t scale well

## VPNs

### Function like private leased lines

### Provide a means of secure point-to-point communications over the public Internet

# VPN Components and Operations

## Many telecommunications companies provide VPN services

## Can be set up with special hardware or with firewall software that includes VPN functionality

### Many firewalls have VPN systems built into them

## Goal of a VPN

### Provide a cost-effective and secure way to connect business locations to one another and connect remote workers to office networks

# VPN Components

## Endpoints

### Hardware and/or software components that perform encryption and encapsulation,

## VPN connection

### Occurs within the context of a TCP/IP tunnel

## Tunnel

### Channel or pathway over a packet network used by the VPN

### Runs through the Internet from one endpoint to another

### Not a physical connection

## Virtual tunnel

### Communications path that makes use of Internet-based hosts and servers to conduct data from one network station to another

## Figure on (next page)

### Illustrates that VPNs in fact traverse the public Internet and must therefore handle the Internet’s protocols and procedures

## Devices that form the endpoints of the VPN:

### Server running VPN protocols

### VPN appliance

### Firewall/VPN combination

### Router-based VPN

## Certificate servers

### Manage certificates if that is required

## Client computers

### Run VPN client software

# Essential Activities of VPNs

## IP encapsulation

## Data payload encryption

## Encrypted authentication

# IP Encapsulation

## Hides the source and destination information of the encapsulated packets

## Encapsulating packet

### Uses the source and destination addresses of the VPN gateway

## Encapsulated packet

### Source and destination IP addresses can be in the private reserved blocks

### Not usually routable over the Internet

# Data Payload Encryption

## Encryption accomplished using:

### Transport mode

#### Host encrypts data when it is transmitted

#### Header is not encrypted

### Tunnel mode

#### Traffic encrypted in transit

#### Entire packet encrypted, including headers

## Level of encryption varies

### Longer keys makes stronger encryption

# Encrypted Authentication

## Encryption domain

### Everything in the protected network and behind the gateway

## Authentication

### Hosts in the network need to know that originating host is an approved user of the VPN

### Exchange of long blocks of code, called keys

#### Generated by complex formulas called algorithms

# Benefits and Drawbacks of VPNs

## Benefits

### Secure networking without the expense of establishing and maintaining leased lines

### Allow the packet encryption/translation overhead to be done on dedicated systems

### Provide control of physical setup

## Drawbacks

### Complex

### If configured improperly, can create significant network vulnerabilities

### Make use of the unpredictable and often unreliable Internet

### Can expose inner workings of network

#### If misconfigured

# VPNs Extend Network Boundaries

## VPN users will have a high-speed connection that is always on

### Can be connected to network around the clock

### Exposes your network to viruses & intrusions

## Guidelines for “always-on” users

### Use two or more authentication tools to identify remote user

#### Multifactor authentication

### Integrate virus protection

### Use Network Access Control (NAC)

#### Checks security settings on the remote computer before granting access

#### Microsoft's Quarantine Server (links Ch 10a, 10b)

### Set usage limits

# Types of VPNs

## Site-to-site VPN

### Links two or more networks

## Client-to-site

### Makes a network accessible to remote users

## Options for configuring VPNs:

### Hardware systems, software systems, and systems that combine hardware and software

# VPN Appliances

## General-purpose hardware device

### Such as a router

### Configure to use IPSec or another VPN protocol

## Obtain a VPN appliance

### Hardware device specially designed to serve as the endpoint for one or more VPNs

# Hardware VPN

# Software VPN Systems

## Less expensive than hardware systems

## Tend to scale better on fast-growing networks

## Use policy manager systems

### For enterprise-wide software distribution, policy creation, and management

## Allows multiple configuration profiles for end users

## SSL-based VPNs

### Use the SSL protocol instead of IPSec

### Only allow access to Web-enabled applications

# VPN Combinations of Hardware and Software

## Implement a VPN appliance at the central network

### Use client software at the remote end of each VPN connection

## Client mode

### Concentrator acts as a software client

## Network extension mode

### Concentrator acts as a hardware device

### Enables a secure site-to-site VPN connection

# Mixed Vendor VPNs

## Might have different vendors for:

### Certificates

### Client software

### VPN termination

## Pick a standard security protocol that is widely supported by all the devices

### Such as IPSec

# VPN Setups

## When three or more networks or individuals need to be connected:

### Mesh configuration

### Hub-and-spoke arrangement

### Hybrid setup

# Mesh Configuration

## Each participant has an approved relationship with every other participant

### Called a security association (SA)

## Need to specifically identify each of these participants to every other participant that uses the VPN

## Figure 10-5

### Four separate LANs are joined in a mesh VPN

# Hub-and-Spoke Configuration

## Single VPN router contains records of all SAs in the VPN

## Any LANs or computers need to connect to the central server

## Makes it easy to increase the size of the VPN

### As more branch offices or computers are added

### Central point of administration is more secure

## Drawback: centralization slows down communications

# Hub-and-Spoke VPN

# Hybrid Configuration

## Mesh configurations tend to operate more efficiently

## As branch offices are added

### Add as spokes that connect to a central VPN router at the central office

## Hybrid setup

### Combines the two configurations

### Benefits from the strengths of each one

# Configurations and Extranet and Intranet Access

## Extranet

### Each end of the VPN represents an extension of your organizational network to a new location

### Each remote user or business partner should have firewalls and antivirus software enabled

## Intranet

### Give parts of organization access to other parts

### VPN users inside organization should have usage limits and antivirus and firewall protection

# Tunneling Protocols Used with VPNs

## Widespread acceptance of the IPSec protocol with the Internet Key Exchange (IKE) system

### Proprietary protocols are used far less often than in the past

# IPSec/IKE

## IPSec

### Standard for secure encrypted communications

### Two security methods:

#### Authenticated Headers (AH) and Encapsulating Security Payload (ESP)

### Different modes:

#### Transport mode and tunnel mode

## However, tunnel mode IPSec does not provide user authentication, so Kerberos or some other authentication protocol will be needed as well

# Transport Mode v. Tunnel Mode

## Transport mode

### Host encrypts data when it is transmitted

### Header is not encrypted

### Can be used to reach any IP address

## Tunnel mode

### Traffic encrypted in transit

### Entire packet encrypted, including headers

### Intended for point-to-point tunnel between two fixed IP addresses

## IKE  (Internet Key Exchange)

### Means of using public key cryptography to encrypt data between LANs or between a client and a LAN

### Provides for the exchange of public and private keys

## If VPN uses more than one kind of firewall

### Check with the manufacturers of those firewalls to see if their products will work with the other firewalls you have

# PPTP

## Point-to-Point Tunneling Protocol (PPTP)

## Commonly used to a network using a dial-in modem connection

## Uses Microsoft Point-to-Point Encryption (MPPE)

## Useful if support for older clients is needed

# L2TP

## Layer 2 Tunneling Protocol (L2TP)

## Extension of the protocol long used to establish dialup connections on the Internet

## Uses IPSec rather than MPPE to encrypt data

## Provides secure authenticated remote access by encapsulating data into packets that are sent over a PPP channel

# PPP Over SSL and PPP Over SSH

## Point-to-Point Protocol (PPP) over Secure Sockets Layer (SSL) and Point-to-Point Protocol (PPP) Over Secure Shell (SSH)

## UNIX-based methods for creating VPNs

## Combine an existing tunnel system (PPP) with a way of encrypting data in transport (SSL or SSH)

## SSL

### Public key encryption system

### Used to provide secure communications over the World Wide Web

# Enabling Remote Access Connections Within VPNs

## Issue VPN client software user

## Make sure the user’s computer is equipped with antivirus software and a firewall

## May need to obtain a key for the remote user

# Configuring the Server

## Firewall-based VPN

### Identify the client computer

### Major operating systems incorporate their own methods of providing secure remote access

## Linux: IP Masquerade

### Similar to a transparent proxy (links Ch 10c, 10d)

## Windows: Wizard that makes it easy to set up a workstation to make a VPN connection

# Configuring Clients

## Configure each client that wants to use the VPN

## Consider whether:

### The client software will work with all client platforms

### The client workstation is itself protected by a firewall

## All users of VPN extend the LAN

### Open up a new “hole” through which viruses and hackers can gain access

# VPN Best Practices

## Security policy rules that specifically apply to the VPN

## Integration of firewall packet filtering with VPN traffic

## Auditing the VPN to make sure it is performing acceptably

# The Need for a VPN Policy

## Essential for:

### Identifying who can use the VPN

### Ensuring that all users know what constitutes proper use of the VPN

## Policy should state:

### Who should have VPN access to network

### Whether authentication is to be used and how it is to be used

### Whether split tunneling is permitted

### How long users can be connected using the VPN at any one session

### Whether virus protection is included

# Connecting from Personal Computers

## Remote users must be carefully trained

## Understand that all organizational security policies apply during VPN use

### Even when connecting from personal equipment

# Packet Filtering and VPNs

## Decide where data encryption and decryption will be performed in relation to packet filtering

## Figure 10-11

### Encryption and decryption outside the packet-filtering perimeter

# Transport Encryption (Encryption Prevents Filtering)

# Figure 10-12

### Encryption and decryption performed inside the packet-filtering perimeter using the tunnel method

# Tunnel Encryption (Encryption does not Prevent Filtering)

# PPTP Filters

## PPTP uses TCP 1723 as the control channel

## It also uses Generic Routing Encapsulation to carry data

### Protocol ID 47

### Link Ch 10e

# L2TP and IPSec Filters

## Negotiation of IPsec security association (SA), typically through Internet key exchange (IKE)

### UDP port 500

## Establishment of Encapsulating Security Payload (ESP) communication in transport mode

### The IP protocol number for ESP is 50

## Negotiation and establishment of L2TP tunnel between the SA endpoints

### UDP port 1701

## From Link Ch 10f

# L2TP and IPSec Filters

## Note: This is Table 10-3 in the book and it is mislabeled "PPTP"

# Auditing and Testing the VPN

## Test VPN client on each computer that might use the VPN

## Choose client software that is easy for end users to install on their own

### Save time and effort

## Check the VPN

### Make sure files are being transferred at an acceptable rate

### All parts of the VPN remain online when needed

## If part of the network goes down frequently:

### Switch to another ISP

### Asking ISPs questions to help determine which ones are best

## Last modified: 8 am 11-8-11