Objectives

## Explain wireless technology

## Describe wireless networking standards

## Describe the process of authentication

## Describe wardriving

## Describe wireless hacking and tools used by hackers and security professionals

Understanding Wireless Technology

## For a wireless network to function, you must have the right hardware and software

## Wireless technology is part of our lives

### Baby monitors

### Cell and cordless phones

### Pagers

### GPS

### Remote controls

### Garage door openers

### Two-way radios

### Wireless PDAs

# Components of a Wireless Network

## A wireless network has only three basic components

### Access Point (AP)

### Wireless network interface card (WNIC)

### Ethernet cable

# Access Points

## An access point (AP) is a transceiver that connects to an Ethernet cable

### It bridges the wireless network with the wired network

#### Not all wireless networks connect to a wired network

### Most companies have Wireless LANs (WLANs) that connect to their wired network topology

## The AP is where channels are configured

## An AP enables users to connect to a LAN using wireless technology

### An AP is available only within a defined area

# Service Set Identifiers (SSIDs)

## Name used to identify the wireless local area network (WLAN)

## The SSID is configured on the AP

### Unique 1- to 32-character alphanumeric name

### Name is case sensitive

## Wireless computers need to configure the SSID before connecting to a wireless network

## SSID is transmitted with each packet

### Identifies which network the packet belongs

## The AP usually broadcasts the SSID

## Many vendors have SSIDs set to a default value that companies never change

## An AP can be configured to not broadcast its SSID until after authentication

### Wireless hackers can attempt to guess the SSID

## Verify that your clients or customers are not using a default SSID

## See links Ch 11a, b

# Configuring an Access Point

## Configuring an AP varies depending on the hardware

### Most devices allow access through any Web browser

### Enter IP address on your Web browser and provide your user logon name and password

# Wireless Router

## A wireless router includes an access point, a router, and a switch

# Configuring an Access Point

## Wireless Configuration Options

### SSID

### Wired Equivalent Privacy (WEP) encryption

### WPA (WiFi Protected Access ) is better

## Steps for configuring a D-Link wireless router (continued)

### Turn off SSID broadcast

### You should also change your SSID

# Wireless NICs

## For wireless technology to work, each node or computer must have a wireless NIC

## NIC’s main function

### Converting the radio waves it receives into digital signals the computer understands

# Wireless NICs

## There are many wireless NICs on the market

### Choose yours depending on how you plan to use it

### Some tools require certain specific brands of NICs

Understanding Wireless Network Standards

## A standard is a set of rules formulated by an organization

## Institute of Electrical and Electronics Engineers (IEEE)

### Defines several standards for wireless networks

# IEEE: CCSF Student Chapter

## Next meeting:

### May 3, 2007 in Cloud 218 4:30 pm

## Email sbowne@ccsf.edu for more info

# IEEE Standards

## Standards pass through these groups:

### Working group (WG)

### Sponsor Executive Committee (SEC)

### Standards Review Committee (RevCom)

### IEEE Standards Board

## IEEE Project 802

### LAN and WAN standards

# The 802.11 Standard

## The first wireless technology standard

## Defined wireless connectivity at 1 Mbps and 2 Mbps within a LAN

## Applied to layers 1 and 2 of the OSI model

## Wireless networks cannot detect collisions

### Carrier sense multiple access/collision avoidance (CSMA/CA) is used instead of CSMA/CD

# Addressing

## Wireless LANs do not have an address associated with a physical location

### An addressable unit is called a station (STA)

# The Basic Architecture of 802.11

## 802.11 uses a basic service set (BSS) as its building block

### Computers within a BSS can communicate with each other

### To connect two BSSs, 802.11 requires a distribution system (DS)

# Frequency Range

## In the United States, Wi-Fi uses frequencies near 2.4 GHz

#### (Except 802.11a at 5 GHz)

### There are 11 channels, but they overlap, so only three are commonly used

#### See link Ch 11c (cisco.com)

## Other terms to define the channel:

### Wavelength

### Frequency

### Cycle

### Hertz or cycles per second

### Bands

# Infrared (IR)

## Infrared light can’t be seen by the human eye

## IR technology is restricted to a single room or line of sight

## IR light cannot penetrate walls, ceilings, or floors

### Image: IR transmitter for wireless headphones

# Narrowband

## Uses microwave radio band frequencies to transmit data

## Popular uses

### Cordless phones

### Garage door openers

# Spread Spectrum

## Data is spread across a large-frequency bandwidth instead of traveling across just one frequency band

## Methods

### Frequency-hopping spread spectrum (FHSS)

### Direct sequence spread spectrum (DSSS)

### Orthogonal frequency division multiplexing (OFDM)

#### See links Ch 11d, Ch 11d1

# IEEE Additional 802.11 Projects

## 802.11a

### Created in 1999

### Operating frequency 5 GHz

### Throughput 54 Mbps

## 802.11b

### Operates in the 2.4 GHz range

### Throughput 11 Mbps

### Also referred as Wi-Fi (wireless fidelity)

### Allows for 11 channels to prevent overlapping signals

#### Effectively only three channels (1, 6, and 11) can be used in combination without overlapping

### Introduced Wired Equivalent Privacy (WEP)

## 802.11e

### It has improvements to address the problem of interference

#### When interference is detected, signals can jump to another frequency more quickly

## 802.11g

### Operates in the 2.4 GHz range

### Uses OFDM for modulation

### Throughput increased from 11 Mbps to 54 Mbps

## 802.11i

### Introduced Wi-Fi Protected Access (WPA)

### Corrected many of the security vulnerabilities of 802.11b

## 802.15

### Addresses networking devices within one person’s workspace

#### Called wireless personal area network (WPAN)

### Bluetooth is a common example

## 802.16

### Addresses the issue of wireless metropolitan area networks (MANs)

### Defines the WirelessMAN Air Interface

### It will have a range of up to 30 miles

### Throughput of up to 120 Mbps

## 802.20

### Addresses wireless MANs for mobile users who are sitting in trains, subways, or cars traveling at speeds up to 150 miles per hour

## Bluetooth

### Defines a method for interconnecting portable devices without wires

### Maximum distance allowed is 10 meters

### It uses the 2.45 GHz frequency band

### Throughput of up to 12 Mbps

## HiperLAN2

### European WLAN standard

### It is not compatible with 802.11 standards

Understanding Authentication

## Wireless technology brings new security risks to a network

## Authentication

### Establishing that a user is authentic—authorized to use the network

### If authentication fails, anyone in radio range can use your network

# The 802.1X Standard

## Defines the process of authenticating and authorizing users on a WLAN

## Basic concepts

### Point-to-Point Protocol (PPP)

### Extensible Authentication Protocol (EAP)

### Wired Equivalent Privacy (WEP)

### Wi-Fi Protected Access (WPA)

# Point-to-Point Protocol (PPP)

## Many ISPs use PPP to connect dial-up or DSL users

## PPP handles authentication with a user name and password, sent with PAP or CHAP

### PAP (Password Authentication Protocol) sends passwords unencrypted

#### Vulnerable to trivial sniffing attacks

## See link Ch 11f

# CHAP Vulnerability

## CHAP (Challenge-Handshake Authentication Protocol)

### Server sends a Challenge with a random value

### Client sends a Response, hashing the random value with the secret password

## This is still vulnerable to a sort of session hijacking attack (see links Ch 11e)

# Extensible Authentication Protocol (EAP)

## EAP is an enhancement to PPP

## Allows a company to select its authentication method

### Certificates

### Kerberos

#### Kerberos is used on LANs for authentication

#### Uses Tickets and Keys

#### Used by Windows 2000, XP, and 2003 Server by default

#### Not common on WLANS (I think)

# X.509 Certificate

## Record that authenticates network entities

## Identifies

### The owner

### The certificate authority (CA)

### The owner’s public key

#### See link Ch 11j

# Sample X.509 Certificate

## Go to gmail.com

## Double-click the padlock

# Public Key

## Your browser uses the Public Key to encrypt data so only Gmail can read it

# LEAP

## Lightweight Extensible Authentication Protocol (LEAP)

### A Cisco product

### Vulnerable, but Cisco didn’t care

### Joshua Wright wrote the ASLEAP hacking tool to crack LEAP, and forced Cisco to develop a better protocol

#### See link Ch 11g

# More Secure EAP Methods

## Extensible Authentication Protocol-Transport Layer Security (EAP-TLS)

### Secure but rarely used, because both client and server need certificates signed by a CA

## Protected EAP (PEAP) and Microsoft PEAP

### Very secure, only requires server to have a certificate signed by a CA

## See link Ch 11h

# 802.1X components

## Supplicant

### The user accessing a WLAN

## Authenticator

### The AP

## Authentication server

### Checks an account database to see if user’s credentials are acceptable

### May use RADIUS (Remote Access Dial-In User Service)

## See link Ch 11k

# Wired Equivalent Privacy (WEP)

## Part of the 802.11b standard

## Encrypts data on a wireless network

## WEP has many vulnerabilities

## To crack WEP, see links Ch 11l, 11m

# Wi-Fi Protected Access (WPA)

## Specified in the 802.11i standard

## Replaces WEP

## WPA improves encryption by using Temporal Key Integrity Protocol (TKIP)

# TKIP Enhancements

## Message Integrity Check (MIC)

### Prevent attacker from injecting forged packets

## Extended Initialization Vector (IV) with sequencing rules

### Prevent replays (attacker re-sending copied packets)

## Per-packet key mixing

### MAC addresses are used to create a key

### Each link uses a different key

## Rekeying mechanism

### Provides fresh keys

### Prevents attackers from reusing old keys

# WPA Adds 802.1x

## WPA also adds an authentication mechanism implementing 802.1X and EAP

### This was not available in WEP

Understanding Wardriving

## Hackers use wardriving

### Finding insecure access points

### Using a laptop or palmtop computer

## Wardriving is not illegal

### But using the resources of these networks is illegal

## Warflying

### Variant where an airplane is used instead of a car

# How It Works

## An attacker or security tester simply drives around with the following equipment

### Laptop computer

### Wireless NIC

### An antenna

### Software that scans the area for SSIDs

## Not all wireless NICs are compatible with scanning programs

## Antenna prices vary depending on the quality and the range they can cover

## Scanning software can identify

### The company’s SSID

### The type of security enabled

### The signal strength

#### Indicating how close the AP is to the attacker

# NetStumbler

## Shareware tool written for Windows that enables you to detect WLANs

### Supports 802.11a, 802.11b, and 802.11g standards

## NetStumbler was primarily designed to

### Verify your WLAN configuration

### Detect other wireless networks

### Detect unauthorized APs

## NetStumbler is capable of interface with a GPS

### Enabling a security tester or hacker to map out locations of all the WLANs the software detects

## NetStumbler logs the following information

### SSID

### MAC address and Manufacturer of the AP

### Channel

### Signal Strength

### Encryption

## Can detect APs within a 350-foot radius

### With a good antenna, they can locate APs a couple of miles away

# Kismet

## Another product for conducting wardriving attacks

## Runs on Linux, BSD, MAC OS X, and Linux PDAs

## Kismet is advertised also as a sniffer and IDS

### Kismet can sniff 802.11b, 802.11a, and 802.11g traffic

# Kismet features

### Ethereal- and Tcpdump-compatible data logging

### AirSnort compatible

### Network IP range detection

### Hidden network SSID detection

### Graphical mapping of networks

### Client-server architecture

### Manufacturer and model identification of APs and clients

### Detection of known default access point configurations

### XML output

### Supports 20 card types

Understanding Wireless Hacking

## Hacking a wireless network is not much different from hacking a wired LAN

## Techniques for hacking wireless networks

### Port scanning

### Enumeration

# Tools of the Trade

## Equipment

### Laptop computer

### A wireless NIC

### An antenna

### Sniffer software

# AirSnort

## Created by Jeremy Bruestle and Blake Hegerle

## It is the tool most hackers wanting to access WEP-enabled WLANs use

## AirSnort limitations

### Runs on either Linux or Windows (textbook is wrong)

### Requires specific drivers

### Not all wireless NICs function with AirSnort

#### See links Ch 11p, 11q

# WEPCrack

## Another open-source tool used to crack WEP encryption

### WEPCrack was released about a week before AirSnort

## It also works on *NIX systems

## WEPCrack uses Perl scripts to carry out attacks on wireless systems

### AirSnort is considered better (link Ch 11r)

Countermeasures for Wireless Attacks

## Anti-wardriving software makes it more difficult for attackers to discover your wireless LAN

### Honeypots

#### Servers with fake data to snare intruders

### Fakeap and Black Alchemy Fake AP

#### Software that makes fake Access Points

##### Link Ch 11s

## Use special paint to stop radio from escaping your building

## Allow only predetermined MAC addresses and IP addresses to have access to the wireless LAN

## Use an authentication server instead of relying on a wireless device to authenticate users

## Use an EAP authentication protocol

## If you use WEP, use 104-bit encryption rather than 40-bit encryption

### But just use WPA instead

Assign static IP addresses to wireless clients instead of using DHCP

DMZ

## Don’t broadcast the SSID

## Place the AP in the demilitarized zone (DMZ) (image from wikipedia)

Last modified 4-15-07 5 pm