Router

Objectives

## Describe network security devices

## Describe firewall technology

## Describe intrusion detection systems

## Describe honeypots

Routers

## Routers are like intersections; switches are like streets

### Image from Wikipedia (link Ch 13a)

# Understanding Routers

## Routers are hardware devices used on a network to send packets to different network segments

### Operate at the network layer of the OSI model

# Routing Protocols

## Routers tell one another what paths are available with Routing Protocols

### Link-state routing protocol

#### Each router has complete information about every network link

#### Example: Open Shortest Path First (OSPF)

### Distance-vector routing protocol

#### Routers only know which direction to send packets, and how far

#### Example: Routing Information Protocol (RIP)

# Cisco Routers

## Image from cisco.com (link Ch 13b)

# Understanding Basic Hardware Routers

## Cisco routers are widely used in the networking community

### More than one million Cisco 2500 series routers are currently being used by companies around the world

## Vulnerabilities exist in Cisco as they do in any operating system

### See link Ch 13c

# Cisco Router Components

## Internetwork Operating System (IOS)

## Random access memory (RAM)

### Holds the router’s running configuration, routing tables, and buffers

### If you turn off the router, the contents stored in RAM are wiped out

## Nonvolatile RAM (NVRAM)

### Holds the router’s configuration file, but the information is not lost if the router is turned off

## Flash memory

### Holds the IOS the router is using

### Is rewritable memory, so you can upgrade the IOS

## Read-only memory (ROM)

### Contains a minimal version of the IOS used to boot the router if flash memory gets corrupted

## Interfaces

### Hardware connectivity points

### Example: an Ethernet port is an interface that connects to a LAN

# Michael Lynn

## He presented a major Cisco security vulnerability at the Black Hat security conference in 2005

## He lost his job, was sued, conference materials were confiscated, etc.

### See links Ch 13 d, e, f, g

## Cisco IOS is controlled from the command line

## The details are not included in this class

## Skip pages 324-329

Understanding Firewalls

## Firewalls are hardware devices or software installed on a system and have two purposes

### Controlling access to all traffic that enters an internal network

### Controlling all traffic that leaves an internal network

# Hardware Firewalls

## Advantage of hardware firewalls

### Faster than software firewalls (more throughput)

## Disadvantages of hardware firewalls

### You are limited by the firewall’s hardware

#### Number of interfaces, etc.

### Usually filter incoming traffic only (link Ch 13i)

# Software Firewalls

## Advantages of software firewalls

### Customizable: can interact with the user to provide more protection

### You can easily add NICs to the server running the firewall software

# Software Firewalls

## Disadvantages of software firewalls

### You might have to worry about configuration problems

### They rely on the OS on which they are running

# Firewall Technologies

## Network address translation (NAT)

## Access control lists (Packet filtering)

## Stateful packet inspection (SPI)

# Network Address Translation (NAT)

## Internal private IP addresses are mapped to public external IP addresses

### Hides the internal infrastructure

## Port Address Translation (PAT)

### This allows thousands of internal IP addresses to be mapped to one external IP address

### Each connection from the private network is mapped to a different public port

# Access Control Lists

## A series of rules to control traffic

## Criteria

### Source IP address

### Destination IP address

### Ports or services

### More possibilities

## Same as “Packet Filtering”

# Stateful Packet Inspection (SPI)

## Stateful packet filters examine the current state of the network

### If you have sent a request to a server, packets from that server may be allowed in

### Packets from the same server might be blocked if no request was sent first

# State Table

## Stateful firewalls maintain a state table showing the current connections

# ACK Port scan

## Used to get information about a firewall

## Stateful firewalls track connection and block unsolicited ACK packets

## Stateless firewalls only block incoming SYN packets, so you get a RST response

## We covered this in chapter 5

# Stateful Packet Inspection (SPI)

## Stateful packet filters recognize types of anomalies that most routers ignore

## Stateless packet filters handle each packet on an individual basis

### This makes them less effective against some attacks

# Implementing a Firewall

## Using only one firewall between a company’s internal network and the Internet is dangerous

### It leaves the company open to attack if a hacker compromises the firewall

## Use a demilitarized zone instead

# Demilitarized Zone (DMZ)

## DMZ is a small network containing resources available to Internet users

### Helps maintain security on the company’s internal network

## Sits between the Internet and the internal network

## It is sometimes referred to as a “perimeter network”

# Understanding the Private Internet Exchange (PIX) Firewall

## Cisco PIX firewall

### One of the most popular firewalls on the market

# Configuration of the PIX Firewall

## Working with a PIX firewall is similar to working with any other Cisco router

## Login prompt

### If you are not authorized to be in this XYZ Hawaii network device,

### log out immediately!

### User Access Verification

### Password:

### This banner serves a legal purpose

### A banner that says “welcome” may prevent prosecution of hackers who enter

# PIX Firewall Features

## One PIX can be used to create a DMZ

### See link Ch 13k

# PIX Firewall Features

## Unicast Reverse Path Forwarding

### Also known as "reverse route lookup"

### Checks to see that packets have correct source IP addresses

## Flood Defender

### Prevents SYN Floods

### Only a limited number of "embryonic connections" are allowed

# PIX Firewall Features

## FragGuard and Virtual Re-Assembly

### Re-assembles IP fragments to prevent some DoS attacks, like the Ping of Death and Teardrop

## Limits

### DNS Responses

### ActiveX controls

### Java applets

## I skipped pages 333-336

# Microsoft ISA

## Internet Security and Acceleration (ISA)

## Microsoft’s software approach to firewalls

## Microsoft Internet Security and Acceleration (ISA) Server

### Software that runs on a Windows Server

### Functions as a software router, firewall, and IDS (Intrusion Detection System)

## ISA protects your network from Internet threats

## ISA lets remote users connect securely, handling authentication and encryption

#### Image from microsoft.com link Ch 13m

## ISA has the same functionality as any hardware router

### Packet filtering to control incoming traffic

### Application filtering through the examination of protocols

### Intrusion detection filters

### Access policies to control outgoing traffic

# IP Packet Filters

## ISA enables administrators to filter IP traffic based on the following:

### Source and destination IP address

### Network protocol, such as HTTP

### Source port or destination port

## ISA provides a GUI for these configurations

### A network segment can be denied or allowed HTTP access in the Remote Computer tab

Denying access to port 80 for the specified subnet

# Application Filters

## Can accept or deny data from specific applications or data containing specific content

## SMTP filter can restrict

### E-mail with specific attachments

### E-mail from a specific user or domain

### E-mail containing specific keywords

### SMTP commands

## Email can also be filtered based o

### Sender's name

### Sender's domain

### Keywords like VIAGRA or Mortgage

#### These techniques are not very effective—spammers know how to defeat them

## SMTP Commands tab

### Administrator can prevent a user from running SMTP commands

## FTP Access filter

## H.323 filter

### real-time multimedia conferences

## See link Ch 13n

# Intrusion Detection Filters

## Analyze all traffic for possible known intrusions

### DNS intrusion detection filter

### POP filter

### RPC filter

### SMTP filter

### SOCKS filter

### Streaming Media filter

### Web Proxy filter

Intrusion Detection Systems (IDSs)

## Monitor network devices so that security administrators can identify attacks in progress and stop them

## An IDS looks at the traffic and compares it with known exploits

### Similar to virus software using a signature file to identify viruses

## Types

### Network-based IDSs

### Host-based IDSs

# Network-based IDSs

### Monitor activity on network segments

### They sniff traffic and alert a security administrator when something suspicious occurs

#### See link Ch 13o

# Host-based IDSs

### The software is installed on the server you’re attempting to protect, like antivirus software

### Used to protect a critical network server or database server

# Passive and Active IDSs

## IDSs are categorized by how they react when they detect suspicious behavior

### Passive systems

#### Send out an alert and log the activity

#### Don't try to stop it

### Active systems

#### Log events and send out alerts

#### Can also interoperate with routers and firewalls to block the activity automatically

Understanding Honeypots

## Honeypot

### Computer placed on the perimeter of a network

### Contains information intended to lure and then trap hackers

## Computer is configured to have vulnerabilities

## Goal

### Keep hackers connected long enough so they can be traced back

# How They Work

## A honeypot appears to have important data or sensitive information stored on it

### Could store fake financial data that tempts hackers to attempt browsing through the data

## Hackers will spend time attacking the honeypot

### And stop looking for real vulnerabilities in the company’s network

## Honeypots also enable security professionals to collect data on attackers

## Virtual honeypots

### Honeypots created using software solutions instead of hardware devices

### Example: Honeyd

# Project Honey Pot

## Web masters install software on their websites

## When spammers harvest email addresses from sites, HoneyNet's servers record the IP of the harvester

### Can help prosecute the spammers and block the spam

#### Link Ch 13p

## Uses a Capture Server and one or more Capture Clients

### The clients run in virtual machines

### Clients connect to suspect Web servers

### If the client detects an infection, it alerts the Capture Server and restores itself to a clean state

### The server gathers data about malicious websites

#### See link Ch 13q

Last modified 6-4-09