## Legal Concerns

## Defeating security to enter a network without permission is clearly illegal

### Even if the security is weak

## Sniffing unencrypted wireless traffic may also be illegal

### It could be regarded as an illegal wiretap

### The situation is unclear, and varies from state to state

### In California, privacy concerns tend to outweigh other considerations

### See links l14v, l14w

Equipment

## Wireless Network Interface Cards (NICs) and Drivers

# The Goal

## All wireless NICs can connect to an Access Point

## But hacking requires more than that, because we need to do

### Sniffing – collecting traffic addressed to other devices

### Injection – transmitting forged packets which will appear to be from other devices

# Windows v. Linux

## The best wireless hacking software is written in Linux

### The Windows tools are inferior, and don't support packet injection

## But all the wireless NICs are designed for Windows

### And the drivers are written for Windows

### Linux drivers are hard to find and confusing to install

# Wireless NIC Modes

## There are four modes a NIC can use

### Master mode

### Managed mode

### Ad-hoc mode

### Monitor mode

#### See link l_14j

# Master Mode

### Also called AP or Infrastructure mode

### Looks like an access point

### Creates a network with

#### A name (SSID)

#### A channel

# Managed Mode

### Also called Client mode

### The usual mode for a Wi-Fi laptop

### Joins a network created by a master

### Automatically changes channel to match the master

### Presents credentials, and if accepted, becomes associated with the master

# Ad-hoc Mode

### Peer-to-peer network

### No master or Access Point

### Nodes must agree on a channel and SSID

# Monitor Mode

### Does not associate with Access Point

### Listens to traffic

### Like a wired NIC in Promiscuous Mode

# Wi-Fi NICs

To connect to a Wi-Fi network, you need a Network Interface Card (NIC)

PCMCIA

# PCMCIA

## The most common type is the PCMCIA card

### Designed for laptop computers

# USB

### Can be used on a laptop or desktop PC

# PCI

### Installs inside a desktop PC

# Choosing a NIC

## For penetration testing (hacking), consider these factors:

Chipset

USB

### Output power

### Receiving sensitivity

### External antenna connectors

### Support for 802.11i and improved WEP versions

# Wi-Fi NIC Manufacturers

## Each wireless card has two manufacturers

### The card itself is made by a company like

#### Netgear

#### Ubiquiti

#### Linksys

#### D-Link

#### many, many others

### But the chipset (control circuitry) is made by a different company

# Chipsets

## To find out what chipset your card uses, you must search on the Web

Card manufacturer's don't want you to know

PCI

## Major chipsets:

### Prism

### Cisco Aironet

### Hermes/Orinoco

### Atheros

#### There are others

# Prism Chipset

## Prism chipset is a favorite among hackers

### Completely open -- specifications available

### Has more Linux drivers than any other chipset

#### See link l_14d

## Prism chipset is the best choice for penetration testing

## HostAP Linux Drivers are highly recommended, supporting:

### NIC acting as an Access Point

### Use of the iwconfig command to configure the NIC

#### See link l_14h

# Cisco Aironet Chipset

## Cisco proprietary – not open

## Based on Prism, with more features

### Regulated power output

### Hardware-based channel-hopping

## Very sensitive – good for wardriving

### Cannot use HostAP drivers

### Not useful for man-in-the-middle or other complex attacks

# Hermes Chipset

## Lucent proprietary – not open

## Lucent published some source code for WaveLAN/ORiNOCO cards

## Useful for all penetration testing, but require

### Shmoo driver patches (link l_14l) to use monitor mode

# Atheros Chipset

## The most common chipset in 802.11a devices

### Best Atheros drivers are MadWIFI (link l_14m)

### Some cards work better than others

### Monitor mode is available, at least for some cards

# Other Cards

## If all else fails, you could use Windows drivers with a wrapper to make them work in Linux

### DriverLoader (link l_14n)

### NdisWrapper (link l_14o)

## But all you'll get is basic functions, not monitor mode or packet injection

### Not much use for hacking

Cracking WEP: Tools and Principles

# A Simple WEP Crack

## The Access Point and Client are using WEP encryption

## The hacker device just listens

# Listening is Slow

## You need to capture 50,000 to 200,000 "interesting" packets to crack a 64-bit WEP key

### The "interesting" packets are the ones containing Initialization Vectors (IVs)

### Only about ¼ of the packets contain IVs

### So you need 200,000 to 800,000 packets

## It can take hours or days to capture that many packets

# Packet Injection

## A second hacker machine injects packets to create more "interesting packet"

# Injection is MUCH Faster

## With packet injection, the listener can collect 200 IVs per second

## 5 – 10 minutes is usually enough to crack a 64-bit key

## Cracking a 128-bit key takes an hour or so

### Link l_14r

# AP & Client Requirements

## Access Point

### Any AP that supports WEP should be fine (they all do)

## Client

### Any computer with any wireless card will do

### Could use Windows or Linux

# Listener Requirements

## NIC must support Monitor Mode

## Could use Windows or Linux

### But you can't use NDISwrapper

## Software

### Airodump (part of the Aircrack Suite) for Windows or Linux (see Link l_14q)

### BackTrack is a live Linux CD with Aircrack on it (and many other hacking tools)

#### Link l_14n

# Injector Requirements

## NIC must support injection

## Must use Linux

## Software

### void11 and aireplay

#### Link l_14q

# Sources

## http://www.aircrack-ng.org/doku.php?id=compatible_cards (link l_14a)

## http://www.wi-foo.com/ (link l_14c)

## http://www.vias.org/wirelessnetw/wndw_05_04.html (link l_14j)

## http://smallnetbuilder.com/content/view/24244/98/ (link l_14p)

Last modified 5-11-09