How HTTPS Works

# HTTP v. HTTPS

## HTTP doesn't encrypt data at all

### You can sniff traffic with Wireshark, ettercap, etc.

### Completely insecure

## HTTPS uses public-key encryption to secure data

### Much safer, but it can still be cracked to some extent by a man-in-the-middle attack

# Components of HTTPS

## When you use a secure session (HTTPS), these protocols work together:

### Address Resolution Protocol (ARP)

### Domain Name System (DNS)

### Secure Sockets Layers (SSL)

# ARP Request and Reply

## Client wants to find Gateway

## ARP Request: Who has 192.168.2.1?

## ARP Reply:

### MAC: 00-30-bd-02-ed-7b has 192.168.2.1

# Demonstration: Sniffing ARP with Wireshark

## Start Wireshark capturing packets

## Clear the ARP cache

### arp –d *

## Ping the default gateway

# DNS Query and Response

## Client wants to find Gmail.com

## DNS Query: Where is Gmail.com?

## DNS Response:

### Gmail.com is at 64.233.171.83

# Demonstration: Sniffing DNS with Wireshark

## Start Wireshark capturing packets

## Clear the DNS cache

### ipconfig /flushdns

## Ping Gmail.com

# SSL Handshake

## SSL handshake has three stages:

### Hellos

### Certificate, Key Exchange, and Authentication

### "Change cipher spec" – handshake finished

#### The Gateway just forwards all this traffic to the Web server

# Demonstration: Sniffing SSL Handshake with Wireshark

## Start Wireshark capturing packets

## Open a browser and go to yahoo.com

## Click the My Mail button

Hand

Hello

Key

# Hand – these three packets are the TCP Handshake, which happens before the SSL handshake

Hello – these two packets are the Hellos, which start the SSL handshake

Key – these packets perform the last two stages of the SSL handshake:

### Certificate, Key Exchange, and Authentication

### "Change cipher spec" – handshake finished

# Open a Socket to Port 443

## This is the usual SYN, SYN/ACK, SYN TCP handshake

## Port 443 is used for HTTPS

# Hellos

## Client Hello

## Server sends Hello

### This exchange is used to agree on a protocol version and encryption method

# Certificate, Key Exchange, and Authentication

## Server sends Certificate

## Client sends Public Key

## Client Authenticates Certificate with Certificate Authority (not visible)

# Change Cipher Spec

## Server sends "Change Cipher Spec"

## Client sends "Change Cipher Spec"

## SSL Handshake is done, now client can send encrypted Application Data

# Summary of HTTPS Process

## SSL handshake has three stages:

### Hellos

### Certificate, Key Exchange, and Authentication

### "Change cipher spec" – handshake finished

Man-in-the-Middle Attack

# ARP Cache Poisoning

## The Linux utility 'arpspoof' sends a constant series of ARP REPLIES

## This diverts Ethernet traffic to the hacker

### Part of the 'dsniff' package

# DNS Spoofing

## The Linux utility 'dnspoof' listens for DNS queries

## Sends DNS responses sending Web server data to the hacker

### Part of the 'dsniff' package

# IP Routing

## 'fragrouter' can forward packets to their correct destination

## That allows normal Web surfing (HTTP)

### Part of the 'dsniff' package

### This could also be done with 'iptables'

# SSL Spoofing

## 'webmitm' creates a Certificate and intercepts SSL handshakes

### Part of the 'dsniff' package

# Limitations of the Attack

## The SSL spoofing is not perfect

## You can't actually log in and read email

### Internet Explorer sends your password to the hacker before giving up on the connection

### Firefox doesn't send your password to the hacker

# Sources

## Hacking videos from link l_15b

### How to decrypt SSL encrypted traffic using a man in the middle attack (Auditor).swf

### MITM Hijacking.wmv

## SSL Handshake information from l_15a (cs.bham.ac.uk)

Last modified 5-11-09