Objectives

## Explain wireless technology

## Describe wireless networking standards

## Describe the process of authentication

## Describe wardriving

## Describe wireless hacking and tools used by hackers and security professionals

Understanding Wireless Technology

## For a wireless network to function, you must have the right hardware and software

## Wireless technology is part of our lives

### Baby monitors

### Cell and cordless phones

### Pagers

### GPS

### Remote controls

### Garage door openers

### Two-way radios

### Wireless PDAs

# Components of a Wireless Network

## A wireless network has only three basic components

### Access Point (AP)

### Wireless network interface card (WNIC)

### Ethernet cable

# Access Points

## An access point (AP) is a transceiver that connects to an Ethernet cable

### It bridges the wireless network with the wired network

#### Not all wireless networks connect to a wired network

### Most companies have Wireless LANs (WLANs) that connect to their wired network topology

## The AP is where channels are configured

## An AP enables users to connect to a LAN using wireless technology

### An AP is available only within a defined area

# Service Set Identifiers (SSIDs)

## Name used to identify the wireless local area network (WLAN)

## The SSID is configured on the AP

### Unique 1- to 32-character alphanumeric name

### Name is case sensitive

## Wireless computers need to configure the SSID before connecting to a wireless network

## SSID is transmitted with each packet

### Identifies which network the packet belongs

## The AP usually broadcasts the SSID

## Many vendors have SSIDs set to a default value that companies never change

## An AP can be configured to not broadcast its SSID until after authentication

### Wireless hackers can attempt to guess the SSID

## Verify that your clients or customers are not using a default SSID

## See links Ch 11a, b

# Configuring an Access Point

## Configuring an AP varies depending on the hardware

### Most devices allow access through any Web browser

### Enter IP address on your Web browser and provide your user logon name and password

# Wireless Router

## A wireless router includes an access point, a router, and a switch

# Configuring an Access Point

## Wireless Configuration Options

### SSID

### Wired Equivalent Privacy (WEP) encryption

### WPA (WiFi Protected Access ) is better

### Turn off SSID broadcast

### You should also change your SSID

# Wireless NICs

## For wireless technology to work, each node or computer must have a wireless NIC

## NIC’s main function

### Converting the radio waves it receives into digital signals the computer understands

## There are many wireless NICs on the market

### Choose yours depending on how you plan to use it

### Some tools require certain specific brands of NICs

#### Cracking WEP requires NICs that can use monitor mode or perform packet injection

Understanding Wireless Network Standards

## A standard is a set of rules formulated by an organization

## Institute of Electrical and Electronics Engineers (IEEE)

### Defines several standards for wireless networks

# IEEE: CCSF Student Chapter

## Next meeting:

### May 3, 2007 in Cloud 218 4:30 pm

## Email sbowne@ccsf.edu for more info

# IEEE Standards

## Standards pass through these groups:

### Working group (WG)

### Sponsor Executive Committee (SEC)

### Standards Review Committee (RevCom)

### IEEE Standards Board

## IEEE Project 802

### LAN and WAN standards

# The 802.11 Standard

## The first wireless technology standard

## Defined wireless connectivity at 1 Mbps and 2 Mbps within a LAN

## Applied to layers 1 and 2 of the OSI model

## Wireless networks cannot detect collisions

### Carrier sense multiple access/collision avoidance (CSMA/CA) is used instead of CSMA/CD

# Addressing

## Wireless LANs do not have an address associated with a physical location

### An addressable unit is called a station (STA)

# The Basic Architecture of 802.11

## 802.11 uses a basic service set (BSS) as its building block

### Computers within a BSS can communicate with each other

### To connect two BSSs, 802.11 requires a distribution system (DS)

# Frequency Range

## In the United States, Wi-Fi uses frequencies near 2.4 GHz

#### (Except 802.11a at 5 GHz)

### There are 11 channels, but they overlap, so only three are commonly used

#### See link Ch 11c & Ch 11zh

## Other terms to define the channel:

### Wavelength

### Frequency

### Cycle

### Hertz or cycles per second

### Bands

# Infrared (IR)

## Infrared light can’t be seen by the human eye

## IR technology is restricted to a single room or line of sight

## IR light cannot penetrate walls, ceilings, or floors

### Image: IR transmitter for wireless headphones

# Narrowband

## Uses microwave radio band frequencies to transmit data

## Popular uses

### Cordless phones

### Garage door openers

# Spread Spectrum

## Data is spread across a large-frequency bandwidth instead of traveling across just one frequency band

## Methods

### Frequency-hopping spread spectrum (FHSS)

### Direct sequence spread spectrum (DSSS)

### Orthogonal frequency division multiplexing (OFDM)

#### See links Ch 11d, Ch 11d1

# IEEE Additional 802.11 Projects

## 802.11a

### Created in 1999

### Operating frequency 5 GHz

### Throughput 54 Mbps

## 802.11b

### Operates in the 2.4 GHz range

### Throughput 11 Mbps

### Also referred as Wi-Fi (wireless fidelity)

### Allows for 11 channels to prevent overlapping signals

#### Effectively only three channels (1, 6, and 11) can be used in combination without overlapping

### Introduced Wired Equivalent Privacy (WEP)

## 802.11e

### It has improvements to address the problem of interference

#### When interference is detected, signals can jump to another frequency more quickly

## 802.11g

### Operates in the 2.4 GHz range

### Uses OFDM for modulation

### Throughput increased from 11 Mbps to 54 Mbps

## 802.11i

### Introduced Wi-Fi Protected Access (WPA)

### Corrected many of the security vulnerabilities of 802.11b

## 802.11n

### Finalized in 2009

### Can use up to four spatial streams with multiple antennas

### Speeds up to 600 Mbps

#### Link 11zh

## 802.15

### Addresses networking devices within one person’s workspace

#### Called wireless personal area network (WPAN)

### Bluetooth is a common example

### Bluetooth

#### Defines a method for interconnecting portable devices without wires

#### Maximum distance allowed is 10 meters

#### It uses the 2.45 GHz frequency band

#### Throughput of up to 2.1 Mbps for Bluetooth 2.0

#### Up to 24 Mbps for Bluetooth 3.0 (not 12 Mbps as stated in the textbook, p. 316)

#### Link Ch 11zg

## HiperLAN2

### European WLAN standard

### It is not compatible with 802.11 standards

## 802.16

### Addresses the issue of wireless metropolitan area networks (MANs)

### Defines the WirelessMAN Air Interface

### It will have a range of up to 30 miles

### Throughput of up to 120 Mbps

### Clear WIMAX

#### 10 Mbps throughout whole cities

#### San Francisco is not yet covered

#### Link Ch 11zi

## 802.20

### Addresses wireless MANs for mobile users who are sitting in trains, subways, or cars traveling at speeds up to 150 miles per hour

24 Mbps

Understanding Authentication

## Wireless technology brings new security risks to a network

## Authentication

### Establishing that a user is authentic—authorized to use the network

### If authentication fails, anyone in radio range can use your network

# The 802.1X Standard

## Defines the process of authenticating and authorizing users on a WLAN

## Basic concepts

### Point-to-Point Protocol (PPP)

### Extensible Authentication Protocol (EAP)

### Wired Equivalent Privacy (WEP)

### Wi-Fi Protected Access (WPA)

# Point-to-Point Protocol (PPP)

## Many ISPs use PPP to connect dial-up or DSL users

## PPP handles authentication with a user name and password, sent with PAP or CHAP

### PAP (Password Authentication Protocol) sends passwords unencrypted

#### Vulnerable to trivial sniffing attacks

#### See link Ch 11f

# CHAP Vulnerability

## CHAP (Challenge-Handshake Authentication Protocol)

### Server sends a Challenge with a random value

### Client sends a Response, hashing the random value with the secret password

## This is still vulnerable to a sort of session hijacking attack (see links Ch 11e)

# Extensible Authentication Protocol (EAP)

## EAP is an enhancement to PPP

## Allows a company to select its authentication method

### Certificates

### Kerberos

#### Kerberos is used on LANs for authentication

#### Uses Tickets and Keys

#### Used by Windows 2000, XP, and 2003 Server by default

#### Not common on WLANS

# X.509 Certificate

## Record that authenticates network entities

## Identifies

### The owner

### The certificate authority (CA)

### The owner’s public key

#### See link Ch 11j

# Sample X.509 Certificate

## Go to gmail.com

## Double-click the padlock

# Public Key

## Your browser uses the Public Key to encrypt data so only Gmail can read it

# LEAP

## Lightweight Extensible Authentication Protocol (LEAP)

### A Cisco product

### Vulnerable, but Cisco didn’t care

### Joshua Wright wrote the ASLEAP hacking tool to crack LEAP, and forced Cisco to develop a better protocol

#### See link Ch 11g

#### “Within months, some "helpful" person invested their time into generating a cracker tool. Publicizing the threat was a service to everyone, but I leave it as an exercise for readers to determine what satisfaction is obtained by the authors of tools that turn threat into reality and lay waste to millions of dollars of investments.”

#### --"Real 802.11 Security", William Arbaugh and Jon Edney, as quoted in link Ch 11g

# More Secure EAP Methods

## Extensible Authentication Protocol-Transport Layer Security (EAP-TLS)

### Secure but rarely used, because both client and server need certificates signed by a CA

## Protected EAP (PEAP) and Microsoft PEAP

### Very secure, only requires server to have a certificate signed by a CA

## See link Ch 11h

# 802.1X components

## Supplicant

### The user accessing a WLAN

## Authenticator

### The AP

## Authentication server

### Checks an account database to see if user’s credentials are acceptable

### May use RADIUS (Remote Access Dial-In User Service)

## See link Ch 11k

# Wired Equivalent Privacy (WEP)

## Part of the 802.11b standard

## Encrypts data on a wireless network

## WEP has many vulnerabilities

## To crack WEP, see links Ch 11l, 11m

# Wi-Fi Protected Access (WPA)

## Specified in the 802.11i standard

## Replaces WEP

## WPA improves encryption by using Temporal Key Integrity Protocol (TKIP)

# TKIP Enhancements

## Message Integrity Check (MIC)

### Prevent attacker from injecting forged packets

## Extended Initialization Vector (IV) with sequencing rules

### Prevent replays (attacker re-sending copied packets)

## Per-packet key mixing

### MAC addresses are used to create a key

### Each link uses a different key

## Rekeying mechanism

### Provides fresh keys

### Prevents attackers from reusing old keys

# WPA Adds 802.1x

## WPA also adds an authentication mechanism implementing 802.1X and EAP

### This was not available in WEP

# WPA Versions

## WPA and WPA-2

### WPA only implements part of 802.11i, using TKIP

#### Can run on older hardware

### WPA2 implements the full IEEE 802.11i security standard, using CCMP

#### More secure

#### Link Ch 11zj

## Pre-Shared Key v. 802.1x

### Both WPA and WPA-2 can run in either mode

### Pre-Shared Key uses a passphrase the user types into each device

#### Less secure because the user might choose a guessable passphrase, and all devices on the WLAN use the same passphrase

### 802.1x uses a server to manage keys

#### Each user has a different key

#### More secure

#### Link Ch 11zj

# Hacking WPA

### In 2008 and 2009, some new WPA attacks were developed, for its weakest form (TKIP)

#### These attacks allow injection of spoofed packets for limited periods of time

#### Up to 18 minutes

#### They don't find the WPA key, but they are a warning that it's time to go to WPA-2

Understanding Wardriving

## Hackers use wardriving

### Finding insecure access points

### Using a laptop or palmtop computer

## Wardriving is not illegal

### But using the resources of these networks is illegal

## Warflying

### Variant where an airplane is used instead of a car

# CCSF Wardriving

# How It Works

## An attacker or security tester simply drives around with the following equipment

### Laptop computer

### Wireless NIC

### An antenna

### Software that scans the area for SSIDs

## Not all wireless NICs are compatible with scanning programs

## Antenna prices vary depending on the quality and the range they can cover

## Scanning software can identify

### The company’s SSID

### The type of security enabled

### The signal strength

#### Indicating how close the AP is to the attacker

Wardriving Software

### For our wardrive, I recommend this software

### Windows XP: 	InSSIDer

### Vista or Win 7: 	VisStumbler

### Linux:		Kismet

### Mac OS X:	KisMac-ng

### NetStumbler is not useful for us, because it doesn't distinguish between WEP and WPA

# NetStumbler

## Shareware tool written for Windows that enables you to detect WLANs

### Supports 802.11a, 802.11b, and 802.11g standards

## NetStumbler was primarily designed to

### Verify your WLAN configuration

### Detect other wireless networks

### Detect unauthorized APs

## NetStumbler is capable of interface with a GPS

### Enabling a security tester or hacker to map out locations of all the WLANs the software detects

## NetStumbler logs the following information

### SSID

### MAC address and Manufacturer of the AP

### Channel

### Signal Strength

### Encryption

## Can detect APs within a 350-foot radius

### With a good antenna, they can locate APs a couple of miles away

# Kismet

## Another product for conducting wardriving attacks

## Runs on Linux, BSD, MAC OS X, and Linux PDAs

## Kismet is advertised also as a sniffer and IDS

### Kismet can sniff 802.11b, 802.11a, and 802.11g traffic

# Kismet features

### Ethereal- and Tcpdump-compatible data logging

### AirSnort compatible

### Network IP range detection

### Hidden network SSID detection

### Graphical mapping of networks

### Client-server architecture

### Manufacturer and model identification of APs and clients

### Detection of known default access point configurations

### XML output

### Supports more than 25 card types

Understanding Wireless Hacking

## Hacking a wireless network is not much different from hacking a wired LAN

## Techniques for hacking wireless networks

### Port scanning

### Enumeration

# Tools of the Trade

## Equipment

### Laptop computer

### A wireless NIC

### An antenna

### Sniffer software

# Aircrack-ng

# Popular WEP-cracking tool

## Included in BackTrack

Countermeasures for Wireless Attacks

## Anti-wardriving software makes it more difficult for attackers to discover your wireless LAN

### Honeypots

#### Servers with fake data to snare intruders

### Fakeap and Black Alchemy Fake AP

#### Software that makes fake Access Points

##### Link Ch 11s

## Use special paint to stop radio from escaping your building

## Allow only predetermined MAC addresses and IP addresses to have access to the wireless LAN

## Use an authentication server instead of relying on a wireless device to authenticate users

## Use an EAP authentication protocol

## If you use WEP, use 104-bit encryption rather than 40-bit encryption

### But just use WPA instead

Assign static IP addresses to wireless clients instead of using DHCP

DMZ

## Don’t broadcast the SSID

## Place the AP in the demilitarized zone (DMZ) (image from wikipedia)

Last modified 10-27-10