Determining If The System Is Alive

## Summary

### Ping Sweeps

#### Fping

#### Nmap

#### SuperScan

#### Ping Sweep from SolarWinds

#### Hping2

#### Icmpenum

### Countermeasures

### ICMP Queries

# Determining If The System Is Alive

## Network Ping Sweeps

### Ping is traditionally used to send ICMP ECHO (Type 8) packets to a target system

### Response is ICMP ECHO_REPLY (Type 0) indicating the target system is alive

# fping

## fping is a fast PING scanner, because it doesn't wait for a response from one system before moving on to the next one

### Available for Linux and Windows

#### Link Ch 2b for Windows version (slower than Linux version)

# Ping Sweep With Nmap

## Use the –sP option

# SuperScan

## Does PING scanning, using several types of ICMP packets

## Also does port scanning, banner grabbing, whois, and enumeration

# Superscan Enumeration

## To run SuperScan, you need Win 2000 or Win XP before SP 2

## Great tool

### Link Ch 2c

# ICMP Packet Types

## Message Type: 0 - Echo Reply

## Message Type: 3 - Destination Unreachable

## Message Type: 4 - Source Quench

## Message Type: 5 - Redirect

## Message Type: 8 - Echo

## Message Type: 11 - Time Exceeded

## Message Type: 12 - Parameter Problem

## Message Type: 13 - Timestamp

## Message Type: 14 - Timestamp Reply

## Message Type: 15 - Information Request

## Message Type: 16 - Information Reply

# Ping Sweep from SolarWinds

## Scans really fast, which can saturate a network

## Commercial tool, but there's a 30-day trial available

### Ch 2d

# icmpenum

## Unix utility that sends the traditional ICMP ECHO packets as well as

### ICMP TIME STAMP REQUEST and

### ICMP INFO requests

## Similar to SuperScan

# ICMP Blocking

## ICMP is often blocked these days

### Blocked by default in Win XP SP2, Win 2003 SP 1, and Vista

## If ICMP is blocked, use port scanning

### Slower than ping sweeping

#### SuperScan for Win 2000 or XP without SP2

#### Nmap for Linux, Unix, or Windows

#### Hping2 for Unix (can fragment packets)

# Nmap

## TCP Ping Scan uses TCP ACK packets instead of ICMP

## Zenmap GUI runs on Vista (as Administrator) – very pretty

## Use –PT 80 to get through many firewalls

### Link Ch 2i

# Other Ports to Use

## Email ports

### SMTP (25)

### POP (110)

### IMAP (143)

## AUTH (113)

### IDENT service – determines remote user of a network connection (link Ch 2g)

# Ping Sweeps Countermeasures

## Detecting Ping Sweeps

### Network-based Intrusion Detection Systems like Snort detect ping sweeps

### Ping scans will be in the host logs

### Firewalls can detect ping scans

# Ping Sweep Detection Tools

## For Unix

### Scanlogd, Courtney, Ippl, Protolog

For Windows

### Snort could be used (link Ch 2z9)

# Blocking ICMP

## Routers may require some ICMP packets, but not all types

## Safest procedure would be to allow ICMP only from your ISP, and only to public servers on your DMZ

# Other ICMP Threats

## ICMP can be used for a Denial of Service attack

## ICMP can be used as a covert channel with Loki

### Allowing unauthorized data transfer

### Such as control signals for a back-door trojan

### Links Ch 2l, Ch 2m

# ICMP Queries

## icmpquery uses ICMP type 13 (TIMESTAMP) to find the system time, which shows its timezone

## ICMP type 17 (ADDRESS MASK REQUEST) shows the subnet mask

### Link Ch 2n

Determining Which Services Are Running Or Listening

## Summary

### Port Scanning

### Scan Types

### Identifying TCP and UDP Services Running

### Windows-Based Port Scanners

### Port Scanning Breakdown

# Port Scan Types

## We covered these ones in CNIT 123

### TCP Connect scan

### TCP SYN scan

### TCP FIN scan

### TCP Xmas Tree scan (FIN, URG, and PUSH)

### TCP Null scan

### TCP ACK scan

### UDP scan

# TCP Header

## WINDOW indicates the amount of data that may be sent before an acknowledgement is required

# TCP Window Scan

## Sends ACK packets

### Both open and closed ports reply with RST packets

### But on some operating systems, the  WINDOW size in the TCP header is non-zero for open ports, because the listening service does sometimes send data

### Link Ch 2x

# RPC Scan

## SunRPC (Sun Remote Procedure Call) is a common UNIX protocol used to implement many services including NFS (Network File System)

## The RPC scan works on Unix systems, including Solaris

## Enumerates RPC services, which are rich in exploitable security holes

### See link Ch 2y

# Nmap

## Interesting options

### -f 	fragments packets

### -D	Launches decoy scans for concealment

### -I	IDENT Scan – finds owners of processes (on Unix systems)

### -b	FTP Bounce (see next slide)

# FTP Bounce

## Old FTP servers allowed a request for a file transfer to a third IP address

## This could be used to send email or other data to the third computer from the FTP server

# Nmap Book Out

## Available from Amazon

Highly recommended

# Older Port Scanning Tools

## strobe – fast TCP scanner

## udp_scan – UDP scanner

## netcat – can do port scanning

# Amap (not in book)

## Application scanner – finds applications even if they are running on unusual ports

## Steps to use amap:

### Create a folder C:\amap

### Download amap from link Ch 2h & extract it there

### Run an nmap scan with this option, to save the output file:

#### –oM c:\amap\filename.nmap

### At Command Prompt in C:\amap

#### amap –bqv –i hackmebank.nmap

# Windows-Based Port Scanners

## SuperScan

### Four different ICMP host-discovery techniques

### Accurate UDP scan sending "nudge strings"

### Banner grabbing

### Many other tools

## Nmap with the Zenmap GUI

### Powerful, runs on Vista

# Popular Scanning Tools and Features

## Add Nmap with Zenmap in the Windows group

# Port Scanning Countermeasures

## Snort (http://www.snort.org) is a great free IDS (Intrusion Detection System)

### [**] spp_portscan: PORTSCAN DETECTED from 192.168.1.10 [**] 05/22-18:48:53.681227 [**] spp_portscan: portscan status from 192.168.1.10: 4 connections across 1 hosts: TCP(0), UDP(4) [**] 05/22-18:49:14.180505 [**] spp_portscan: End of portscan from 192.168.1.10 [**] 05/22-18:49:34.180236

# Other Detection Tools

# Scanlogd

## Detects TCP port scans on Unix

# Firewalls can detect port scans

## Use threshold logging to limit the volume of email alerts sent by your firewall

That groups similar alerts into a single email

# Preventing Port Scans

## You can't stop the scans from coming in, but you can mimimize your attack surface

## Disable unnecessary services

Detecting the Operating System

## Banner-Grabbing

### Many services announce what they are in response to requests

### Banner grabbers just collect those banners

### But they could be spoofed

# Active Stack Fingerprinting

## Details of the TCP Packets are used to identify the operating system

## Nmap does this, using these probes:

### FIN probe

### Bogus Flag probe

### Initial Sequence Number (ISN) sampling

### "Don't fragment bit" monitoring

### TCP initial window size

#### And many others

# Operating System Detection Countermeasures

## IDS can detect operating system detection scans

## Hacking the OS to change its TCP stack is dangerous, and not recommended

## Best policy: Accept that your firewalls and proxy servers will be scanned and fingerprinted, and harden them against attackers who know the OS

# Passive Operating System Identification

## Sniff traffic and guess the OS from that

## Examine these features

### TTL (time-to-live)

### Window size

### DF (Don't fragment bit)

## siphon was the first tool to do this, it's out of date

## p0f is a newer one (link Ch 2z6)

# p0f on Vista

## Run p0f in a Command Prompt Window

## Open a Web page

## It fingerprints any OS it can see on the LAN

# Automated Discovery Tool: Cheops-ng

## Combines Ping, Traceroute, Port Scans, and OS Detection to draw a network map

### Link Ch 2z7

# Vista's "Network Map" is worth a look

## Last modified 1-23-09