# Analog Dial-up Hacking

## The public switched telephone network (PSTN) is still a popular network connection

## Most large companies are more vulnerable through poorly inventoried modem lines than via firewall-protected Internet gateways

War-Dialing

## War-dialers automate the process: footprint, scan, enumerate, exploit

### Programmatically dial large banks of phone numbers

### Log valid data connections (called carriers )

### Attempt to identify the system on the other end of the phone line

### Optionally attempt a log on by guessing common usernames and passphrases

# Preparing to Dial Up

# Phone Number Footprinting

## Phone directories

## War-dial a whole exchange

### CCSF's exchange is 415-239-XXXX

### Using 4 modems, a four-digit exchange can be war-dialed in a day or two

### Of course, don't war-dial anyone without permission

# Other Ways to Find Phone Numbers

## Social engineering phone company reps

## Corporate websites

## Whois at http://www.arin.net will provide primary administrative, technical, and billing contact information

# Leaks Countermeasures

## Limit publication of phone numbers

## Work closely with your telecommunications provider to ensure

### Proper numbers are being published

### Establish a list of valid personnel authorized to perform account management

### Require a password to make any inquiries about an account

## Staff should be suspicious of unidentified callers requesting information

# War-Dialing

## Freeware tools run in DOS

## A basic PC with two standard COM ports and a serial card to add two more is recommended

## A multiport card, sometimes referred to as a "digiboard" card, can allow for four or eight modems on one system

## Standard time-out is 46 to 60 seconds

### 10,000 numbers will take 7 days of 24-hour calling

### Legitimate pen-tests typically only take place on evenings and weekends

### Several modems make it a lot faster

# Legal Issues

## In some localities, it is illegal to dial large quantities of numbers in sequence, and local phone companies will take a very dim view of this activity, if their equipment allows it at all

## Randomizing numbers won't make it legal

## Obtain written legal permission

# ToneLoc

## Free DOS tool

# THC-Scan

## Another free DOS tool

# PhoneSweep

## Commercial product, costs $1200 or more (link Ch 601)

# Things to Find by Wardialing

## Remote control services like pcAnywhere

## Dial-up connections to servers

## PBX and voicemail systems

### Much more info at link Ch 605

# pcAnywhere

## Remote control via modem

## Still popular, often used insecurely

### Links Ch 602-604

Brute-Force Scripting

# War-Dialing Penetration Domains

# Default Password List

## Link Ch 606

# ProComm Plus and ASPECT

## ProComm Plus emulates a terminal

## ASPECT is the scripting language

## Old tools intended for Windows 2000 and earlier versions

## Very useful, but no longer supported by Symantec

### Links Ch 609 & 610

# Dial-Up Security Measures

## Inventory existing dial-up lines

### By war-dialing every six months

## Consolidate all dial-up connectivity

### Put it in the DMZ

### Add intrusion detection and firewall technology

### Limit connections to trusted subnets

### Monitor connections

## Make analog lines harder to find

### Use a different phone number range

### Don't publish the numbers

## Ensure physical security of telecommunications equipment closets

## Regularly monitor existing log features within your dial-up software

### Look for failed login attempts, late-night activity, and unusual usage patterns

### Use Caller ID to store all incoming phone numbers

## Disable any banner information presented upon connect

## Require two-factor authentication systems for all remote access

### Link Ch 607

## Require dial-back authentication

### The remote system hangs up and calls back to a predetermined number

## Make sure the help desk personnel are careful giving out or resetting remote access credentials

## Centralize the provisioning of dial-up connectivity within one security-aware department in your organization

### Make it very difficult to get a POTS (plain old telephone service)  line

# Dial-up Security is Important

## Experience shows that many companies have glaring, trivially navigated POTS dial-up holes that lead right to the heart of their IT infrastructure

## Going to war with your modems may be the single most important step toward improving the security of your network

PBX Hacking

# PBX (Private Branch Exchange)

## A PBX connects the internal telephones of a company

## That saves money on intra-company calls

### Link Ch 608

# Dial-up Access to a PBX

## Consoles used to be hard-wired to a PBX

## PBX vendors usually tell their customers that they need dial-in access for external support

### But it is often done insecurely, leaving a modem always on and connected to the PBX

### It should be turned off except when needed

# Voicemail Hacking

## Voicemail is often important, confidential, and poorly secured

## Top executives often neglect to pick an unique code for voicemail

## Brute-force ASPECT scripts work well

Example Script

# Common Voicemail Passwords

## People often use simple geometrical patterns on the keypads

Virtual Private Network (VPN) Hacking

# Virtual Private Network (VPN)

## A VPN connects two computers securely over an insecure network (usually the Internet), using tunneling

# Tunneling

## An Ethernet frame is encapsulated in an IP packet, so it can be sent over the Internet

### It can be done with other protocols too

## Usually the frame is also encrypted, so that only the intended recipient can read it

## The end result is like you used a long cable to connect the two computers

# Cost Savings

## You could use a T-1 line or a POTS phone call with a modem, to make a secure connection between two computers

## But a VPN is much cheaper, requiring only an Internet connection at each end

# VPN Standards

## The modern way

### IP Security (IPSec) and the Layer 2 Tunneling Protocol (L2TP)

## Older techniques

### Point-to-Point Tunneling Protocol (PPTP)

#### Microsoft proprietary

### Layer 2 Forwarding (L2F)

#### An obsolete Cisco protocol

## For more details, see link Ch 611

# Breaking Microsoft PPTP

## Microsoft's secure authentication protocol, MS-CHAP, uses LM Hashes

### Easily cracked with Ophcrack

## Session keys and encryption are poorly implemented and vulnerable to attacks

## The control channel is open to snooping and denial of service

## PPTP clients could act as a backdoor into the network

### See links Ch 612 & 613

# Fixing PPTP

## Microsoft patched PPTP in Win NT Service Pack 4 by using MS-CHAPv2

### And it's really much better (link Ch 614)

## Win 2000 and later also offer IPSec and L2TP, which is safer

### "In our opinion, IPSec is too complex to be secure" -- Schneier and Ferguson (link Ch 615)

### But it's the best IP security available now

Voice Over IP (VoIP) Attacks

# Voice over IP (VoIP)

## Voice on an IP Network

## Most VoIP solutions rely on multiple protocols, at least one for signaling and one for transport of the encoded voice traffic

## The two most common signaling protocols are H.323 and Session Initiation Protocol (SIP)

### Their role is to manage call setup, modification, and closing

# H.323

## H.323 is a suite of protocols

### Defined by the International Telecommunication Union (ITU

### The deployed base is larger than SIP

### Encoding is ASN.1 – different than text, a bit like C++ Data Structures (link Ch 618)

### Designed to make integration with the public switched telephone network (PSTN) easier

# Session Initiation Protocol (SIP)

## The Internet Engineering Task Force (IETF) protocol

## People are migrating from H.323 to SIP

## Used to signal voice traffic, and also other data like instant messaging (IM)

## Similar to the HTTP protocol

## The encoding is text (UTF8)

## SIP uses port 5060 (TCP/UDP) for communication

# Real-time Transport Protocol (RTP)

## Transports the encoded voice traffic

## Control channel for RTP is provided by the Real-time Control Protocol (RTCP)

## Consists mainly of quality of service (QoS) information (delay, packet loss, jitter, and so on)

### Timing is more critical for VoIP than other IP traffic

# Most Common VoIP Attacks

## Denial of Service

### Send a lot of SIP INVITE packets, initiating calls

### Flood a phone with unwanted IP traffic

## Spoofing the CLID (Caller ID)

### Swatting is a popular and dangerous attack, spoofing caller ID and calling police (link Ch 619)

## Injecting data into an established call

## Altering the phone's configuration

### Connect to the phone via Telnet or HTTP

### Sometimes no password is needed

### Or upload malicious code with your own DHCP and TFTP servers

#### When a phone boots, it can upload updated firmware with TFTP

## Attacking though services linked to VoIP

### Advanced voicemail

### Instant messaging

### Calendar services

### User management

## Attacks may use XSS (cross-site scripting), client-side JavaScript alteration, SQL injection, and so on

## Accessing repository of recorded calls

## Making free calls through a company's VoIP-to-PSTN gateway

# Interception Attack

## Sniff the IP Packets

### With ARP poisoning

## Attacker is set to route traffic, but not decrement the TTL

# Captured RTP Traffic

## It's compressed with a codec

## Common codecs

### G.711 (uses up a lot of bandwidth)

### G.729 (uses less bandwidth)

# VOMIT

## vomit - voice over misconfigured internet telephones

### Converts G.711 to WAV

### It works because many IP phones don't or can't encrypt traffic

### Link Ch 620

## Scapy is an even better tool, plays traffic from eth0 right out the speakers

### Link Ch 621

# Interception Countermeasures

## Turn on the security features available for your phones, such as encryption

## They are often left turned off, to get higher quality or just through laziness

Summary

## Remote access is dangerous!

## Enforce strong password policy for remote users

### Consider two-factor authentication

## Don't neglect dial-up security—perform wardialing

### Find and remove unauthorized remote control software (such as pcAnywhere)

## Educate support personnel to require some other form of identification, such as a personnel number, to receive any support for remote access issues

## VPNs are vulnerable

### Don't believe vendor security claims easily

#### Last modified 3-8-08