Discovery

# Detecting Network Devices

## Port Scanning

### traceroute, netcat, nmap, and SuperScan

## dig

### An undated replacement for nslookup in Unix/Linux

### If it's not installed by default in your Ubuntu (or andLinux), use

#### apt-get install dnsutils

# Finding Mail Exchanges with nslookup

# Finding Mail Exchanges with dig

# Types of DNS Records

## A – maps a hostname to an IPv4 address

## AAAA - maps a hostname to an IPv6 address

## CNAME - Canonical name - an alias of one name to another

## MX - mail exchange record

## PTR - maps an IPv4 address to the canonical name for that host (allows reverse DNS lookups)

# Types of DNS Records

## SOA - start of authority record – the authoritative DNS server for a domain

## SRV - a generalized service location record, used for VoIP SIP servers

### See link Ch 705

## For more about DNS Records, see link Ch 704 (Wikipedia)

# dig Countermeasures

## Secure your DNS infrastructure

## Block or restrict zone transfers

## Leave hosts out of your DNS records unless you want direct traffic to them from the Internet

# traceroute

## Tracert in Windows uses ICMP packets

## Traceroute in Unix/Linux uses UDP packets

### The packets have low TTLs, starting with 1

### When the packet traverses a router, its TTL is decreased by 1

### If the TTL ever hits zero, the packet is dropped

### A notification is sent back to the originating source host in the form of an ICMP error packet

# Finding Routing Devices at CCSF

## Hops 10 and 11 both appear to be routing devices on campus

# traceroute Countermeasures

## Stop your routers from responding to TTL-exceeded packets

## Deny all traffic specifically addressed to a router

## Permit ICMP only from the LAN, not from the Internet

Autonomous System Lookup

# Autonomous Systems

## Autonomous System (AS)

### A collection of gateways (routers) that controlled by one organization

## Autonomous System Number (ASN)

### a numerical identifier for networks participating in Border Gateway Protocol (BGP)

## Border Gateway Protocol (BGP)

### A protocol used to advertise routes worldwide

# traceroute with ASN Information

## Run traceroute from a Cisco router participating in BGP to see the ASNs

## Hop 8 is a T-1; hops 4-9 all same company

# Demo

## Public Looking Glass sites let you test routing from various servers

See Links 724-727

# show ip bgp

## From a Cisco router, we can find the other possible network paths

Public Newsgroups

# Careless Postings

## Careless admins may announce network vulnerabilities on newsgroups

## Countermeasures:

### Be wary of what you say and where you say it

Service Detection

# Port Scanning

## Common ports are known for each device

# Nmap Results

## Nmap also does OS detection, as we discussed in a previous chapter

# Familiar Prompts

## If Telnet is enabled on a Cisco router, you will see this prompt

## A Cisco router configured for SSH still shows a banner to Telnet

# Service Detection Countermeasures

## Deny all unwanted traffic at network borders

## PortSentry will detect port scans and block traffic from that IP

### But PortSentry itself could be used to perform a DoS attack if you don't check for spoofed packets

Network Vulnerability

# The OSI Model

# Data Units

## APDU - Application Protocol Data Unit

## PPDU - Presentation Protocol Data Unit

## SPDU - Session Protocol Data Unit

## TPDU - Transport Protocol Data Unit

### But our focus is on the first 3 layers

# OSI Layer 1: Physical

## Physical media that carry data: usually copper or fiber optics

## Traffic can be intercepted with a physical man-in-the-middle attack

## The figure to the right shows a T1 man-in-the-middle attack (copper lines)

# Fiber Optic Physical MITM Attack

## See link Ch 709

# OSI Layer 2: Data Link

## Layer 2 is the layer where the electrical impulses from Layer 1 have MAC addresses associated with them

## Early Ethernet sent traffic to every node connected to the hub or backbone

## Modern switched networks don't do that

# Unswitched Ethernet

## Most wired networks use switches instead of hubs now

## Wi-Fi networks still work this way

# Switched Ethernet

## Switches make sniffing harder

## They also make networks faster

# Switch Sniffing

## Some switches allow an administrator to monitor all traffic on a special port

## ARP cache poisoning is the most common way to sniff traffic on a switch

# ARP Poisoning Countermeasures

## Use static ARP routes, with manually entered MAC addresses

### This prevents abuse of ARP redirection, but it is a LOT of tedious work

### Every time you change a NIC, you need to manually add the new MAC address to the tables

# Broadcast Sniffing

## Connect to a port

## It doesn't matter what your IP address is

## Just sniff for broadcast packets

### Using Wireshark or any other sniffer

# DHCP Packets

## Give out IP addresses, and may also contain brand of router

# DEMO:

# Start Wireshark

# Open Command Prompt

# ipconfig /release

# ipconfig /renew

# ARP Packets

## These give you IP addresses and MAC addresses

# WINS Packets

## Note Computer Description field at the end "Accounting"

# Broadcast Sniffing Countermeasures

## To limit broadcasts, split your network into different segments

## Use VLANS – Virtual Local Area Networks

### Switches add a VLAN tag to each frame

### Broadcasts only reach machines on the same VLAN

### Link Ch 710

# VLANs

## Virtual LANs are logically separate LANs on the same physical medium

## Each VLAN has its own VLAN Number

## 802.1q is the standard for VLAN Tagging

# VLAN Tagging

## Links Ch 712, 713

# Port-Based VLANs

## Each port on the switch is assigned to a VLAN by the administrator

## The clients send in normal Ethernet frames, and the VLAN tag is added by the switch

## When tagged frames are received, the switch removes the VLAN tags

## This is the most secure method

# Native VLANs

## Suppose you want to use a single network link to carry traffic from multiple VLANs?

### For example, a long line connecting two buildings

## One VLAN can be defined as the "Native VLAN" or "Management VLAN"

### Frames belonging to the "Native VLAN" are not modified—no VLAN header is added to them, or removed

# VLAN Jumping

## This allows an attacker to craft a frame with two VLAN tags

### The first switch removes one tag

### The second switch sees the extra tag, so the frame hops from one VLAN to another

# VLAN Jumping Countermeasures

## Don't trust VLANS to enforce network security boundaries

## Restrict access to the native VLAN port (VLAN ID 1)

# We'll skip these sections

## Internetwork Routing Protocol Attack Suite (IRPAS) and Cisco Discovery Protocol (CDP)

## Spanning Tree Protocol (STP) Attacks

## VLAN Trunking Protocol (VTP) Attacks

# OSI Layer 3

## Internet Protocol Version 4 (IPv4)

### Has no built-in security measures

## TCP Sequence Numbers

### Example: tcpdump showing a Telnet connection

### S = SYN, A = ACK; note increasing Sequence and Acknowledgement numbers; the ACK number is one more than the corresponding SYN number

# Demonstration of Sequence Numbers

## Use Ubuntu

## In one Terminal window:

### sudo apt-get install tcpdump

### sudo tcpdump –tnlS | tee capture

#### (no timestamps, numerical IP addresses, line buffered, absolute sequence numbers)

## In another Terminal window:

### telnet 147.144.1.2

## In first Terminal window:

### pico capture

# tcpdump Results

## This has been cleaned up somewhat

## Note increasing Sequence and Acknowledgement numbers

## The ACK number is one more than the corresponding SYN number

# Attacks Using Sequence Numbers

## Non-Blind Spoofing

### Attacker is on the target's LAN

### Sequence and acknowledgement numbers can be sniffed

### Session can be hijacked with a simple man-in-the-middle attack, such as ARP cache poisoning

## Blind Spoofing

### Attacker not on the target's LAN

### Attacker sends several packets to the target machine in order to sample sequence numbers

### If the target machine's OS uses easily-predicted Initial Sequence Numbers, the attacker can forge packets and hijack a later session

# Vulnerabilities to ISN Prediction

## Windows NT4 SP3	Attack feasibility: 97.00%

## Windows 98 SE	Attack feasibility: 100.00%

## Windows 95	Attack feasibility: 100.00%

## AIX 4.3 	Attack feasibility: 100%

## HPUX11	Attack feasibility: 100%

## Solaris 7	Attack feasability: 66.00%

## MacOS 9	Attack feasability: 89.00%

### See links Ch 718, 719, 720

# IP Version 6 (IPv6)

## Long addresses like this

### ABCD:EF01:2345:6789:0123:4567:8FF1:2345

## Native security

## IPSec encryption framework has two modes:

### Tunnel mode encrypts whole packet (most secure)

### Transport mode just encrypts the data, not the IP header

## Both modes are much more secure than IPv4

# Sniffing Attacks

## Steal passwords or hijack sessions

## Generally require access to the LAN

## Tools: Wireshark, tcpdump, Cain, ettercap, hamster, ferret

## Older tools: dsniff, webmitm, mail snarf, webspy

# Sniffing Countermeasures

## Segment network with switches, routers, or VLANS

## Use encrypted protocols like SSL/TLS

# Misconfigurations

## Read/Write MIB

### Network devices that allow anyone with the community name to download the router or switch's configuration file via TFTP

## To test, go to link Ch 722, open support.txt, look for OLDCISCO-SYS-MIB

## If it's listed, you are probably vulnerable.

### C2610 is vulnerable, but not C2950

# Read/Write MIB Countermeasures for Cisco

## Restrict the use of SNMP to approved hosts or networks

## Use Read-Only SNMP

## Turn off SNMP altogether

# Cisco Weak Encryption

## Cisco passwords are stored with a weak, easily broken encryption method

## Cisco admits this, and does not see it as a problem or have plans to change it

## "Customer demand for stronger reversible password encryption has been small"

### Link Ch 723

# Cisco Password Decryption Countermeasures

## The "enable secret" command will hash passwords with MD5, which is much stronger

### But it does not hash all passwords

# TFTP Downloads

## Almost all routers support the use of the Trivial File Transfer Protocol (TFTP)

## This is a UDP-based file-transfer mechanism used for backing up and restoring configuration files, and it runs on UDP port 69

## You can turn TFTP off on Cisco routers if you want to

# We'll skip this section

## Route Protocol Hacking

## Last modified 12-30-08