Getting in the Door

# Normal Key

## Link Ch 920

### (At bottom of my CNIT 124 page)

# Bump Key

## Every key pin falls to its lowest point

## The key is hit with a screwdriver  to create mechanical shocks

## The key pins move up and briefly pass through the shear line

## The lock can be opened at the instant the key pins align on the shear line

# Results of Bump Key Use

## A experienced bumper can open the lock as quickly as a person with the correct key

## Bumping does not damage the lock

### Unless it is done many times, or clumsily

## Bumping leaves no evidence behind

## Even Medeco locks used in the White House can be bumped

### Link Ch 921

# Bump Key Countermeasures

## Some locks (like Medeco) are designed to make bumping difficult

## They use a sidebar  and angled pins to make  normal picking and bumping ineffective

### Don't trust their claims too far

## Don't rely solely on locks: use two-factor authentication

### PIN keypad

### Fingerprint

### Security guard

### etc.

# Cloning Access Cards

## Two Varieties

### Magnetic stripe cards

### RFID (Radio Frequency Identification) cards

Magstripe Cards

## ISO Standards specify three tracks of data

## There are various standards, but usually no encryption is used

### Link Ch 922

# Magstripe Card Reader/Writer

## USB connector

## About $35o

### Link Ch 923

# Magnetic-Stripe Card Explorer

## Software

## Link Ch924 for more images

# Hacking RFID Cards

## RFID cards use radio signals instead of  magnetism

## Now required in passports

## Data can be read at a distance, and is usually unencrypted

# Cloning Passports

## $250 in equipment

## Can steal passport data from a moving car

## Link Ch 925

# MiFare Classic

## Mifare is most widely deployed brand of secure RFID chips

## Dutch researchers found weaknesses have been found in its proprietary encryption in 2008

### Don't roll your own crypto!

### Links Ch 926 & 927

# Boston Subway Hack

## The Massachusetts Bay Transportation Authority claims that they added proprietary encryption to make their MiFare Classic cards secure

## But Ron Rivest's students from MIT hacked into it anyway

## Link Ch 928

# Countermeasures for Cloning Access Cards

## We are at the mercy of card vendors

## Newer  cards have a full challenge-response algorithm

### Resists cloning and replay attacks

### Some use open algorithms, others are proprietary

Hacking Devices

# ATA Interfaces for Hard Drives

## Two kinds of ATA (AT Attachment ) interfaces are used

## PATA (Parallel ATA)

### IDE is now called PATA

## SATA (Serial ATA)

### Newer and faster than PATA

# ATA Security

## Requires a password to access the hard disk

## Virtually every hard drive made since 2000 has this feature

## It is part of the ATA specification, and thus not specific to any brand or device.

## Does not encrypt the disk, but prevents access

# ATA Password Virus

## ATA Security is used on Microsoft Xbox hard drives and laptops

## BUT desktop machines' BIOS is often unaware of ATA security

## An attacker could turn on ATA security, and effectively  destroy a hard drive, or hold it for ransom

### The machine won't boot, and no BIOS command can help

### This is only a theoretical attack at the moment

# Bypassing ATA Passwords

## Hot Swap

### With an unlocked drive plugged in, enter the BIOS and navigate to the menu that allows you to set a HDD Password

### Plug in the locked drive and reset the password

## Use factory default master password

### Not easy to find

### Some examples given in 2600 magazine vol 26 #1

# Bypassing ATA Passwords

## Vogon Password Cracker POD

### Changes the password from a simple GUI

### Allows law enforcement to image the drive, then restore the original password, so the owner never knows anything has happened

### Works by accessing the drive service area

#### A special area on a disk used for firmware, geometry information, etc.

#### Inaccessible to the user

# ATA Security Sources

## Hacking Exposed 6th Ed.

## Links Ch 929 - 931

## 2600  Magazine

# ATA Password Bypass Countermeasure

## Don't trust ATA Security

## Encrypt the drive with Bitlocker, TrueCrypt, PGP, etc.

U3 Drives

# U3: Software on a Flash Drive

## Carry your data and your applications in your pocket!

## It’s like a tiny laptop!

# U3 Launchpad

## Just plug it in, and the Launchpad appears

## Run your applications on anyone’s machine

## Take all data away with you

# How U3 Works

## The U3 drive appears as two devices in My Computer

### A “Removable Disk”

### A hidden CD drive named “U3”

## The CD contains software that automatically runs on computers that have Autorun enabled

### For more details, see http://www.everythingusb.com/u3.html

# Hak9’s PocketKnife

# Software On The Disk Partition

## PocketKnife is a suite of powerful hacking tools that lives on the disk partition of the U3 drive

## Just like other applications

# U3 PocketKnife

## Steal  passwords

## Product keys

## Steal files

## Kill antivirus software

## Turn off theFirewall

## And more…

## For details see  http://wapurl.co.uk/?719WZ2T

# Custom Launchpad

# Customizing U3

## You can create a custom file to be executed when a U3 drive is plugged in

# Automatically Run PocketKnife

## The custom U3 launcher runs PocketKnife

## So all those things are stolen and put on the flash drive

# Defense

# Military Bans USB Thumb Drives

# Immediate Risk Reduction

## Block all USB devices in Group Policy

## Disable AutoRun

## Glue USB ports shut

# Better Solution: IEEE 1667

## Standard Protocol for Authentication in Host Attachments of Transient Storage Devices

## USB devices can be signed and authenticates, so only authorized devices are allowed

## Will be implemented in Windows 7

### See http://wapurl.co.uk/?QXASJBK

Default Configurations (Pre-0wned)

# ASUS Eee PC Rooted Out of the Box

## The Eee PC 701 shipped with Xandros Linux

## The Samba file-sharing service was on by default

## It was a vulnerable version, easily rooted by Metasploit

## Easy to learn, Easy to work, Easy to root

### Link Ch 933

# Default Passwords

## Many devices ship with default passwords that are often left unchanged

### Especially routers (see link Ch 934)

# ATM Passwords

## In 2008, these men used default passwords to reprogram ATM machines to hand out $20 bills like they were $1 bills

### Link Ch 936

# Bluetooth Attacks

## Bluetooth supports encryption, but it's off by default, and the password is 0000 by default

### Link Ch 935

Reverse Engineering Hardware

## We will skip this section

#### Last modified 4-17-09