Wardialing

# Hardware

## If using dial-up modems, use COM ports

## Multiple lines make it easier to dial ranges of numbers more quickly

### One call per minute per modem

### A 10,000-number range takes a whole week of 24-hour dialing for a single modem

# Legal Issues

## There are a lot of laws restricting telephone hacking

## Get written permission, specifying the phone number ranges

## WarVOX records calls; may be illegal to use

# Software

## Old school

### ToneLoc ,THC-Scan, TeleSweep,  PhoneSweep

## HD Moore's New Tool

### WarVOX

## Uses VoIP

## Captures audio

## Dials ranges of numbers

## Records 53 seconds of audio from each one

## Captured audio analyzed with Digital Signal Processing – Fast Fourier Transform to create a signature

# Dial-Up Security Measures

## Inventory dial-up lines

## Consolidate all dial-up connectivity to a central modem bank in the DMZ, with an IDS, firewall, and logging

## Make analog lines harder to find

## Physically secure telecommunications closets

## Monitor dial-up logs

## Don't use a company banner; just a warning to unauthorized users

## Require multifactor authentication

## Require dial-back authentication

## Train help desk to be cautious giving out or resetting access credentials

## Centralize provisioning of dial-up connectivity

## Establish firm policies for dial-up access

## Return to step 1: wardial your network every six months

# Voicemail Hacking

## Brute-force attack tools are available

## But usually users leave the password at an obvious default

Virtual Private Network (VPN) Hacking

# Virtual Private Network (VPN)

## A VPN connects two computers securely over an insecure network (usually the Internet), using tunneling

# Tunneling

## An Ethernet frame is encapsulated in an IP packet, so it can be sent over the Internet

### It can be done with other protocols too

## Usually the frame is also encrypted, so that only the intended recipient can read it

## The end result is like you used a long cable to connect the two computers

# Cost Savings

## You could use a T-1 line or a POTS phone call with a modem, to make a secure connection between two computers

## But a VPN is much cheaper, requiring only an Internet connection at each end

# Two Common VPN Standards

## IPsec/L2TP (IP Security with Layer 2 Tunneling Protocol)

### Most secure and modern method

## PPTP (Point-to-Point Tunneling Protocol)

### Microsoft proprietary

# PPTP is Less Secure than IPsec/L2TP

## Links Ch 7e, 7f

# Site-to-Site VPN

## VPN Concentrators also called VPN  Gateways

## Cisco VPN Concentrators allow IPsec to be used without L2TP

## No client interaction required

## No login or credentials; to users, it's just another network link

# Client-to-Site VPN

## Remote user needs a software VPN client

### Cisco VPN "thick client"

### Web browser for SSL VPNs

## Two modes

### All traffic from client system goes through VPN

### "Split Tunnel" – Internet traffic does not pass through VPN

### Split Tunnels bridge corporate network to Internet and should be avoided

# Authentication and Tunnel Establishment in IPsec

## IKE (Internet Key Exchange) Phase 1

### Mutual authentication (both client & server)

### Main mode

#### Three separate 2-way handshakes

### Aggressive mode

#### Only three messages

#### Faster but less secure

# Authentication and Tunnel Establishment in IPsec

## IKE (Internet Key Exchange) Phase 2

### Establishing the IPsec tunnel

# Google Hacking for VPN

## Search for filetype:pcf

## Stored profile settings for the Cisco  VPN client

## You get encrypted passwords in this file

### I truncated the hash in this example

# Cracking VPN Password with Cain

## It cracked instantly for me

### Password removed from figure

## It’s obfuscated, not encrypted

### Link Ch 625

# Probing IPsec VPN Servers

## UDP port 500

## Tools

### Nmap

#### Basic detection

### ike-scan

#### Fingerprinting

### IKEProber

#### Older tool

#### Allows attacker to create IKE initiator packets

# Attacking IKE Aggressive Mode

## IKEProbe can identify whether a VPN server is in Aggressive Mode

## IKECrack can capture authentication messages and perform an offline brute force attack

## Cain can also do this

Hacking Citrix VPNs

# Citrix is Popular

## 100% of Fortune 100 companies

## Citrix Access Gateway

# Common Citrix Deployments

## Remote Desktop

### Whole computer desktop accessed remotely

## Commercial Off-The-Shelf (COTS) application

### Often Microsoft Office

## Custom application

### Often too insecure to expose to the Internet

# Kiosk Mode

## Limited access to desktop

## Only one application window displayed

## Microsoft calls this "kiosk-mode"

## Intended to prevent launching arbitrary code

# Help

## Windows OS Help easily gives you a shell

### Menu bar

### F1 key

### Logo+F1

# Microsoft Office

## Ways to spawn a shell

### Help

### Printing

### Hyperlinks

### Saving

### Visual Basic for Applications (VBA)

# Using VBA in MS Office to Spawn a Shell

## Alt+F11 opens a VBA editor

## Right-click in left pane, Insert, Module

## Enter this script

### Sub getCMD()

### Shell "cmd.exe /c cmd.exe"

### End Sub

## Press F5 to run it

# Internet Explorer Shells

## Help

## Print

## Internet access

## Text editors

## Save

## Local file exploration

### Enter a local path in the URL like

### C:\Windows\System32\cmd.exe

# Microsoft Games and Calculator

## Help

## "About Calculator"

# Task Manager

## Windows shortcut: Ctrl+Shift+Esc

## Citrix shortcut: Ctrl+F3 or Ctrl+F1

### File, "New Task (Run…)"

# Printing

## "Find Printer", then navigate to CMD

# Hyperlinks

## Just type into the application

### file:///c:/windows/system32/cmd.exe

### Enter, Shift+Click to launch CMD

# Internet Access

## Post cmd.exe on a Web site

## Download & run it

## Use SET to post a malicious Java applet & run it

# EULAs/Text Editors

## If EULA is spawned in Notpad, Wordpad, or a similar text editor

### Help

### Print

### Click a link

### Save

# Save As

## Navigate to the binary

## Create a shortcut, Web shortcut, VBS file, or WSF file

# Citrix Hacking Countermeasures

## Place Citrix instances in a segmented VPN

## Multifactor authentication

Voice Over IP (VoIP) Attacks

# Voice over IP (VoIP)

## Voice on an IP Network

## Most VoIP solutions rely on multiple protocols, at least one for signaling and one for transport of the encoded voice traffic

## The two most common signaling protocols are 
H.323 and Session Initiation Protocol (SIP)

### Their role is to manage call setup, modification, and closing

# H.323

## H.323 is a suite of protocols

### Defined by the International Telecommunication Union (ITU

### The deployed base is larger than SIP

### Encoding is ASN.1 – different than text, a bit like C++ Data Structures (link Ch 618)

### Designed to make integration with the public switched telephone network (PSTN) easier

# Session Initiation Protocol (SIP)

## The Internet Engineering Task Force (IETF) protocol

## People are migrating from H.323 to SIP

## Used to signal voice traffic, and also other data like instant messaging (IM)

## Similar to the HTTP protocol

## The encoding is text (UTF8)

## SIP uses port 5060 (TCP/UDP) for communication

# SIP Methods

## INVITE		Start a new conversation

## ACK		Acknowledges an INVITE

## BYE		Terminate session

## CANCEL	Cancel pending requests

## OPTIONS	Identify server capabilities

## REGISTER	SIP location registration

# SIP Responses

## SIP 1xx	Informational response messages

## SIP 2xx	Successful response messages

## SIP 3xx	Redirection

## SIP 4xx	Client request failure

# Real-time Transport Protocol (RTP)

## Transports the encoded voice traffic

## Control channel for RTP is provided by the Real-time Control Protocol (RTCP)

## Consists mainly of quality of service (QoS) information (delay, packet loss, jitter, and so on)

### Timing is more critical for VoIP than other IP traffic

# SIPVicious

## Link Ch 7c

# Pillaging TFTP

## SIP phones load configuration settings from TFTP servers on boot-up

### Files contain usernames, passwords, etc.

### Security through obscurity: filenames are "secret"

## Scan for UDP port 69 to find TFTP server

## Brute force the filenames

# Pillaging TFTP Countermeasures

## Network-layer access restrictions

### Only allow known VoIP phones to access the TFTP server

## Disable access to the settings menu on IP phones

## Disable Web server on IP phones

## Use signed configuration files to prevent tampering

# Enumerating VoIP Users

## Wardialing works, of course

## But there are other techniques for VoIP, specific to the SIP Gateway

## Two open source SIP Gateways

### Asterisk

### SIP Express Router

## Cisco VoIP Systems

# Asterisk REGISTER Messages

# Asterisk REGISTER User Enumeration

## User agent identifies the server

## REGISTER request with a valid username but unauthorized returns 401 error

## REGISTER request with invalid username returns 403 error

## User enumeration is easy :)

# SIP Express OPTIONS User Enumeration

## Same trick works

### OPTIONS with valid user returns 200

### OPTIONS with invalid user returns 404: User Not Found

# SIPVicious svwar

## Enumerates users with OPTIONS, REGISTER, and INVITE techniques

## Other tools

### SiVuS

### SIPScan

### sipsak

# Cisco IP Phone Boot Process

## Phone sends Cisco Discovery Protocol (CDP) Voice VLAN Query request

## A Cisco device in range responds with the Voice VLAN information

## The phone configures that VLAN on its Ethernet port

## Phone sends DHCP request to find TFTP server

## DHCP server tells the phone the TFTP server's address

## Phone downloads Certificate Trust List, Initial Trust List, and phone configuration from the TFTP server

## Configuration file contains all settings needed to register the phone with the call server

# Cisco User Enumeration

## Cisco Directory Services can be dumped completely with Automated Corporate Enumerator

# VoIP Enumeration Countermeasures

## Segment networks

## Place IDS/IPS systems in strategic areas

## Software developers need to fix these vulns

# Interception Attacks

## Use ARP poisoning to get in the middle

## Sniff UDP traffic

## Identify RTP codec

### G.711 is "toll quality" but uses a lot of bandwidth

### G.729 uses less bandwidth but lowers call quality

### G.722 is common in enterprises today; same bandwidth consumed as G.711 but better quality

# Interception Attack

## Sniff the IP Packets

### With ARP poisoning

## Attacker is set to route traffic, but not decrement the TTL

# Captured RTP Traffic

## It's compressed with a codec

## Common codecs

### G.711 (uses up a lot of bandwidth)

### G.729 (uses less bandwidth)

# VOMIT

## vomit - voice over misconfigured internet telephones

### Converts G.711 to WAV

### It works because many IP phones don't or can't encrypt traffic

### Link Ch 620

## Scapy is an even better tool, plays traffic from eth0 right out the speakers

### Link Ch 621

# Playing the Captured Traffic

## Wireshark

## VOMIT

## scapy

## UCSniff handles many codecs

# Offline Analysis

## Wireshark can play streams

## Dialed numbers appear in Wireshark packet parsing

# Interception Countermeasures

## Encryption: Secure RTP, TLS, and Multimedia Internet Keying (MIKEY)

### Often disabled for performance

## Layer 7 firewall can block rogue RTP traffic and DoS attacks

## Signed configuration and firmware files

# Denial of Service

## SIP INVITE flood

## Flood with any sort of traffic

## Countermeasures

### Network segmentation

### Encrypted protocols

# Skype Information Leak

## Variable-bitrate encoding leaks information

## Link Ch 7d

## Last modified 10-12-12