# Objectives

## Wireline and wireless telecommunication technologies

## Wired and wireless network technologies

## Network topologies and cabling

## The OSI and TCP/IP network models

## TCP/IP networks, protocols, addressing, devices, routing, authentication, access control, tunneling, and services

## Network based threats, attacks, vulnerabilities, and countermeasures

Telecommunications Technologies

# Wired Telecom Technologies

## DS-1, aka T-1

### 24 voice or data channels, each 1.544 Mbit/sec

## Other T-carrier protocols

### DS-3 aka T-3 (673 voice channels, 45mBit/s)

### DS-4 (4,032 channels, 274mBit/s)

### DS-5 (5,760 channels, 400mBit/s)

## E-1 – Euro version

### 32 channels instead of 24, otherwise similar

## SONET (Synchronous Optical NETwork)

### High speed, fiber optic, encapsulates T-protocols, ATM, TCP/IP

### OC-1 - 48.960 Mbit/sec

### OC-3 - 150.336 Mbit/sec

### OC-12 - 601.344 Mbit/sec

### OC-24 - 1,202.688 Mbit/sec

### OC-48 - 2,405.376 Mbit/sec

### OC-96 - 4,810.752Mbit/sec

### OC-192 - 9,621.504 Mbit/sec

## Frame Relay

### Data-Link layer protocol

### Early packet-switched technology that used to transmit data between locations at a lower cost than dedicated T-1 lines

### Switched Virtual Circuits (SVCs) and Permanent Virtual Circuits (PVCs) emulate dedicated T-1

### Frame Relay succeeded X.25

### Being replaced by DSL and MPLS

## ATM (Asynchronous Transfer Mode)

### Synchronous, connection-oriented packet protocol

### Packets called cells, are fixed length (5 byte header, 48 byte payload)

### Not common in LANs, but widely used for WAN links

### Giving way to MPLS.

## DSL (Digital Subscriber Line)

### Digital packet over copper voice circuits at higher clock rate, coexists with low frequency voice

### Modem used on subscriber side to convert DSL signals to Ethernet (and sometimes Wi-Fi)

### DSLAM (Digital Subscriber Line Access Multiplexer) on telco end aggregates signals

## MPLS (Multiprotocol Label Switching)

### Packet switched technology, encapsulates TCP/IP, ATM, SONET, Ethernet frames)

### Carries voice + data, has QoS (quality of service) capabilities to guarantee jitter-free voice and other media such as video

### Replacing Frame Relay and ATM

## Other wired telecom technologies

### Data Over Cable Service Interface Specification (DOCSIS)

#### Used for cable Internet service

### PSTN (Public Switched Telephone Network)

#### 56 Kbps modem

### ISDN (Integrated Services Digital Network)

#### Limited to 128 Kbps

## Other wired telecom technologies

### SDH (Synchronous Digital Hierarchy)

#### Similar to SONET

#### Used outside the USA

### X.25

#### Old packet-switching technology

#### Rarely used anymore in the USA, replaced in the 1990s by Frame Relay

#### A variant of X.25 is used for "Packet radio"

#### X.25 is widely used outside the USA

# Wireless Telecom Technologies

## CDMA2000 (code division multiple access)

### Data transport: 1XRTT (153 kbit/s), EVDO (2.4 Mbit/s), EVDV (3.1 Mbit/s)

### Used by Verizon for the Droid cell phone (link Ch 10b)

## GPRS (General Packet Radio Service)

### Encapsulated in GSM (Global System for Mobile communications) protocol (114kbit/s)

## EDGE (Enhanced Data rates for GSM Evolution)

### Up to 1Mbit/s

## UMTS (Universal Mobile Telecommunications System)

### Transported over WCDMA, up to 14Mbit/s)

## WiMAX (Worldwide Interoperability for Microwave Access)

### Based on IEEE 802.16, WiMAX is a wireless competitor to DSL and cable modems, also competes with CDMA, GPRS, EDGE, UMTS

### Rates range from 2 to 12 Mbit/s, theoretically as high as 70 Mbit/s

### Clear calls this 4G and provides it in the USA, but not yet in San Francisco (link Ch 10c)

## CDPD (Cellular Digital Packet Data)

### first data over cellular, used AMPS analog carrier, up to 19.2 kbit/s

## Packet Radio

### Transmits data over amateur radio bands, using AX.25

Network Technologies

# Wired Network Technologies

## Ethernet

### Frame-based protocol

#### 14 byte header

#### Payload (46-1500 bytes)

#### Checksum

#### Inter-frame gap

### Error detection: Carrier Sense Multiple Access with Collision Detection (CSMA/CD)

### MAC address: 6 bytes. Format xx.xx.xx.yy.yy.yy.

#### xx.xx.xx assigned to manufacturer

## Network cabling

### Ethernet

#### 10BASE-T – this is the commonly twisted-pair network cable that supports the Category 3, 5, or 6 ANSI standard. This cable has 8 conductors, of which 4 are used. An 8-pin RJ45 connector is used to connect a cable to a device.

#### 100BASE-TX – the same twisted-pair network cable (Category 5 and 6) and connectors as 10BASE-T, and also uses just 4 of the 8 conductors

#### 1000BASE-T – the same twisted-pair networkcable and connectors as 100BASE-TX, except that all 8 conductors are used.

#### 10BASE2 – the old “thinnet” coaxial cabling with twist-lock BNC connectors – rarely used.

#### 10BASE5 – the old “thicknet” coaxial cabling that is rarely used.

## Twisted pair cabling

### Category 3 – consists of four twisted pairs in a single jacket.  Suitable only for 10Mbit/s Ethernet.  Superseded by Category 5 and 5e.

### Category 5 – consists of four twisted pairs in a single jacket. Maximum length is 100m.  Suitable for 100Mbit/s and can be used for Gigabit Ethernet.

### Category 5e – supersedes Category 5 and includes specifications for far end crosstalk.

### Category 6 – backward compatible with Category 5 and 5e, but higher specifications for noise and crosstalk, making it more suitable for Gigabit Ethernet.

### Category 7 – even more stringent than Category 6 cabling, Cat-7 is suitable for 10Gbit/s networks.

## Cabling

### Optical

#### Carries signal in the form of light instead of electricity

#### Greater speeds and distances possible

#### More expensive

## Ethernet devices

### Hub – connects local stations together; sends each frame to every connected node

### Repeater – extend signal over distances

### Switch – like a hub but sends data only to the correct node

### Router – connect networks to each other

### Gateway – translates various types of communications

## Token ring

### Logical ring

### Speed: 4Mbit/s and 16Mbit/s

### Mostly replaced by Ethernet

## Universal Serial Bus (USB)

### Successor to RS-232 serial

### Speeds

#### USB 1.0/1.1 – 1.5 Mbits/s and 12 Mbits/s

#### USB 2.0 – 480 Mbits/s

#### USB 3.0 – 4.8 Gbits/s

### Hot pluggable

### Used to connect peripheral and human interface devices

## RS-232

### Serial communications, speeds 110 bit/s – 57.7 kbit/s

### Used to connect communications devices such as modems, and human interface devices such as mice

### Largely replaced by USB

## HSSI (High Speed Serial Interface)

### 52Mbits/s, cable length 50’, used to connect WAN devices

## FDDI (Fiber Distributed Data Interface)

### Token technology over fiber that has been replaced by gigabit Ethernet and SONET

## Fibre Channel

### Gigabit protocol used in SANs (Storage Area Networks)

# Common Network Topologies

# Network Topologies

## Bus. All of the nodes in the network are connected to a single conductor.  A break in the network conductor will cause some or the entire network to stop functioning.  Early Ethernet networks consisting of thinnet coaxial cabling were bus networks.

## Ring.  All of the nodes are connected to exactly two other nodes, forming a circular loop. Breaking any conductor will cause the network to stop functioning.

## Star.  All nodes are connected to a central device.  A break in a conductor will disconnect only one node, and the remaining nodes will continue functioning.  Ethernet networks are physical stars, with computers connected to central hubs or switches.  Token ring networks, while logically as a ring, are physically wired as a star.

# Wireless Network Technologies

## Wi-Fi, also known as WLAN, Wireless LAN

### Wireless data link layer network protocol

### Bandwidth up to 54Mbit/s for 802.11g, distances to 100m

### Some people claim up to 600 Mbps for 802.11n (link Ch 10d)

## Wi-Fi security

### SSID should be a non-default value

### SSID broadcast should be disabled

### MAC access control

### Authentication

#### Require ID and password, may use a RADIUS server

### Encryption

#### WEP (Wired Equivalent Privacy)

#### WPA (Wireless Protected Access)

#### WPA2 (superset of WPA, full standard

# PSK v. RADIUS

## WPA and WPA-2 operate in two modes

## Pre-Shared Key (PSK)

### Users must enter the key on each device

## RADIUS server

### Used with 802.1x authentication

### Each user has an individual key

### More secure, recommended for enterprises

## Bluetooth

### Personal Area Network (PAN) technology

### Data rate: 1Mbit/s – 3Mbit/s

### Distance: up to 10 m

### Devices can authenticate through a process called “pairing”, during which two devices can exchange a cryptographic secret key that the two devices can later use

### Communications between paired devices can also be encrypted

## IrDA

### Infrared Data Association standard

### Infrared light spectrum from 2.4kbit/s to 16Mbit/s

#### Requires line-of-sight

### Once popular, now being replaced with Bluetooth

## Wireless USB (WUSB)

### Wireless protocol designed for wireless connectivity of various computer peripherals

#### Printers, digital cameras, hard disks, and other high-throughput devices.

### Bandwidth ranges from 110 Mbit/s at 10 meters to 480 Mbit/s at 3 meters

### 3.1 to 10.6 GHz frequency range

## Near Field Communication (NFC)

### Ultra-short distance (up to 10cm or 4”)

### Works like RFID

### Intended for cell phones

### Rates: 106 kbit/s, 212 kbit/s, or 424 kbit/s

### Active or passive mode

#### Passive mode ideal for key card access control

### See link Ch 10e

Network Protocols

# OSI Protocol Model

## Application

## Presentation

## Session

## Transport

## Network

## Data link

## Physical

# OSI Mnemonics

## Please Do Not Throw Sausage Pizza Away

## All People Seem To Need Data Processing

# OSI Protocol Model: Physical

## Concerned with a network’s physical media

### Electrical

### Optical

### Radio frequency

## Example standards

### RS-232, RS-422, T1, E1, 10Base-T, SONET, DSL, 802.11a (physical), Twinax

# OSI Protocol Model: Data Link

## Concerned with the transfer of data between nodes

## Manages error correction for any errors that take place at the physical layer

## Example standards

### 802.3 (Ethernet), 802.11a MAC, GPRS, AppleTalk, ATM, FDDI, Fibre Channel, Frame Relay, PPP, SLIP, Token Ring, Wi-MAX

### ARP could be placed here, or in layer 3 (link Ch 10f)

# OSI Protocol Model: Network

## Used to transport variable-length data sequences between nodes

## Manages fragmentation and reassembly

## Communications are point-to-point

## No notion of a “connection”

## Data packets may not arrive in order

## Example standards

### IP, ICMP, ARP, IPX

# OSI Protocol Model: Transport

## Manages the delivery of data from node to node on a network

### Even when there are intermediate devices such as routers and a variety of physical media between the nodes

### Manages “connections”

#### Guarantee the order of delivery of data packets, packet reassembly, error recovery

### Examples: UDP, TCP, IPsec, PPTP, L2TP, SPX

# OSI Protocol Model: Session

## Manages connections between nodes, including session establishment, communication, and teardown

## Example standards

### NetBIOS, TCP sessions, SIP

# OSI Protocol Model: Presentation

## Deals with the presentation or representation of data in a communications session

### Character set translation

### Compression

### Encryption

## Examples of presentation - layer standards include SSL, TLS, MIME, and MPEG

# OSI Protocol Model: Application

## Top-most layer in the OSI network model

## Concerned with the delivery of data to and from applications

## Examples standards

### DNS, NFS, NTP, DHCP, SMTP, HTTP, SNMP, SSH, Telnet, WHOIS

# TCP/IP Protocol Model

## Application

## Transport

## Internet

## Link

### Image from link Ch 10g

# TCP/IP Protocol Model: Link

## Concerned with node to node delivery

## Example standards

### Wi-Fi

### Ethernet

### Token Ring

### ATM

### Frame Relay

### PPP

# TCP/IP Protocol Model: Internet

## Also known as the Internet layer

## Concerned with end-to-end packet delivery, even through intermediate devices such as switches and routers

## Protocols

### IPv4

### IPv6

### ARP

### RARP

### ICMP

### IGMP

### IPsec

## Internet layer routing protocols

### RIP

### OSPF

### IS-IS

### BGP

## Network layer addressing

### Network addresses in IPv4 are 32 bits in length

#### Expressed as a dot-decimal notation, xx.xx.xx.xx, where the range of each ‘xx’ is 0-255 decimal.

#### Typical network address is 141.204.13.200

### Subnets and subnet masking

#### IP address divided into two parts: network and node

#### Subnet mask used to distinguish network and node portions; e.g. 255.255.255.0

### Default gateway – node that connects to other networks

### Address allocation by Regional Internet Registry (RIR), ISPs

### Reserved address blocks

#### Private networks

#### 10.0.0.0 – 10.255.255.255

#### 172.16.0.0 - 172.31.255.255

#### 192.168.0.0 - 192.168.255.255

#### Loopback: 127.0.0.1 - 127.0.0.255 (127.0.0.1 = “me”)

#### Multicast: 224.0.0.0-239.255.255.255

### Network address translation (NAT)

#### Internal private addresses are translated into public routable addresses at the network boundary

### Classful networks

#### Class A

#### Class B

#### Class C

### Classless networks (Classless Internet Domain Routing (CIDR)

#### Variable length subnet masks, not limited to just Class A, B, C

### Types of addressing

#### Unicast (regular node addresses)

#### Broadcast (send to all nodes on a subnet)

#### Multicast (send to a group of notes on different networks)

#### Anycast (send to only one of a group of nodes)

#### See link Ch 10h

# TCP/IP Protocol Model: Transport

## TCP Protocol

### Connection oriented, persistent connections, dedicated and ephemeral ports, sequencing, guaranteed delivery

### Examples: FTP, HTTP, Telnet

## UDP Protocol

### Connectionless, dedicated port numbers only, no sequencing, no guarantee of delivery

### Examples: DNS, TFTP, VoIP

# TCP/IP Protocol Model: Application

## Topmost layer in the TCP/IP protocol stack

## Protocols: DHCP, DNS, Finger, FTP, HTTP, LDAP, NFS, NIS, NTP, Rlogin, RPC, Rsh, SIP, SMTP, SNMP, Telnet, TFTP, VoIP, Whois

# TCP/IP Routing Protocols

## Router-to-router communication protocol used by routers to help determine the most efficient network routes between two nodes on a network

## Helps routers make good routing decisions (making the right choice about which way to forward packets)

## RIP (Routing Information Protocol) – one of the early routing protocols

### Hop count is the metric, maximum = 15

## IGRP (Interior Gateway Routing Protocol) – Cisco proprietary, obsolete

### Multiple metrics: bandwidth, delay, load, and reliability

## EIGRP (Enhanced Interior Gateway Routing Protocol) – Cisco proprietary

### Advances over IGRP including VLSM

## OSPF (Open Shortest Path First) – Open standard for enterprise networks

### Metric is “path cost” (primarily speed)

### Can use authentication to prevent route spoofing

## BGP (Border Gateway Protocol) – the dominant Internet routing algorithm

## IS-IS (Intermediate system to intermediate system) – used primarily by large ISP networks

# Remote Access / Tunneling Protocols

## Tunneling: encapsulating packets of one protocol within another – can include encryption

### Reasons: protection of encapsulated protocol; hide details of intermediary network, authentication of traffic

### VPN – generic term for tunneled (and usually encrypted) network connection from a public network to a private network

#### SSL / TLS

#### SSH

#### IPsec

#### Others: L2TP, PPP, PPTP, SLIP

Network Authentication Protocols

# Authentication Protocols

## RADIUS (Remote Authentication Dial In User Service)

### Over-the-wire protocol from client to AAA (authentication, authorization, accounting) server

## Diameter – more advanced RADIUS replacement

## TACACS (Terminal Access Controller Access-Control System) – authenticates user to a network.

### Between access point or gateway and an AAA server

### Replaced by TACACS+ and RADIUS

## 802.1X – port level access control. System authenticates before user authenticates

## CHAP (Challenge-Handshake Authentication Protocol)

### Between client system and gateway

## PPP uses CHAP

## EAP (Extensible Authentication Protocol)

### Authentication Framework – used to authenticate users in wired and wireless networks.  Used by WPA and WPA2 wireless network standards.

## PEAP (Protected Extensible Authentication Protocol)

### used in wireless networks to authenticate users

### PEAP uses an SSL/TLS tunnel to encrypt authentication information

## PAP (Password Authentication Protocol)

### unsecure because protocol is unencrypted

Network-Based Threats, Attacks, and Vulnerabilities

# Network Threats

## The expressed potential for the occurrence of a harmful event such as an attack

### DoS / DDoS – designed to flood or cause malfunction

### Teardrop - attacker sends mangled packet fragments with overlapping and oversized payloads to a target system

### Sequence number – guesses upcoming sequence numbers as a method for hijacking a session

### Smurf - large number of forged ICMP echo requests.  The packets are sent to a target network’s broadcast address, which causes all systems on the network to respond

### Ping of Death – ICMP echo request, 64k length

### SYN flood – large volume of TCP SYN packets, consumes resources on target system

### Worm – automated, self-replicating program

### Spam – unsolicited commercial e-mail (UCE): fraud, malware, marketing

### Phishing – emails luring users to fraudulent sites

### Pharming – attack on DNS that redirects access to legitimate sites to imposter sites

# Network Vulnerabilities

## Unnecessary open ports

## Unpatched systems

## Poor and outdated configurations

## Exposed cabling

Network Countermeasures

# Network Countermeasures

## Access control lists

## Firewalls

## Intrusion Detection System (IDS)

### Network based (NIDS)

### Host based (HIDS)

## Intrusion Prevention System (IPS)

### Network and host based

## Protection of network cabling

## Anti-virus software

## Private addressing (10.*.*.*, etc.)

## Close unnecessary ports and services

## Security patches

## Unified Threat Management (UTM)

### Security appliances that perform many functions, such as Firewall, IDS, IPS, Antiirus, Anti-spam, Web content filtering

## Gateways – filtering intermediaries

### Last modified 5-5-10