# Topics

## OSI Model and TCP/IP Model

## Securing wired and wireless networks

## Securing e-mail, Internet, Fax, and phone lines

## Network attacks and countermeasures

### In other words, Net+ or CCNA

Data Network Types

## LAN and WAN

# Local Area Network (LAN)

## Small geographic area

### One building, or a few nearby buildings

## Inexpensive to set up and maintain

### PCs, switches, router, WAPs, printers

## Wired, wireless, or both

## High speed

### Wired: 10 Mbps, 100 Mbps,  1 Gbps, or 10 Gbps

### Wireless: 11 Mbps, 54 Mbps, or approx. 300 Mbps

# MBps v Mbps

## Mbps: Megabit per second

## MBps: Megabyte per second (rarely used)

# Wide Area Network (WAN)

## Connect multiple LANS over a larger geographic area

### Metropolitan Area Network (MAN)

### Internet

## Expensive to set up and maintain

### Routers, CSU/DSU, firewalls, VPN concentrators, etc.

### Low speed

# WAN Links

## Dial-up or POTS: 56 Kbps

## DSL: 128 Kbps to 3 Mbps or faster

## T1: 1.544 Mbps

## T3: 45 Mbps

## OC-12: 622 Mbps

## OC-255: 13 Gbps

# WAN Types

## Internet

### Whole globe

## Intranet

### Private WAN of company locations

## Extranet

### Includes a company and its business partners

# Other Network Types

## Personal Area Network (PAN)

### Devices carried on a person: cell phone, PDA, Bluetooth headset, etc.

## Storage Area Network (SAN)

### Large hard drives connected in a network

### Use: SCSI, iSCSI, Fibre Channel, FCoE (Fibre Channel over Ethernet)

### Scalable, enable virtualization, migration, and snapshots

### Facilitate disaster recovery

### Reduce cost for large data storage

# Other Network Types

## Virtual Local Area Network (VLAN)

### Devices connected to different physical switches but traffic delivered as if they shared layer 2 connectivity

## Wireless Local Area Network (WLAN)

## Campus Area Network (CAN)

## Metropolitan Area Network (MAN)

# Other Network Types

## Value-Added Network (VAN)

### Extranet that allows businesses to share information

### e.g. Electronic Data Interchange (EDI) for financial documents

The OSI Model

# Open Systems Interconnection (OSI) Model

### Adopted in 1984 by the International Organization for Standardization (ISO)

## Reduces complexity of networking by separating functions into layers

## Promotes interoperability

## Facilitates troubleshooting

# Please Do Not Throw Sausage Pizza Away
All People Seem To Need Data Processing

## 7: Application

## 6: Presentation

## 5: Session

## 4: Transport

## 3: Network

## 2: Data Link

## 1: Physical

# Data Encapsulation

Layer 1: Physical

# Four Network Topologies

## Star

## Mesh

## Ring

## Bus

# Star Topology

## Ethernet

## Single point of failure in center

# MeshTopology

## Full-mesh

## Partial-mesh

## Internet

## Eliminates single point of failure

# Ring Topology

## Token Ring

## FDDI

## SONET

## Lower image

### Logical ring

### Physical star

# Bus Topology

## Backbone

## A single break isolates portions of the network

## Rare on LANs, common on simple WLANS

# Cables and Connectors

## Analog v. Digital

## Asynchronous v. Synchronous

## Baseband v. Broadband

## Four cable types

### Coaxial

### Twinaxial

### Twisted-pair

### Fiber optic

# Analog v. Digital

## Analog

### Continuously varying

### Subject to distortion, loses information each time it is copied

## Digital

### Uses pulses meaning 0 or 1

### Allows for error-correction

### Can be copied without loss of data

# Asynchronous v. Synchronous

## Asynchronous

### Start and stop bits demarcate frames

### Speed negotiated to match the slowest device

### No clock signal required

## Synchronous

### Clock signal required

### Used by serial links

# Baseband v. Broadband

## Baseband

### Wire is dedicated to one signal

### Used with twister-pair wiring

## Broadband

### Many channels over same medium

### Each signal gets a share of the bandwidth

### Used on coaxial cables or wireless links

# Four cable types

## Coaxial

### Thicknet: RG8 or RG11 (obsolete)

### Thinnet: RG58 with BNC connector (common)

## Twinaxial

### Two cores

### Used in 10 Gb Ethernet for short links (10 m)

### SANs and high-speed core switches

## Twisted-pair

### Cat 3, 5, 5e, 6, 6a

### STP v. UTP

### TEMPEST was a US Military project to study electromagnetic emissions from computers

## Fiber optic

### Singlemode v. Multimode

### SC, ST, or LC connectors

# IEEE Ethernet Designations

## 10Base-T

### 10 Mbps, Baseband, Twisted Pair

## 100Base-TX

### 100 Mbps, Baseband, Twisted Pair

## 100Base-F

### 100 Mbps, Baseband, Fiber Optic

## 1000Base-T/GbE

### Gigabit Ethernet over twisted pair

# Interface Standards

## EIA/TIA-232 (formerly RS-232)

### Up to 115,200 bps

## EIA/TIA-449

### Up to 2 Mbps

## HSSI (High Speed Serial Interface)

### Up to 52 Mbps

# Networking Equipment

## Physical layer

### NIC

### Cables (media)

### Repeaters

### Hubs

Layer 2: Data Link

# Data Link Layer

## Delivers frames to the proper device across a physical network link

## Uses MAC addresses

# Sublayers

## Logical Link Control (LLC)

### Control, sequencing, and acknowledgement of frames

### Timing and flow control

## Media Access Control (MAC)

### Framing

### Error control with CRC

### Identifies MAC addresses

### Controls media access with contention, token-passing, or polling

# Contention

## Each device must compete to use physical link

## Ethernet uses CSMA/CD (Carrier Sense Multiple Access/Collision Detection)

## Wireless networks use CSMA/CA (Carrier Sense Multiple Access/Collision Avoidance)

## These networks cannot really operate at their maximum rated speed under normal conditions

# Token-Passing

## Token frame determines which device may send data

## Deterministic

## Can run right up to maximum speed

## Lost in the marketplace; Token Ring is obsolete

# Polling

## Primary host polls secondary hosts to see if they have data to send

## Used in mainframe environments

# LAN Protocols

## Ethernet

### Most common, up to 100 Gbps

## Token Ring

### 4 or 16 Mbps; obsolete

## FDDI (Fiber Distributed Data Interface)

### 100 Mbps

## ARP (Address Resolution Protocol)

### Finds MAC address from IP Address

### Ubiquitous on modern LANs

## RARP (Reverse ARP)

### Finds IP Address from MAC Address

### Rare, replaced by DHCP

# Unicast, Multicast, Broadcast

## Unicast

### Single source, single destination

## Multicast

### Single source, a group of destination devices

## Broadcast

### Single source to all devices on a LAN

# WLAN Standards

## 802.11a

### 54 Mbps, 5 GHz

## 802.11b

### 11 Mbps, 2.4 GHz

## 802.11g

### 54 Mbps, 2.4 GHz

## 802.11n

### 600 Mbps, 2.4 GHz or 5 GHz

# WLAN Encryption

## WEP

### Easily broken, unsafe to use

## WPA and WPA2

### Much safer thn WEP

### Weakest version of WPA is TKIP

### TKIP isn't broken yet, but has some troubling weaknesses

# WAN Technologies and Protocols: Point-to-Point Links

## L2F (Layer 2 Forwarding)

### An old Cisco tunneling protocol

### No encryption

## L2TP (Layer 2 Tunneling Protocol)

### Used for VPNs

### Derived from L2F and PPTP

### Uses UDP 1701

### No encryption, commonly paired with IPSec

## PPTP (Point-to-Point Tunneling Protocol)

### From Microsoft

### Very commonly used for VPNs

### A tunneling protocol, no authentication or encryption included in the specification

### Uses MS-CHAP in most configurations

### Considered broken, no longer recommended by Microsoft

## SLIP (Serial Line IP)

### Predecessor of Point-to-point Protocol

### Encapsulation, not encryption

# WAN Technologies and Protocols:  Circuit-Switched Networks

## Circuit Switching

### Dedicated physical path connects sender and receiver

### Used in old analog phone networks

### Image from www.guardian.co.uk

## xDSL (Digital Subscriber Line)

### Uses analog lines for broadband digital traffic

### Must be close to a main phone switching station (within 3 miles)

### ADSL (Asynchronous DSL)

#### Up to 12 Mbps down, 3.5 Mbps up

### SDSL (Single-line DSL) (1.544 Mbps up & down)

### HDSL (High-rate DSL)

#### Up to 52 Mbps down, 1,544 Mbps up

### VDSL (Very high Data-rate DSL)

#### Up to 52 Mbps down, 2.3 Mbps up

## DOCSIS (Data Over Cable Services interface Specification)

### Cable Internet

## ISDN (Intgrated Services Digital Network)

### Data over analog phone lines

### BRI (Basic Rate Interface)

#### One 16-Kbps D-Channel and two 64-Kbps B-channels (128 Kbps)

### PRI (Primary Rate Interface)

#### One 64-Kbps D-Channel and 23 64-Kbps B-channels (1.544 Mbps)

# WAN Technologies and Protocols: Packet-Switched Networks

## Packet Switching

### Devices share bandwidth of a each line

### Much more efficient than circuit switching

## ATM (Asynchronous Transfer Mode)

### High-speed, low latency

### Uses fixed-size 53-byte cells

### Best for fiber-optic networks with bursty traffic

## Frame Relay

### Handles multiple virtual circuits

### Uses HDLC encapsulation

## MPLS (Multi-Protocol Label Switching)

### High-speed and versatile

### Can carry IP, ATM, SONET, or Ethernet frames

### Specified at Layers 2 and 3

### Uses labels and Label Switch Routers (LSRs)

## SONET (Synchronous Optical  Network) and SDH (Synchronous Digital Hierarchy)

### High-availability, high-speed, multiplexes, low-latency

### Runs on fiber optics

### Data rates specified by OC levels

# Common Telecommunications Circuits

## DS0 64 Kbps

## DS1 1.544 Mbps (T1)

### 2.048 Mbps in Europe (E1)

## DS3 44.7 Mbps (T3)

## OC-1 51.84 Mbps

## OC-3 155.52 Mbps

## OC-12 622 Mbps

## OC-48 2.488 Gbps

## OC-192 9.9 Gbps

# WAN Technologies and Protocols: Packet-Switched Networks (continued)

## SMDS (Switched Multimegabit Data Service)

### High-speed, connectionless, datagram-based

## X.25

### The first packet-switched network

### Being replaced by Frame Relay

# Other WAN Protocols

## HDLC (High-level Data Link Control)

### Bit-oriented, synchronous

### Default for serial links on Cisco routers

### Various vendor implementations are incompatible

## SDLC (Synchronous Data Link Control)

### Bit-oriented, full-duplex, serial

### Developed by IBM

### Uses polling

# Networking Equipment at Layer 2

## Bridge

### Repeater that connects two network segments

### They may use different layer 2 protocols

## Switch

### A multiport bridge

### Learns MAC addresses from traffic

### Separates collision domains

### Implements VLANs

## DTE (Data Terminal Equipment)

### Generic term for devices at the user end of a user-to-network link

### e.g. Workstation, NIC, Modem, CSU/DSU

### Connects to DCE (Data Communications Equipment) at the network end (ISP)

Layer 3 (Network)

# Layer 3

## Routing

### Directing packets to the destination computer

## Dynamic Routing

### Uses routing protocols to exchange information between routers

### RIP, OSPF, BGP

## Logical Addressing (IP addresses)

# Routing Protocols

## Static Routing

### Administrator manually enters and updates routes

### Cannot automatically route around congestion or outages

### Only practical for small networks

## Dynamic Routing

### Finds best route automatically

### Redirects traffic around outages or congested links

# Distance-Vector Routing Protocols

## RIP, RIPv2, and RIPv3

## Routers only know distance and direction of networks

## Can be slow to converge

### Image from www.rgbstock.com

# Link-State Routing Protocols

## OSPF

## Every router has the complete network map

## Each link's state is sent to every router whenever it changes

## Requires more computation at each router

## Converges rapidly

# Path-Vector Protocol

## Border Gateway Protocol

## Similar to distance-vector but scales better

# RIP (Routing Information Protocol)

## Distance-vector

## Uses hop count as metric

## Hop limit 15

## Has a problem with routing loops which is reduced by

### Split horizon

### Route poisoning

### Holddown timers

## RIP uses UDP port 520

# RIP Security

## RIPv1 has no authentication

## RIPv2 can use plaintext or MD5 authentication

## RIPng supports IPv6 but not authentication

## RIP is very old and simple, but in common use on small networks because it's easy to configure

# OSPF (Open Shortest Path First)

## Link-state

## Widely used in large networks

## Doesn't use TCP or UDP, but its own protocol 89

## Allows various authentication measures

## OSPFv1 is obsolete, everyone uses OSPFv2

## OSPFv3 allows IPv6 but no authentication

# (IS-IS) Intermediate System to Intermediate System

## Link-state

## Used in large service-provider backbone networks

# BGP (Border Gateway Protocol)

## Path-vector

## Used on the Internet backbone

# IP Addressing

## Consists of four bytes, like 147.144.20.1

## Two components

### Network address

### Host address

#### Neither portion may be all 1s or all 0s

## Classes

### Class A

### Class B

### Class C

## Class A

### First byte is reserved for network address

### Last three bytes are for host address

### Supports more than 16 million host computers

### Limited number of Class A networks

### Reserved for large corporations and governments (see link Ch 2b)

### Format: network.node.node.node

## Class B

### First two bytes are reserved for network address

### Last two bytes are for host address

### Supports more than 65,000 host computers

### Assigned to large corporations and Internet Service Providers (ISPs)

### Format: network.network.node.node

#### CCSF has 147.144.0.0 – 147.144.255.255

## Class C

### First three bytes are reserved for network address

### Last byte is for host address

### Supports up to 254 host computers

### Usually available for small business and home networks

### Format: network.network.network.node

## Subnetting

### Each network can be assigned a subnet mask

### Helps identify the network address bits from the host address bits

## Class A uses a subnet mask of 255.0.0.0

### Also called /8

## Class B uses a subnet mask of 255.255.0.0

### Also called /16

## Class C uses a subnet mask of 255.255.255.0

### Also called /24

# Planning IP Address Assignments

## Each network segment must have a unique network address

## Address cannot contain all 0s or all 1s

## To access computers on other networks

### Each computer needs IP address of gateway

## TCP/IP uses subnet mask to determine if the destination computer is on the same network or a different network

### If destination is on a different network, it relays packet to gateway

### Gateway forwards packet to its next destination (routing)

### Packet eventually reaches destination

# IPv6

## 128-bit addresses in hexadecimal

## 2001:db8:1:2:3:4:5:acdc

## Not widely deployed yet

# ICMP (Internet Control Message Protocol)

## Reports errors

## Used by Ping and Traceroute

## Regarded as a layer 3 protocol

# Networking Equipment at Layer 3

## Routers

### Forward data packets to logical addresses (IP)

## Gateways

### Connect dissimilar networks

### IP-to-voice

### Exchange-to-Lotus Notes (email)

Layer 4 (Transport)

# Flow Control

## Windows size and acknowledgements allow the receiving device to control the rate of data transmission

## Multiplexing

### Traffic for many services share a single physical link

## Virtual circuit management

## Error checking and recovery

# TCP (Transmission Control Protocol)

## Connection-oriented and reliable, but slow

## Begins with handshake

### SYN, SYN/ACK, ACK

# UDP (User Datagram Protocol)

## Connectionless, best-effort (unreliable), fast

## No handshake

## No re-sequencing

## No error-checking or recovery

# SSL/TLS (Secure Sockets Layer / Transport Layer Security)

## Session-based

## Encrypted

## Authentication provided with certificates

Layer 5: Session

# Three Stages of a Session

## Connection establishment

### Simplex, half-duplex, or full-duplex

## Data transfer

## Connection release

# Session Layer Protocols

## NetBIOS

### Microsoft LANs

## NFS (Network File System)

### Unix shared resources

## RCP (Remote Procedure Call)

### Client-server redirection tool

## SSH and SSH-2 (Secure Shell)

### Alternative to Telnet

## Session Initiation Protocol (SIP)

### Used for IP telephony

Layer 6: Presentation

# Layer 6 Tasks

## Data representation

## Character conversion

## Data compression

## Data encryption

# Layer 6 Protocols

## ASCII

## EBCDIC

## GIF

## JPEG

## MPEG

Layer 7 (Application)

# Layer 7 Tasks

## Provides interface to the user

## Identifying and establishing availability of communication partners

## Determining resource availability

## Synchronizing communication

# Application Layer Protocols

## FTP

## HTTP

## HTTPS

## IMAP

## POP3

## PEM (Privacy Enhanced Mail)

## SET (Secure Electronic Transaction)

## S-HTTP

## S/MIME (Secure Multipurpose Internet Mail Extensions)

## S-RPC

## SMTP

## SNMP

## Telnet

## TFTP

### Last modified 2-7-13