Ch 11: Fiber Optic Installation Safety

# Bare Fiber Safety

## If the shards get into your skin, you will have to wait for them to work out

## Wear eye protection!

## Put shards on sticky tape

## Account for all shards

# Eye Safety (Laser Damage)

## Most sources are low-power and no great risk

## High power sources might burn the retina with invisible light

### Images from tsogalveston.com and chppm-www.apgea.army.mil (Links Ch 11c-e)

Never look into the end of a cable that might be in use

Normal retina

Laser Damage

## Cap unused connectors

## Tape unterminated fiber ends

## Mark enclosures with warning labels

## Laser test sources are dangerous, don’t leave them lying around

# Chemical Safety

## MSDS: Material Safety Data Sheet – a document that lists the risks of a chemical

### Alcohol – flammable

### Adhesives – irritate skin, require ventilation

### Index matching gel – irritate eyes

### Silicone adhesives like RTV are used as mechanical sealant – can irritate skin or eyes

# UV Light Sources

## Used to cure some adhesives

## Harmful to eyes

## Causes skin cancer

# Canned Compressed Air

## Can explode if heated or damaged

## Some use flammable propellant

# Gas in Manholes

## There may be no oxygen in the hole

### Working Alone – test the space first

### Working as a Crew

#### One worker in the hole, wearing a harness

#### One above hole

## Image from cpchem.com (Link Ch 11a)

# Tools and Equipment

## Step Ladders

### Don’t stand on top step

### Must be in good repair

## Extension Ladders

### Must reach three feet beyond bearing point

### Angle: 1 foot of spacing for every 3 or 4 feet of height

## Knives

### Hook blade is most common

### Cut away from yourself

## Syringes and Needles

### They are blunt and not a medical hazard

## Microscopes

### Use a power meter first to make sure there is no light coming out of the fiber

## Cable Installation Equipment

### Trenchers, backhoes, boring machines

### Obvious big-equipment hazards

## Toolbox

### Have a container for spare knife blades

### Prevent liquid spills, battery shorts, etc.

# Personal Safety Equipment

## Gloves protect you from

### Corrugated armor sheaths

### Fiberglass splinters

## Safety Glasses

# Safe Working Area

## Enough table space

## Good Lighting

## Don’t eat or drink in work area

Prevent falls – loose cables, open holes

# Measuring Wheel

## Image from englo.ee (link Ch 11g)

## Clean work area after you are done, properly dispose of all shards

## Aerial Installation Hazards

### Ladder safety

### Electric power lines

## Weather

## Employee Training

Ch 12: Planning the Installation

# Why is Planning Necessary?

## Construction is not repetitive

### Each job is different

## By the time you perfect your procedure, it’s time for the next job

## You must avoid mistakes through planning

# Measuring for Conduit Pulls

## Avoid unneeded splices – they lose signal and cost time and money

## Measuring from prints

### Inaccurate, so you waste cable

## With a Wheel On-Site

### Much more accurate

Measurement/Pull Tape

# Measurement/Pull Tape

## Image from arncocorp.com (Link Ch 11f)

### Most accurate

# Extra Length

## Leave 7 to 10 feet extra for termination

## Access or repair coils are needed

### Michigan State U. required 30 feet extra cable in conduit at each termination point (link Ch 11h)

## Allow 1% extra length outside

## Allow 5% - 7% extra length inside

# Splicing

## Fusion splicing cannot be done in manholes

### So 30 feet slack needed on each side of the splice (60 feet extra cable)

## Mechanical splices can be done in manholes

Only 10 feet of extra cable needed

## Innerduct

## Image from

## store.cablesplususa.com

## (link Ch 11i)

# Efficient Pulling

## If a pull is too long, the tension will grow too high

### Break up the pull with figure-eighting  in locations with enough room

## Don’t leave reels of cable unprotected

### Cable can cost thousands of dollars

# Duct Space

## Large ducts are divided by innerduct

## Protects the fiber cable

## Reduces friction

# Initial Planning Walkout

## Project Engineer

### Knows customer’s needs

## Lead Technician

### An experienced fiber installer

## Project Superintendent

### Must coordinate activities of workers

## All obstacles should be found and planned for, so crews don’t sit idle waiting during the job

Last modified 3-29-06