Ch 13: Fiber Optic Cable Plant Documentation

# Why Document?

## Save time and material during installation

## Help in planning upgrades and troubleshooting

## Test data with loss measurements are usually required for acceptance by end user

# Required Data

## Cable manufacturer, type, length

## Fiber type and size, splices and termination points, losses

## Connections: types, locations, losses

## Path of every cable

### Store all this in a database, often with OTDR traces

# Cable Plant Record Keeping

## Designing

### Plan each cable run, patch panel, etc.

## Bidding

## Installing

### Tape patch panel printouts inside panel covers

### Unless the records are clear, time and money will be wasted

## Testing

### Test each link and keep the loss data

### Future changes can be guided by comparing to the original data

# Cable Plant Record Keeping

## Troubleshooting

### Good documentation helps greatly

## Documenting for Customer Acceptance

### If you document properly all along, you can just print a copy for the customer

# The Documentation Process

## Network Diagram

### Sketch for small network

### CAD (Computer-aided Design) for large projects

#### Such as Visio -- see link Ch 13a

#### Image from TechRepublic.com

# Items to Document

## Where all cables go

## What every fiber connects to

## Specifications of every cable and fiber

## Lengths and installation techniques

## Panels and closet hardware types

## What end equipment is connected

## Dark fibers (dark fibers are unused fibers available for future use)

# Computers and Software

## Spreadsheets

## Databases

## Visio drawings

### Give end user electronic and paper copies

### Keep backup copies

### Store instructions for use with files

# Documentation Nomenclature

## TIA/EIA-606 Administration Standard

## Alphanumeric ID codes are used for each item

## These codes are only examples

### From anixter.com

### Link Ch_13b

# Grounding Busbars

## Required in each closet and in the main telco entrance

## Document grounding so it can be inspected to meet codes

Ch 14: Estimating and Bidding Fiber Optic Installation

# Required Skills

## Arithmetic (not math)

## Command of language and acronyms

### Others will need to read and understand your notes and calculations

## Imagination

### Visualize the project and portions of it

## Understanding of On-Site Expenses

### You will need experience with all installation procedures and products

## Handwriting

### If your writing is illegible, PRINT

## Computer Skills

### Database, spreadsheet, or estimating software

### Email

# Tools

## Pencil

## Printing Calculator

## Tape recorder

## Camera

## Lined estimating sheets

### See example: link

## Computer estimating software

# Site Visit

## You need to notice locations, routing, and elevations

## Check for errors in the drawings

## Notice construction in progress that may affect your job

# Reality of Fiber Optic Installations

## Usually you are upgrading old systems to a new high-speed infrastructure

## Existing telco closets are usually small and crowded, and the different signals interfere

## The cure is to consolidate it all onto fiber – this is network convergence

# Estimating Process

## Takeoff

### Taking information off the plans and transferring it to estimating sheets

## Writing up the estimate

## Summarizing the estimate

# Takeoff

## Review the symbol list and specifications

## Mark all items – fill them in as they are counted

## Count the most expensive items first

### Later passes may reveal missed items

## Obtain a quantity from a quantity

### Count length, calculate straps from it

## Don’t rush

## Work in a quiet comfortable area

## Develop mental pictures of the project

# Writing Up the Estimate

## Transfer the takeoff information to sheets

## Assign material and labor costs

## Subtotal each sheet

# Summarizing the Estimate

## Add all the pricing sheets

## Add other costs, overhead and profit

## Result is the bid price

# Labor Units

## Normal conditions

### Average worker

### Max. working height 12 feet

### Normal availability of workers

### Reasonably accessible work area

### Proper tools and equipment

### Building not exceeding three stories

### Normal weather condition

# Fiber Optic Labor Units 			Normal — Difficult

## Installing cable in conduit, 1-4 fibers		0.016 – 0.02 hours per foot

## Installing cable in conduit, 144 fibers		0.05 – 0.065 hours per foot

## Testing, per fiber				0.12 – 0.24 hours

## Terminating (polish required)			0.40 – 0.6 hours apiece

# Removal of Abandoned Cables

## Required by NEC starting in 2002

## Can be much more difficult than installing new cable

### Because you must avoid harming other cables still in use

# Training

## You may have to teach the owners or their representatives how to use the system

## You will also have to handle phone calls from them later

## It’s best to incorporate these costs in the bid, as special labor

## Don’t try to skimp or avoid this cost!

# Overhead

## Overhead is indirect costs

### Site security

### On-site trailers

### Health and safety

### Office space (rent, utilities, etc.)

### Insurance

### Equipment & software

#### From links Ch 13d & 13e

## High-tech work has a higher overhead

### Because it’s more complex and less established

### Higher risk

Last modified 4-21-06