Optical Fiber

# Fiber v. Copper

## Optical fiber transmits light pulses

### Can be used for analog or digital transmission

### Voice, computer data, video, etc.

## Copper wires (or other metals) can carry the same types of signals with electrical pulses

# Advantages of Fiber

## Fiber has these advantages compared with metal wires

### Bandwidth – more data per second

### Longer distance

### Faster

### Special applications like medical imaging and quantum key distribution are only possible with fiber because they use light directly

# Elements of a Fiber Data Link

## Transmitter emits light pulses (LED or Laser)

## Connectors and Cables passively carry the pulses

## Receiver detects the light pulses

# Repeaters

## For long links, repeaters are needed to compensate for signal loss

# Optical Fiber

## Core

### Glass or plastic with a higher index of refraction than the cladding

### Carries the signal

## Cladding

### Glass or plastic with a lower index of refraction than the core

## Buffer

### Protects the fiber from damage and moisture

## Jacket

### Holds one or more fibers in a cable

# Singlemode Fiber

## Singlemode fiber has a core diameter of 8 to 9 microns, which only allows one light path or mode

### Images from arcelect.com (Link Ch 2a)

# Multimode Step-Index Fiber

## Multimode fiber has a core diameter of 50 or 62.5 microns (sometimes even larger)

### Allows several light paths or modes

### This causes modal dispersion – some modes take longer to pass through the fiber than others because they travel a longer distance

### See animation at link Ch 2f

# Multimode Graded-Index Fiber

## The index of refraction gradually changes across the core

### Modes that travel further also move faster

### This reduces modal dispersion so the bandwidth is greatly increased

# Step-index and Graded-index

## Step index multimode was developed first, but rare today because it has a low bandwidth (50 MHz-km)

## It has been replaced by graded-index multimode with a bandwidth up to 2 GHz-km

# Plastic Optical Fiber

## Large core (1 mm) step-index multimode fiber

## Easy to cut and work with, but high attenuation (1 dB / meter) makes it useless for long distances

# Sources and Wavelengths

## Multimode fiber is used with

### LED sources at wavelengths of 850 and 1300 nm for slower local area networks

### Lasers at 850 and 1310 nm for networks running at gigabits per second or more

# Sources and Wavelengths

## Singlemode fiber is used with

### Laser sources at 1300 and 1550 nm

### Bandwidth is extremely high, around 100 THz-km

# Fiber Optic Specifications

## Attenuation

### Loss of signal, measured in dB

## Dispersion

### Blurring of a signal, affects bandwidth

## Bandwidth

### The number of bits per second that can be sent through a data link

## Numerical Aperture

### Measures the largest angle of light that can be accepted into the core

# Attenuation and Dispersion

## See animation at link Ch 2e

# Measuring Bandwidth

## The bandwidth-distance product in units of MHz×km shows how fast data can be sent through a cable

## A common multimode fiber with bandwidth-distance product of 500 MHz×km could carry

### A 500 MHz signal for 1 km, or

### A 1000 MHz signal for 0.5 km

#### From Wikipedia

# Numerical Aperture

## If the core and cladding have almost the same index of refraction, the numerical aperture will be small

## This means that light must be shooting right down the center of the fiber to stay in the core

## See Link Ch 4d

# Fiber Types and Specifications

## From Lennie Lightwave  (www.jimhayes.com/lennielw/fiber.html)

# Popular Fiber Types

## At first there were only two common types of fiber

### 62.5 micron multimode, intended for LEDs and 100 Mbps networks

#### There is a large installed base of 62.5 micron fiber

### 8 micron single-mode for long distances or high bandwidths, requiring laser sources

#### Corning’s SMF-28 fiber is the largest base of installed fiber in the world (links Ch 2j, 2k)

# Gigabit Ethernet

## 62.5 micron multimode fiber did not have enough bandwidth for Gigabit Ethernet (1000 Mbps)

## LEDs cannot be used as sources for Gigabit Ethernet – they are too slow

## So Gigabit Ethernet used a new, inexpensive source:

### Vertical Cavity Surface Emitting Laser (VCSEL)

# Multimode Fiber Designed for VCSELs

## First came laser-rated 50 micron multimode

### Bandwidth 500 MHz-km at 850 nm

## Then came laser-optimized 50 micron multimode

### Bandwidth 2000 MHz-km at 850 nm

### Distinctive aqua-colored jacket

#### See links Ch 2g, 2h, 2i

# Don’t Mix Fiber Types

## You can’t mix singlemode and multimode fiber – you lose 20 dB at the junction (99% of the light!)

## Mixing 50 micron and 62.5 micron multimode is not as bad, but you lose 3 dB (half the power) which is usually unacceptable

# Flash Cards

## To memorize this stuff, I use online flash cards

### Go to samsclass.info

### Click on CNIT 211

### Click on Flashcards

### Choose Ch 2a: Fiber Types

Fiber Manufacture

# Three Methods

## Modified Chemical Vapor Deposition (MCVD)

## Outside Vapor Deposition (OVD)

## Vapor Axial Deposition (VAD)

# Modified Chemical Vapor Deposition (MCVD)

## A hollow, rotating glass tube is heated with a torch

## Chemicals inside the tube precipitate to form soot

## Rod is collapsed to crate a preform

## Preform is stretched in a drawing tower to form a single fiber up to 10 km long

### Image from thefoa.org

# Outside Vapor Deposition (OVD)

## A mandrel is coated with a porous preform in a furnace

## Then the mandrel is removed and the preform is collapsed in a process called sintering

### Image from csrg.ch.pw.edu.pl

# Vapor Axial Deposition (VAD)

## Preform is fabricated continuously

## When the preform is long enough, it goes directly to the drawing tower

### Image from csrg.ch.pw.edu.pl

# Drawing

## The fiber is drawn from the preform and then coated with a protective coating

# Index of Refraction

## When light enters a dense medium like glass or water, it slows down

## The index of refraction (n) is the ratio of the speed of light in vacuum to the speed of light in the medium

## Water has n = 1.3

### Light takes 30% longer to travel through it

## Fiber optic glass has n = 1.5

### Light takes 50% longer to travel through it

Fiber Applications

# Step-index Multimode

## Large core size, so source power can be efficiently coupled to the fiber

## High attenuation (4-6 dB / km)

## Low bandwidth (50 MHz-km)

## Used in short, low-speed datalinks

## Also useful in high-radiation environments, because it can be made with pure silica core

# Graded-index Multimode

## Useful for “premises networks” like LANs, security systems, etc.

## 62.5/125 micron has been most widely used

### Works well with LEDs, but cannot be used for Gigabit Ethernet

## 50/125 micron fiber and VSELS are used for faster networks

# Singlemode FIber

## Best for high speeds and long distances

## Used by telephone companies and CATV

Fiber Performance

# Attenuation

## Modern fiber material is very pure, but there is still some attenuation

## The wavelengths used are chosen to avoid absorption bands

### 850 nm, 1300 nm, and 1550 nm

### Plastic fiber uses 660 nm LEDs

#### Image from iec.org (Link Ch 2n)

# Three Types of Dispersion

## Dispersion is the spreading out of a light pulse as it travels through the fiber

## Three types:

### Modal Dispersion

### Chromatic Dispersion

### Polarization Mode Dispersion (PMD)

# Modal Dispersion

## Modal Dispersion

### Spreading of a pulse because different modes (paths) through the fiber take different times

### Only happens in multimode fiber

### Reduced, but not eliminated, with graded-index fiber

# Chromatic Dispersion

## Different wavelengths travel at different speeds through the fiber

## This spreads a pulse in an effect named chromatic dispersion

## Chromatic dispersion occurs in both singlemode and multimode fiber

### Larger effect with LEDs than with lasers

### A far smaller effect than modal dispersion

# Polarization Mode Dispersion

## Light with different polarization can travel at different speeds, if the fiber is not perfectly symmetric at the atomic level

## This could come from imperfect circular geometry or stress on the cable, and there is no easy way to correct it

## It can affect both singlemode and multimode fiber.

# Modal Distribution

## In graded-index fiber, the off-axis modes go a longer distance than the axial mode, but they travel faster, compensating for dispersion

### But because the off-axis modes travel further, they suffer more attenuation

# Equilibrium Modal Distribution

## A long fiber that has lost the high-order modes is said to have an equilibrium modal distribution

## For testing fibers, devices can be used to condition the modal distribution so measurements will be accurate

# Mode Stripper

## An index-matching substance is put on the outside of the fiber to remove light travelling through the cladding

### Figure from fiber-optics.info (Link Ch 2o)

# Mode Scrambler

## Mode scramblers mix light to excite every possible mode of transmission within the fiber

### Used for accurate measurements of attenuation

### Figure from fiber-optics.info (Link Ch 2o)

# Mode Filter

## Wrapping the fiber around a 12.5 mm mandrel

### Exceeds the critical angle for total internal reflection for very oblique modes

### The high-order modes leak into the cladding and are lost

### That creates an equilibrium modal distribution

### Allows an accurate test with a short test cable

#### Figure from fiber-optics.info (Link Ch 2o)

Decibel Units

# Optical Loss in dB (decibels)

## If the data link is perfect, and loses no power

### The loss is 0 dB

## If the data link loses 50% of the power

### The loss is 3 dB, or a change of – 3 dB

## If the data link loses 90% of the power

### The loss is 10 dB, or a change of – 10 dB

## If the data link loses 99% of the power

### The loss is 20 dB, or a change of – 20 dB

## dB = 10 log (Power Out / Power In)

# Absolute Power in dBm

## The power of a light is measured in milliwatts

## For convenience, we use the dBm units, where

#### -20 dBm 	= 0.01 milliwatt

#### -10 dBm 	= 0.1 milliwatt

#### 0 dBm 	= 1 milliwatt

#### 10 dBm 	= 10 milliwatts

#### 20 dBm 	= 100 milliwatts

Last modified 1-30-06