Where Fiber is Used

# History

## First fiber optic network installed in Chicago in 1976 (Link Ch 3k)

## Now more than 90% of long-distance voice links use fiber optics

## CATV is sent over fiber optics using an analog transmission

### Soon it will convert to digital compressed video

## The Internet backbone is also fiber

# Where New Fiber is Going

## “The Last Mile”

### Fiber to the home

### Fiber to the Curb

### Fiber to the office

### Fiber to the telephone

### Fiber to the desktop

#### See links Ch 3a, 3b, 3c

Economics of Fiber

# Telephony

## A single strand of fiber can carry more traffic than thousands of copper pairs

### Fiber saves in materials costs, installation, splicing labor and reliability, and uses less space in the conduits

## Fiber is the clear choice for high traffic and long distance links

# FTTC and FTTH

## Fiber To The Curb and

## Fiber To The Home

### Not cost-efficient yet, but will become needed as customers demand more bandwidth

# SONET

## Synchronous Optical Network

### A standard to enable telecommunications networks to interoperate

### The international equivalent to SONET is SDH (Synchronous Digital Hierarchy)

### Now that data traffic surpasses voice traffic, SONET is threatened by IP (Internet Protocol) and VoIP (Voice over IP)

# Fiber To The Premises (FTTP)

## Telcos are deploying FTTP

### Verizon spent $1 billion in 2005 to deploy FTTP to 3 million homes and businesses in 16 states

### It plans to continue spending $1 to $2 billion per year on FTTP until 2010

## AT&T is spending $6 billion on FTTN (Fiber To The Neighborhood or Fiber To The Node) to be completed in 2008 (links Ch 3d, 3e)

# CATV (Cable television)

## CATV often converts from copper to fiber by overbuilding existing metal cable

### Attaching fiber to existing hanging cables, or

### Pulling fiber through existing underground ducts

# LANs (Local Area Networks)

## Most computer networks use UTP (Unshielded Twisted-Pair) copper cables

### UTP has a maximum length of 100 meters, and a maximum speed of 1 Gbps

### Fiber is not popular to the desktop because the electronics cost more

### Fiber is popular on network backbones

# Ethernet

## Ethernet is the most popular LAN standard

### Runs at 10 Mbps, 100 Mbps, 1 Gbps, or 10Gbps

### Can run over copper or fiber

### 10 Gbps is new and slowly gaining acceptance, and may replace Fibre Channel for server clusters (link Ch 3f)

# Media Converters

## Convert UTP Ethernet to Fiber Optics

### Allows an Ethernet network to be greatly extended, mixing fiber and copper on the LAN

#### Image from link Ch 3g (versitron.com)

# Media Converters

## This is a 1 Gbps media converter

### See links Ch 3h, 3i

### Image from link Ch 3h (arcelect.com)

# Specifications for the 1 Gbps Converter

## Note error on last line – 500 km is impossible for multimode

### From link Ch 3h (arcelect.com)

# Older LAN Standards

## All these obsolete LAN standards could run over either copper or fiber

### ARCnet

### WangNet

### DECnet

### Token Ring

# Older Fiber LAN Standards

## These obsolete LAN standards were designed for fiber

### Fiber Distributed Data Interface (FDDI)

#### Replaced by Ethernet

### Enterprise System Connection Architecture (ESCON)

#### Replaced by Fibre Channel

# Fibre Channel

## Fibre Channel is used for server clusters and storage area networks

### Image from link Ch 3j (hsi.web.cern.ch)

# Specialized Networks

## IEEE 1394 and TOSLINK

### Used for consumer electronics over copper or plastic optical fiber

## MOST (Media Oriented System Transport)

### Automotive multimedia connecting media devices within the car

## Flexray

### Automotive network for safety devices like airbags

# Other Applications of Fiber Optics

## Surveillance and news cameras

## Security devices like card readers

## Electric utilities control the grid with fiber optics, to avoid electromagnetic interference (EMI)

Fiber v. Copper

## Copper cables must be 100 meters or less

## That means you need a powered telecom closet on each floor

## Fiber only needs one telecom closet per building

### Image from Lennie Lightwave

# Cost of Fiber v. Copper

## Copper components are cheaper, but you need more total equipment and it uses more space

## If you include everything, fiber may even be cheaper than copper

### Cost model at fols.org (link Ch 3l)

# Future-Proofing the Installation

## Plan for growth

## Install extra fibers

### Most people still use 62.5/125 multimode

#### 50/125 multimode runs faster and will make future expansion of bandwidth possible

### Singlemode fiber is all the same

#### Except specialty dispersion-shifted or dispersion-flattened which are not recommended for general use

# Termination

## Termination is attaching connectors to the end of the fiber strands

## Fiber cannot be used until it is terminated

## Termination is the largest labor cost for fiber

### Install extra strands, but don’t terminate them till you need them

#### Image from Lennie Lightwave

The Future of Fiber

## Fiber offers the most bandwidth, which will be increasingly needed by:

### Voice over IP

### Video conferencing

### Video and high-speed gaming over the Internet

#### Korean Internet users have 20 Mbps connections, and are currently installing  100 Mbps FTTH (see links Ch 2m, 2n, 2o)

Last modified 1-30-06