Optical Fiber Cable Construction

# Fiber and Acrylate Coating

## Optical fiber is covered by an acrylate coating during manufacture

### Coating protects the fiber from moisture and mechanical damage

# Three Cable Types

## Tight-buffer

## Loose-tube

## Ribbon

# Loose-Tube and Tight-Buffer

## A cable has:

### Optical fiber (core, cladding, and coating)

### Buffer (loose or tight)

### Kevlar strength member

### Outer Jacket

#### Image from commspecial.com (link Ch 4a)

# Loose-Tube Cable

## Used for long distances and outside-plant

## Low attenuation

## High pulling strength

### Fibers are completely separated from environment

### Image from alphawire.com (link Ch 4e)

## Tubes contain gel or absorbent tape to protect fibers from water

### Image from alphawire.com (link Ch 4e)

## Fibers must be terminated with breakout kits

### Or fusion splicing to pre-connectorized pigtails

### Image from fonetworks.com (link Ch 4d)

## Used Outdoor

Ducts or conduits

Breakout Kit

### Aerial lashed

### Directly buried (armored)

## Weather-resistant

### From alphawire.com (link Ch 4k)

Armored cable

# Tight-Buffer Cable

## PVC Buffer is extruded directly onto the coating

### Diameter is 900 microns

### Makes cable more flexible

### Easier to terminate

### The most common indoor cable type

### Not good for outside use

#### Because the buffer strains the fiber as temperature fluctuates, increasing attenuation

#### Image from mohawk-cdt.com (link Ch 4f)

# Distribution Cables

### Several tight-buffer fibers

### Kevlar reinforcement (Aramid)

### One jacket

### Contains tight-buffered fibers in bundles of up to 12 each

### Used for Riser and Office Cabling

### Must be terminated inside a patch panel or junction box

#### From alphawire.com (link Ch 4i)

# Breakout Cables

### Reinforce each tight-buffer fiber with Kevlar and jacket it

### Each fiber can be broken out and individually connectorized

#### Image from arcelect.com (link Ch 4g)

### Several simplex units cabled together

### Stronger, larger and more expensive than distribution cables

### Used for Riser and Office Cabling

### Cables can be terminated individually and connected directly to computers

#### From alphawire.com (link Ch 4j)

# Ribbon cable

## Dozens of fibers packed together

## Can be mass fusion spliced or mass terminated

## Images from gore.com (link Ch 4b) and alcatel.com (link Ch 4c)

# Cable Jacketing

## Chemical Resistance

### Kynar, PFA, Teflon, Tefzel, or Halar

## Aerospace

### Wide temperature range, ½ inch sustained bend radius

# Fire Safety

## Inside cable must meet National Electric Code fire safety requirements

### Horizontal Fire Propagation

### Dripping of flaming material

#### Irradiated Hypalon or XLPE meets these requirements (plenum rated)

## Use of black outside-plant cable in buildings is limited to 50 feet

### See link Ch 4l

# Five Standard Cable Types

## Simplex and Zipcord

## Distribution

## Breakout

## Loose-tube

## Hybrid or Composite

# Simplex and Zipcord

## Used For:

### Patch Cables

### Short Run Office Cabling

### Test Equipment

#### From alphawire.com (link Ch 4h)

# Hybrid or Composite Cable

## Hybrid cable

### Contains both singlemode and multimode fibers

## Composite cable

### Contains both copper wires and fiber optics

## The two terms are often confused

Choice of Cables

# Choosing a Cable

### From lanshack.com (link Ch 4m)

| ## Cable Type | ## Application | ## Advantages |
| --- | --- | --- |
| ## Tight Buffer | ## Premises | ## Makes rugged patch cords |
| ## Distribution | ## Premises | ## Small size for lots of fibers, inexpensive |
| ## Breakout | ## Premises | ## Rugged, easy to terminate, no hardware needed |
| ## Loose Tube | ## Outside Plant | ## Rugged, gel or dry water-blocking |
| ## Armored | ## Outside Plant | ## Prevents rodent damage |
| ## Ribbon | ## Outside Plant | ## Highest fiber count for small size |

# Factors to Consider when Choosing Cable

## Bandwidth

## Attenuation & Length

## Cost

## Mechanical & UL/NEC Requirements

## Signal loss

## Connectors

## Cable Dimensions & Environment

## Existing systems

# Installing Fiber Optic Cable

## Never pull on the fiber itself

### Pull only from the strength members

### Maximum pulling force 300 lb.

## Never exceed bend radius

### 10 x diameter without tension

### 20 x diameter under tension

#### Images from Lennie Lightwave and bwcecom.belden.com (Link Ch 4n)

# Vertical Installations

## Clamp the cable to distribute weight evenly

### Every 50 feet indoors

### More often, up to every 3 feet, outdoors

## Fiber migrates down, increasing attenuation

### Place 1 foot loops at the top of the run, bottom of run, and every 500 ft in between

# Pulling Cables through Conduits

## Loose-buffer cables work best

### Video from American Polywater (link Ch 4o)

# Direct Burial

## Armor is desirable to protect the cable from gophers, construction digging, etc.

## Gel filling is a water barrier

# Aerial Installation

## Supported by messenger wire, or

## Self-supporting

### Images of self-supporting cable from ericsson.net and  Arcelect.com (link Ch 4p & 4g)

## OGW (Optical Ground Wire) cables carry both electricity and fiber optic signals

# Blown-in Fiber

## Fiber is blown through plastic tubes with air

## Faster than pulling

### Image from fpnmag.com (link Ch 4q)

# Fire Code Ratings

| NEC Rating | Description |
| --- | --- |
| OFN | Optical fiber non-conductive |
| OFC | Optical fiber conductive |
| OFNG or OFCG | General purpose |
| OFNR or OFCR | Riser rated cable for vertical runs |
| OFNP or OFCP | Plenum rated cables for use in air-handling plenums |
| OFN-LS | Low smoke density |

### See link Ch 4r for details

# NEC Requirements

## OFC cables have noncurrent-carrying conductive elements

### Must be grounded

### Cannot share a raceway with electric power lines

### Can share with communications lines

## Abandoned cables must be removed

Last modified 4-15-06