Cable Parameters and Typical Values

# Installation v. Environmental Specifications

## Installation Specifications

### Ex: Installation Load (Pulling force)

## Environmental Specifications

### Determine the cable’s long-term performance

### Ex: Temperature range of operation

# Installation Specifications

## Maximum recommended installation load

### Force in lb. (or kg-force or N)

### Pulling with more than this force will permanently change the attenuation of the fiber

### Typical values:

#### 67-125 lb. for 1-fiber cables

#### 250-500 lb. for cables with 6-12 fibers

#### 600 lb. for self-supporting aerial cables

## Minimum recommended installation bend radius

### Don’t bend the cable under tension through a corner sharper than this

### If you violate the bend radius, you may damage some part of the cable structure

### Typical value: 20 times cable diameter

## Diameter

### Important when fitting the cable into a crowded conduit

## Temperature range for installation and storage

# Environmental Specifications

## Temperature range

### Outside this range, the plastic may crack, or

### Expansion cycles will create microbends in the fiber, increasing attenuation

#### Indoor, typically -10 to 50 Centigrade

#### Outdoor, typically -20 to 60 Centigrade

#### Military: -55 to 85 Centigrade

#### Image: a Teflon-coated fiber optic thermometer that operates down to 5 degrees Kelvin (link Ch 5i)

## Minimum long-term bend radius

### With the cable not under tension

### Typically 10 diameters

#### There is a new Japanese fiber with bend radius under 1 cm (link Ch 5h)

## NEC (National Electrical Code)

### Three cable fire ratings:

#### No letter or G:  General use—least stringent fire test

#### R: Riser—can be used in vertical shafts

#### P: Plenum—most strict test

## Long-term use load

### Important for long vertical installations

### Aerial installation

## Vertical rise distance

### Must put in strain-relief loops

## Flame resistance

### Non-building applications

## UV stability

## Resistance to Rodent Damage

### Inner ducts are an alternative to armor

#### Steel armor

#### Copper tape armor

#### Braided armor

#### Dielectric armor

##### Image from arcelect.com

### Armor makes the cable much less flexible

## Resistance to water damage

### Filled and Blocked

#### Each loose buffer tube is filled (with gel or tape)

#### A blocking material fills the space between the tubes

## Crush load

### Short-term v. long-term

## Abrasion resistance

## Resistance to chemicals

## Resistance to conduction under high voltage

## Toxicity

### “Halogen-free” cables produce less harmful smoke

### Required in Japanese and European buildings

## High flexibility

### If constantly bending, like an elevator

## Hermetically sealed fiber

### Protect it from water pressure, etc.

## Radiation resistance

### Nuclear reactors or satellites

## Impact Resistance

### Dropping heavy objects on the cable

## Gas permeability

### Preventing gas from escaping through the cable

## Stability of filling compounds

### Temperature cycles can pump filling compounds out the end of the cable

## Vibration

Design Shortcuts

# Future-Proofing a System

## Include extra fibers in cables

### It costs very little more to get a cable with more fibers in it

## Include singlemode fibers in multimode cables

### Allows enormous bandwidth increases later

## Use dual-wavelength multimode fiber

### Or even laser-optimized fiber

# Multimode Fibers

## Early multimode systems used 62.5/125 micron fiber

### LED light sources at 850 or 1300 nm

### 10 Mbps or 100 Mbps

### Huge installed base

## 50/125 micron fiber

### Faster with VCSEL sources at 850 nm

## Laser-optimized 50/125 micron fiber

### Fastest, using VCSEL sources

# Singlemode Fibers

## Usually 1300 nm singlemode fiber is good enough

## Cheaper than 1550 nm or two-wavelength systems

# Cable Types

## Indoor

### Short distance – Break-out cable

### Longer distance – Distribution cable

### Rugged environment – Break-out cable

### Use all-dielectric cable

### Plenum-rated PVC is recommended

## Outdoor

### Cable should be water-blocked and gel-filled

### Many fibers (>36) – consider ribbon cable

### For midspan access, use stranded loose-tube cable

### Use all-dielectric cable

# Stranded Loose-Tube

## Same as loose-tube table

### Image from Corning (Link Ch 5f)

## Indoor/Outdoor Cable

### You could splice indoor to outdoor at the building entrance

### Or use indoor-outdoor cable like Corning’s FREEDM

### Image from Corning (Link Ch 5g)

Cable and Source Prices (Not in textbook)

# History of Ethernet

## From Corning (link Ch 5b)

# Sources

## From Corning (link Ch 5b)

# Cable Prices

## For 500 feet of riser-rated indoor bulk cable, 12-fiber

### 62.5/125 micron MM		$889

### 50/125 micron MM 		$889

### 50/125 micron
laser-optimized MM 		$1143

### 8.5/125 micron SM		$584

#### From blackbox.com (link Ch 5c)

# Media Converter Prices

## 100 Mbps Multimode:	$   229

## 1 Gbps Multimode:		$   760

## 1 Gbps Singlemode:		$1,180

### Prices from L-Com.com (Links Ch 5d & 5e)

Last modified 2-24-06