Fiber Joints

## Fibers must be joined when

### You need more length than you can get on a single roll

### Connecting distribution cable to backbone

### Connecting to electronic source and transmitter

### Repairing a broken cable

# Splices v. Connectors

## A permanent join is a splice

## Connectors are used at patch panels, and can be disconnected

# Optical Loss

## Intrinsic Loss

### Problems the splicer cannot fix

#### Core diameter mismatch

#### Concentricity of fiber core or connector ferrules

#### Core ellipticity

#### Numerical Aperture mismatch

##### Images from LANshack and tpub.com (links Ch 6a & 6c)

# Optical Loss

## Extrinsic Loss

### Problems the person doing the splicing can avoid

#### Misalignment

#### Bad cleaves

#### Air gaps

#### Contamination: Dirt, dust, oil, etc.

#### Reflectance

# Measuring Reflectance

## The reflected light is a fraction of the incoming light

### If 10% of the light is reflected, that is a reflectance of 10 dB

| If 1% of the light is reflected, 20 dB ### Acceptable Losses |  |  |
| --- | --- | --- |
| ### Fiber & Joint | ### Loss (max) | ### Reflectance (min) |
| ### SM splice | ### 0.15 dB | ### 50 dB |
| ### SM connector pair | ### 1 dB | ### 30 dB |
| ### MM splice | ### 0.25 dB | ### 50 dB |
| ### MM connector pair | ### 0.75 dB | ### 25 dB |

### Reflectance is not usually a problem for data networks, but causes ghosting in analog cable TV transmission

### Angled connectors reduce reflectance

Connectors

## There are four types

### Rigid Ferrule (most common)

### Resilient ferrule

### Grooved plate hybrids

### Expanded beam

#### Image shows ferrules from swiss-jewel.com (link Ch 6e)

#### Image to the right shows LC, SC, Biconic, and the obsolete Deutsch 1000

##### From thefoa.org (link Ch 6d)

# Rigid Ferrule Connectors

## 2.5 mm ferrule

###### ST

###### SC

# Obsolete Connectors

## Simplex (1-fiber)

###### SMA

###### D4

###### Biconic

###### FC

## 1.25 mm ferrule

## Small Form Factor

###### LC

###### MU

###### LX-5

# Duplex Connectors

## Old, bulky

###### FDDI

###### ESCON

## Images from thefoa.org (link Ch 6d) except SMA pic from www.i-fiberoptics.com/conn/IFOconn.htm

## Duplex Connectors

## Newer, smaller

## Small Form Factor

###### MT-RJ

###### Opti-Jack

###### Volition

Popular

###### Duplex LC

## Images from thefoa.org (link Ch 6d)

## Duplex LC Image from globalsources.com (link Ch 6f)

# Ferrule Polish

## To avoid an air gap

## Ferrule is polished flat, or

## Rounded (PC—Physical Contact), or

## Angled (APC)

### Reduces reflectance

### Cannot be mated with the other polish types

#### Image from LANshack (link Ch 6a)

# FOCIS

## Fiber Optic Connector Intermateability Standard

### A document produced by a connector manufacturer so others can mate to their connector

### Connectors with the same ferrule size can be mated with adaptors

### But 2.5 mm ferrules can not be mated with 1.25 mm ferrules

# Telecommunications

## In telecommunications, SC and FC are being replaced by

### LC in the USA

### MU in other countries

# Data

## In data communications, SC and ST are being replaced by LC

Connectorizing a Cable

## Epoxy-polish process (Proj. 4)

### Strip cable, strip and clean fiber

### Inject adhesive, put primer on fiber, insert fiber

### Crimp connector, cleave protruding fiber

### Air polish, final polish

### Clean and inspect by microscope

### Test connector loss with power meter

# Cable Type and Connectors

## Epoxy-polish process requires a cable jacket and strength member to make the connector durable

### It works for simplex, zip, or breakout cables

### But loose-tube cables and ribbon cables contain bare fiber, and cannot be connectorized this way

### Distribution cables contain 900 micron buffered fiber – can be connectorized, but the connectors are not very strong and must be protected by hardware such as a junction box

# Breakout Kit

## Provides tubing that protects the bare fiber so it can be terminated

### Picture from fonetworks.com (link Ch 4d)

# Mounting Methods for Connectors

## Adhesives

### Epoxy (room temperature-cure or oven-cure)

### Quick-curing anaerobic adhesives (we used this method in Proj 4)

### Hot-Melt adhesive

## Crimping to hold the fiber

### Like the Unicam – see link Ch 6h

## Splicing to preconnectorized pigtails

### Image of pigtail from fiberdyne.com (link Ch 6g)

# Mounting Methods Comparison

## Epoxy-Polish

### Takes longer, but costs less and has lowest loss and reflectance

## Anaerobic adhesive

### Faster than epoxy-polish but higher loss because polishing is difficult

## Crimping

### Easier, but more expensive and more loss

## Splicing to preconnectorized pigtail

### Very easy, but expensive and higher loss

# Strip, Clean and Cleave

## Strip – remove 900 micron buffer (if present) and 250 micron coating

## Clean with alcohol and lint-free wipe

## Cleave – scribe and snap; goal is a 90 degree flat break

# End-Face Polish

## Polish on a flat glass plate for a flat finish

## Polish on a rubber mat for a domed PC finish (Physical Contact)

## Angled PC finish is tilted at 8 degrees to avoid reflectance (difficult to field-terminate)

# Cleaning Connectors

## Keep dust caps on

## Use lint-free wipes and reagent-grade isopropyl alcohol to avoid attacking epoxy

## “Canned air” has propellant, so does compressed air from a hose

Splices

## Splices are a permanent join of two fibers

### Lower attenuation and reflectance than connectors

### Stronger and cheaper than connectors

### Easier to perform than connectorization

### Mass splicing does 12 fibers at a time, for ribbon cables

# Mass Fusion Splicing

## Video from fitel.fiberoptic.com (link Ch 6i)

# Fusion Splicing

## Melts the fibers together to form a continuous fiber

## Expensive machine

## Strongest and best join for singlemode fiber

### May lower bandwidth of multimode fiber

## Corning videos 1-7 & 12

# Mechanical Splicing

## Mechanically aligns fibers

## Contains index-matching gel to transmit light

## Equipment cost is low

## Per-splice cost is high

## Quality of splice varies, but better than connectors

## Fiber alignment can be tuned using a Visual Fault Locator

# Tools

## We covered them in Project 2

Last modified 4-15-06