Chapter 7: Fiber Optic Hardware

# Hardware

## Hardware protects and organizes cable, splices, and termination points

## Premises Hardware – indoor

## Outside plant Hardware – outdoor

# Premises Cabling

## TIA Standards

### 2000 m max. backbone wiring

### Based on telco wiring standards

#### Image adapted from siemon.com (link Ch 7a)

# Main Cross-Connect (MC)

## Should be near the data center (for LANs) or the PBX (for telephones)

### PBX: Private Branch Exchange; i.e., the campus switchboard

# MC equipment should:

## Handle enough fibers for all transmission needs

## Accept:

### Direct termination

### Pigtail splices

### Preterminated assemblies (see next slide)

## Provide jumper storage and reconfiguration

## Allow for growth

# Preterminated Assemblies (Plug & Play)

## Preterminated cable

### With SC or other connectors in a special sock for pulling, or

### MTP ribbon cable connectors (12 fibers in a single SC-size plug) that plugs into a preconnectorized patch panel

#### Pictures from Corning (link Ch 7b & 7c)

# Intermediate Cross-Connect

## Connects interbuilding cable plant to intrabuilding cable plant

## May be wall-mounted

## Image from apacn.com (link Ch 7e)

# Telecom Closet

## Transition from backbone to horizontal cable plant

### Image icc.com (links Ch 7j)

# Homerun Cabling

## This simpler, centralized network design is cheaper and popular

## It works because fiber carries data over a long distance with passive links

# Hardware for Distribution Points

## Splice panel

## Patch panel

## Wall outlet

## Conduit and innerduct

# Splice Panel

## Holds and protects splices in one or more splice trays

### Images from aflfiber.com, siemon.com, and  Corning (links Ch 7f, 7g, 7h)

# Patch Panel

## Provides a centralized location for connecting fibers, testing, monitoring, and restoring riser or trunk cables

## Requires short patch cords

### Image from aflfiber.com (link Ch 7f)

# Wall Outlets

## Terminates permanent wiring

## Provides a connection for a jumper cable to equipment

### Image from icc.com (links Ch 7j)

# Outside Plant Hardware

## Splice Closures

## Conduit or innerduct

### No connectors are usually used outdoors – only splices

# Dome-type Splice Closure

# Inline Splice Closure

# Splice Closures

## Protect splices from water and the elements

## Image from pacificinterco.com (link Ch 7k)

# Conduit and Innerduct

## Cable is installed in conduit

## Large conduits may have several innerducts inside them

### Images from glenair.com and fpnmag.com links Ch 7l and 4q)

# Cable Installed into Pavement

## MCS Road Cable is designed to be inserted directly into a slit cut into pavement

## This makes installation convenient – you don’t have to block traffic much

### Image from Corning & CENIC, from link Ch 7m

Ch 8: Getting Started in Fiber Optics

# Outside Plant

## All singlemode fiber

## High fiber counts, up to 288 or more

## Cable reels are fusion spliced together for long distances

## Preconnectorized pigtails are spliced onto the ends of the cables

## Every fiber is tested with an OTDR (Optical Time Domain Reflectometer)

## Very expensive equipment needed

# Premises Cabling

## Short lengths, fewer fibers per cable

## Multimode fiber

## Splicing is rare

## Connectors and patch cords are used for flexibility

## Installer often also installs power and copper LAN cables

## Equipment is much cheaper

# The Installers

## Outside Plant installers work for telephone companies or other large corporations

## Inside Plant installers are often independent contractors

# Training

## Go to an FOA-approved school

## Get the FOA certification

### CFOT or Certified Fiber Optic Technician (basic)

### CFOS or Certified Fiber Optic Specialist (advanced)

### CFxT (Certified FTTx Technician) (new, see link Ch 8c)

# Tools and Test Equipment

## You will need the basic tools, as found in our tool kits, including a microscope

## And a power meter

## If you do outside plant work, you will need a fusion splicer and OTDR

# Keeping Up To Date

## Like all other high-tech, fiber optics changes rapidly

## You will have to constantly learn new things

## LAN administrators typically spend 10 hours per week studying new technology

# Entering the Market

## Start slowly

## Get a small job and do it carefully

## Promote your services and grow

# Promoting and Selling Fiber Optic Services

## Make a brochure and Web site

## Take photographs of your work

## Give references of previous customers

## Ask your suppliers to recommend you to customers

# Overseeing Fiber Installations

## Supervisors must know how to install fiber correctly

## Make sure cables are pulled and terminated correctly

## Workmanship should meet standards

### NECA/FOA 301-2004, Standard for Installing and Testing Fiber Optic Cables (ANSI) (see link Ch 6d)

## Test every fiber in every cable for loss

## Document every fiber

# Plan for Inspection

## Before bidding on a job, make sure you know who the inspector will be and what will be required

## Usually the person who hired you will be inspecting, or someone at their company

## Local interpretations of fire codes and other regulations vary!

# Pricing Fiber Installations

## Request for Proposal

### Gives you general description of a project

### You must desicn, schedule, and price the job

## Calculate real cost, add overhead and some extra for contingencies

Last modified 3-10-06