Ch 9: Guidelines for Fiber Optic Design and Installation

# General Guidelines

## List Specifications

### Know just what you want

## Find Out how Performance Numbers were Measured

### If a vendor says you can terminate each fiber in 3 minutes, make sure that’s really true

## 4 Vendors for Each Component

### See table

## Talk With Vendors Who Know More Than You Do

### Avoid “the blind leading the blind”

## Expect and Resolve Contradictions

## Use Vendors With 5 Years in Business

## Use “High Serial Numbers”

### Tried and True

## Design Decisions Impact Cost

### Wavelength, connectors, cable

## Buy Only Performance you Need

## Ask Vendors About Competitors

# Cable Guidelines

## Check for Bargain Products Close to Your Needs

### Leftovers, or with unimportant defects

## Look for Best Price Possible

### Often there are great deals

## Be cautious about paying for premium fiber

### Calculate the bandwidth limits to make sure you really need it

# Other Considerations

## Most manufacturers of cable provide about the same level of reliability

# Connector Guidelines

## No connector type is “the best”

### Some are cheaper

### Some are easier to install

### Some require special tools

### etc.

## Ceramic Ferrules

### They are all manufactured by the same few companies

### Price and performance should not vary much

## Ease of Installation May Lead to Poor Performance

### Unicam-type connectors (no adhesive) use clamps to hold the fiber

### They are subject to “pistoning”

#### Fiber moving in and out under thermal stress

# Pistoning Damage

## Pistoning causes cracking on optical fiber endface

### Image from nasa.gov (link Ch 9a)

# Connector Guidelines

## There is no Substitute for Experience

### Connectorization is a skill that requires constant practice

Ch 10: Cable Plant Link Loss Budget Analysis

# Losses

## Light is lost by

### Distance along fiber (attenuation)

### Connector pairs

### Splices

### Splitters (if any)

# Loss and Bandwidth

## Loss is an important measure of a network’s quality

## But for high data rates the bandwidth of the fiber can also be important

# Attenuation-Limited Networks

## Some networks are attenuation-limited

### The max. length is determined by power loss

### Adding another connector pair means the network must be shortened

# Low Data Rate

| This is an Attenuation-Limited Network ### # Conn. Prs. | ### 2 | ### 3 | ### 4 | ### 5 | ### 6 |
| --- | --- | --- | --- | --- | --- |
| ### Max Length (m) | ### 300 | ### 280 | ### 240 | ### 190 | ### 130 |

### Standard 62.5/125 micron multimode

### 0.75 dB loss per connector pair

### 3.5 dB loss per km

### 1 Gb/s

# Dispersion-Limited Networks

## Some networks are dispersion-limited

### The max. length is determined by spreading of the light pulses

### Adding another connector pair does not affect max. length, until the loss becomes very severe

# High Data Rate

## This is a Dispersion-Limited Network

| Standard 62.5/125 micron multimode ### # Conn. Prs. | ### 2 | ### 3 | ### 4 | ### 5 | ### 6 |
| --- | --- | --- | --- | --- | --- |
| ### Max Length (m) | ### 33 | ### 33 | ### 33 | ### 30 | ### 25 |

### 0.75 dB loss per connector pair

### 3.5 dB loss per km

### 10 Gb/s

# Link Margin

## Just take the difference between transmitter power and receiver sensitivity

## Example:	T=-20 dBm	R = -30 dBm		Link Margin = 10 dB

## The loss of the total cable system must not exceed the Link Margin

# Example: Media Converters

## Top line: device with a link margin of 7 dB

## 4th line: device with a link margin of 24 dB

## Last line: link margin of 7.5 dB

### From link Ch 3h (arcelect.com)

# Attenuation

## Typical Attenuation for Multimode fiber:

### 3 dB/km @ 850 nm

### 1 dB/km @ 1300 nm

## Typical Attenuation for Singlemode fiber:

### 0.4 dB/km @ 1310 nm

### 0.25 dB/km @ 1550 nm

# Connector Loss

## Typically 0.2 – 0.5 dB per mated pair of connectors

## Industry standard max. 0.75 dB

### A single connector has “no loss” – the light comes out of it

### The measurable unit is a connector pair

# Splice Loss

## Mechanical splices – 0.3 dB max., 0.2 typical

## Fusion splices – 0.05 dB max.

# Stress Loss

## Poor installation practices can stress the fiber, causing loss

### Pulling with too much tension

### Bending past the allowable bend radius

### Tight cable ties

# Calculated Loss Budget

## Using standard values, calculate the expected loss of the link

## Actual measured loss should be less

Transmit-ter

Receiver

Connector

Splice

Connector

If it is higher, something is wrong

# Cable Plant Passive Component Loss Calculation

## MM @1300 nm; typical values

### 2 km length x 1dB/km 		= 2.0 dB

### 4 connector pairs x 0.5 dB 	= 2.0 dB

### 1 splice (mechanical) x 0.2 dB 	= 0.2 dB

### Total	 			= 4.2 dB

# Equipment Link Loss Budget Calc.

## Last line: dynamic range of 7.5 dB

## Recommended excess margin: 3 dB

## Maximum cable plant loss: 4.5 dB

## So our cable plant with 4.2 dB has a link loss margin of 0.3 dB

# To Prepare for the Next Quiz

## Study the questions on p. 137-138

## I plan to give you a problem like that

# Cable Plant Passive Component Loss Calculation

## MM @850 nm; typical values

### 2 km length x 3dB/km 		= 6.0 dB

### 4 connector pairs x 0.5 dB 	= 2.0 dB

### 1 splice (mechanical) x 0.2 dB 	= 0.2 dB

### Total	 			= 8.2 dB

7-9-08