Names:	_________________________________			Score: ________

_________________________________

_________________________________

_________________________________

Required Equipment

### Cables with SC on one end, and SC, ST, or any other 2.5 mm ferrule connector on the other end to test, with these three core sizes:

### 62.5 micron multimode (2 cables)

### 50 micron multimode (1 cable)

### 9 micron single mode (1 cable)

### A patch panel with an available SC-to-SC adapter

### Continuity tester (looks like a flashlight)

I:	Estimating Loss With the Continuity Tester

One 62.5 micron multimode cable:

Use the continuity tester to shine red light through one of the 62.5 micron multimode cables, as shown below:

62.5 micron cable

Look at the red light coming out the end of the cable, and answer these two questions:

Can you see the light at all? 	__________________

Shine the light onto a white piece of paper.  Move the paper away until the red dot becomes invisible.

How far away can you move the paper

before the red dot vanishes (estimate the distance)?	__________________

Two 62.5 micron multimode cables:

Use the patch panel to connect a second 62.5 micron cable to the end of the first 62.5 micron cable, as shown below:

62.5 micron cable

62.5 micron cable

Look at the red light coming out the end of the cable, and answer these two questions:

Can you see the light at all? 	__________________

Shine the light onto a white piece of paper.  Move the paper away until the red dot becomes invisible.

How far away can you move the paper

before the red dot vanishes (estimate the distance)?	__________________

One 62.5 micron multimode cable and one 50 micron multimode cable:

Use the patch panel to connect the 50 micron cable to the end of the 62.5 micron cable, as shown below:

62.5 micron cable

50 micron cable

Look at the red light coming out the end of the cable, and answer these two questions:

Can you see the light at all? 	__________________

Shine the light onto a white piece of paper.  Move the paper away until the red dot becomes invisible.

How far away can you move the paper

before the red dot vanishes (estimate the distance)?	__________________

One 62.5 micron multimode cable and one 9 micron singlemode cable:

Use the patch panel to connect a singlemode cable to the end of the first one, as shown below:

62.5 micron cable

Singlemode cable

Look at the red light coming out the end of the cable, and answer these two questions:

Can you see the light at all? 	__________________

Shine the light onto a white piece of paper.  Move the paper away until the red dot becomes invisible.

How far away can you move the paper

before the red dot vanishes (estimate the distance)?	__________________

Fiber Continuity Tester image from http://www.elexp.com/opt_fblp.htm