Names:	_________________________________			Score: ________

_________________________________

_________________________________

_________________________________

Required Equipment

### Cables with SC on one end, and SC, ST, or any other 2.5 mm ferrule connector on the other end to test, with these three core sizes:

### 62.5 micron multimode (2 cables)

### 50 micron multimode (1 cable)

### 9 micron single mode (1 cable)

### Connector cleaning equipment:

### Kimwipes

### Alcohol bottle

### Connector Cleaner (white tape in blue plastic dispenser)

### A patch panel with an available SC-to-SC adapter

### Power meter, including source and detector (either the Advanced Fiber Systems or Fluke)

Clean the ST connector on the end of a 62.5 micron cable, and insert it directly into the optical source.  Use a source wavelength of 850 nm.  Put the other end into the detector and record the transmitted power.  It should be approximately -20 dBm.  (If you are using the Fluke meter, you will need to use a second cable and an adapter to connect to the detector.)

Condition: 62.5 micron Launch Cable(s) Only: 	Power: __________________________

SC

Source

Receiver

ST

Advanced Fiber Systems

Meter

ST

SC to SC adapter

Source

Receiver

ST

Fluke

Meter

Two 62.5 micron multimode cables:

Add a second 62.5 micron cable to the end of the first 62.5 micron cable, and measure the transmitted power:

Condition: Two 62.5 micron multimode cables: 	Power: __________________________

Source

Receiver

ST    62.5

62.5    SC

Advanced Fiber

Systems Meter

Receiver

62.5      ST

ST   62.5

Fluke

Meter

62.5

Source

One 62.5 micron multimode cable and one 50 micron multimode cable:

Replace the second 62.5 micron cable with a 50 micron cable, and measure the transmitted power:

Condition: 62.5 micron & 50 micron cables: 	Power: __________________________

Source

Receiver

ST    62.5

50    SC

Advanced Fiber

Systems Meter

Receiver

62.5      ST

ST   62.5

Fluke

Meter

50

Source

One 62.5 micron multimode cable and one 9 micron singlemode cable:

Replace the 50 micron cable with a 9 micron singlemode cable, and measure the transmitted power:

Condition: 62.5 micron & 9 micron cables: 	Power: __________________________

Source

Receiver

ST    62.5

9    SC

Advanced Fiber

Systems Meter

Receiver

62.5      ST

ST   62.5

Fluke

Meter

9

Source