## Saving the Original Specifications of the Network

- Use a wavelength of 850 nm for all measurements.

- Use the OTDR to measure the way light propagates through the whole network.

- Draw a diagram showing all the connector pairs, with the distance to each one and the amount of loss at each connector pair.

- Measure the system from the other end (the other connector at the same physical patch panel).

- Draw a diagram showing all the connector pairs, with the distance to each one and the amount of loss at each connector pair.

- Use a power meter to measure the loss of the whole network.  Record the value.

## Having the Network Altered

- Call over the instructor to modify the network.

- Measure the power loss through the altered network.  Record the value.

- Use the OTDR to locate the change, and guess what change has been made.

- After you have found the location and nature of the problem from the end of the network, walk to the problem area, see what the problem is, and correct it.

- Measure the network again and verify that it is restored to its original condition.

Last modified 10-20-08