Chapter 23 - Setting Up a Small Network

## Workgroups Only in This Chapter – They are the same for Windows XP Home Edition and Windows XP Professional

A Workgroup Lets You

NIC

## Share folders

## Share printers

## Share an Internet connection

## But no central administration of accounts or resources – each machine has to be set up individually

# Network adapters

## Each computer needs an adapter (also called a network interface card, or NIC)

Network adapters can be internal (usually installed in a PCI slot) or external (typically connected to a USB port)

Hub

## Ethernet is the most popular by far

## Pic from Network+ Guide to Networks, Second Edition by Tamara Dean

# Hub

## A hub or switch can be used to connect the computers in an Ethernet network

### To connect two computers, you can use a crossover cable and no hub

# Router(see photo to the right)

You can also use a router or residential gateway, which typically adds network address translation (NAT) capabilities and security features

Router

# Wireless Network Access Point

## On wireless networks, a wireless access point handles these duties

# Cables

## On an Ethernet network, eight-wire Category 5 patch cable with RJ-45 connectors on each end

## Installing and Configuring Network Adapters

### Happens automatically for Plug and Play adapters

# Ethernet, Wireless, or Phone Line?

## Three popular technologies, all supported by Windows XP

### Ethernet/Fast Ethernet

#### 10, 100 or 1000 Megabits per second

### Wireless

#### IEEE 802.11b, also known as Wi-Fi -- 11 megabits per second

#### IEEE 802.11g and 802.11a, -- 54 megabits per second

Phone Line

Network Access Point

#### Home Phoneline Networking Alliance (HomePNA) -- 10 megabits per second, using normal phone lines.

# Making Connections: Cables and Hubs

## Place the hub in a central location

## The segment length (distance between furthest points) should not exceed 100 meters (328.1 feet)

## All the ports are the same on a hub, except the uplink port

#### Uplink ports are used to expand a network’s capacity by connecting two hubs

#### The uplink port achieves the same purpose as a crossover cable

## In addition to (or in place of) a hub, your network may use a router, switch, or residential gateway

### Often used to share a fast Internet connection

## If you plan to use Internet Connection Sharing and you have an external DSL or cable modem, you’ll need to install two network adapters in the computer with the shared Internet connection

# Wired Equivalent Privacy

## On wireless networks, anyone who comes into range of your wireless access point can tap into your network and intercept signals from it.

## To add basic security, use Wireless Equivalent Privacy (WEP)

## WEP encrypts the data with a 40-bit or 104-bit key (sometimes called 64-bit and 128-bit)

## For details, see Link Ch 23a

# Wi-Fi Protected Access (WPA)

## WEP has weaknesses – because it uses only a 24-bit initialization vector, it can be cracked by collecting a lot of traffic and analyzing it mathematically

## WPA is a newer, more secure, encryption technique

## Use it if your hardware supports itInfrastructure and Ad Hoc Modes

# Infrastructure and Ad Hoc Modes

## Wireless LANs have two modes

## Infrastructure mode

### Connects to a Network Access Point

### Several computers can share an Internet connection

## Ad hoc mode

### Connect directly to other computers

### No Network Access Point

## At a coffeehouse, a single laptop in ad hoc mode can stop other laptops from accessing the Internet

## Pic from http://www.hardavenue.com/reviews/adhocguide.shtml

# Configuring a Default Network Connection

## Windows creates a local connection with four default items

### Start, Right-click My Network Places, Properties, Right-click a connection, Properties

## Client For Microsoft Networks

### Lets you connect to other Windows computers as a client

## File And Printer Sharing For Microsoft Networks

### This service allows other computers on your LAN to access shared resources on your computer – your computer acts as the server

## QoS Packet Scheduler

### Not used much yet, will ensure Quality of Service in IPv6

## Internet Protocol (TCP/IP)

### Used for all modern network traffic, and on the Internet

# To See Network Connections

## Open Control Panel, Network Connections

### To see more detailed information about a network connection, double-click its icon in the Network Connections window

### Or Right-click, Properties

### Or look in left pane at the bottom

# The Network Setup Wizard

## Sets the proper permissions on shared folders

## Adds required keys to the registry

## Configures protocols and binds them to network cards

## Enables or disables the internet connection firewall

## Adjusts system policies so that file sharing works properly over the network

# Internet Connection Sharing

## If you use Internet Connection Sharing run the Network Setup Wizard on the Internet Connection host first

## If all your networked computers have separate Internet connections, or if you use a router or residential gateway to share an Internet connection, you can start with any computer.

# To start the Network Setup Wizard

## Start, My Network Places, Set up a home or small office network

# Select A Connection Method

## This Computer Connects Directly To The Internet…

### Select this option if you’ve already set up the Internet connection on the computer

## This Computer Connects To The Internet Through Another Computer …

### If you use a router or residential gateway to manage shared Internet access, select this option

### If you use Internet Connection Sharing and you see this screen, stop and check your Internet connection

#### If ICS is properly set up, the wizard should display a different page

## Other

### Select this option if your computer is connected to the Internet directly or through a hub, or if your network has no Internet access

# Other Internet Connection Methods

## If you selected “Other”, you see this

## This Computer Connects To The Internet Directly…

### This configuration exposes your system to serious security risks

## This Computer Connects Directly To The Internet. I Do Not Have A Network Yet

## This Computer Belongs To A Network That Does Not Have An Internet Connection

# Select Your Internet Connection

## Confirm that the wizard has selected the correct connection and click Next to continue

# Give This Computer A Description And Name

## Next, Workgroup Name and Ready To Apply Network Settings page.

# Network Setup Disk

## The network setup disk is used on systems running previous versions of Windows that are connected to the same network

# Run the Network Setup Wizard on Other Computers Using ICS

## Choose the first option: Yes, Use The Existing Shared Connection For This Computer's Internet Access.

# Adjusting Network Settings

## Although the Network Setup Wizard is the preferred way to work with network settings, you may need to adjust settings manually under some circumstances

# Installing and Configuring Protocols

## For virtually any networking task, TCP/IP is the preferred solution

## But if you have old networks, you may be using an old protocol like

## IPX/SPX

### Originally developed for Novell NetWare

## NetBEUI

### Originally developed for use on small networks running Windows 3.x

### Short for NetBIOS Extended User Interface

# To Install an Additional Protocol

## Start, Right-click My Network Places, Properties

## Select the connection and click the Change Settings Of This Connection link under Network Tasks

## From the properties dialog box, click the Install button

## Choose Protocol and click Add

# Configuring Workgroup Settings

## In My Network Places, click the View Workgroup Computers link under Network Tasks in the left pane

### You should see the other computers on your workgroup

# Workgroup Name

## To communicate properly, all the computers on must be members of the same workgroup

## To change your workgroup name:  Right-click My Computer, Properties, Computer Name tab, Change

# IP Addresses

## On a TCP/IP network, every computer has a unique IP address, which consists of four 8-bit numbers (each one represented in decimal format by a number between 0 and 255) separated by periods

## TCP/IP configuration has three additional settings:

### Subnet Mask

### Default Gateway

### DNS Server

# Subnet Mask, Default Gateway, DNS Server

## Subnet mask

### Tells the network how to distinguish between IP addresses that are part of the same network and those that belong to other networks.

## Default Gateway

### A computer that can send packets outside the local network

## Domain Name System (DNS) Servers

### Computers that translate domain names (such as www.microsoft.com) into IP addresses

# Methods For Assigning IP Addresses

## Dynamic Host Configuration Protocol (DHCP)

### The most common method

### IP configuration is set automatically by the server

## Automatic Private IP Addressing (APIPA)

### If DHCP fails, the machines make up their own addresses starting with 169.254.

## Static IP Addressing

### Administrator must manually type in the IP address for each machine

### Servers typically use static IP addresses

### Requires more administrative effort and not commonly used for workstations anymore

## Alternate IP Configuration

### New feature in Windows XP

### You set the address used if DHCP fails

### Useful for a laptop that travels between two different LANs

# Public IP Addresses

## Public IP addresses are like public telephone numbers – every computer that is directly connected to the Internet needs one

## When you connect to an Internet service provider, you’re assigned a public IP address from a block of addresses registered to that ISP

## Dynamic IP Address

### If you use a dial-up connection, your ISP probably assigns a different IP address to your computer (drawn from its pool of available addresses) each time you connect

## Static IP Address

### If you have a persistent connection to your ISP via a DSL or cable modem, your IP address may be permanent.

# Private IP Addresses

## A whole network can share a single Public IP Address

### Better Security

### Lower Cost

## Each of the computers on the local network has a Private IP Address that is not reachable from the outside world.

## The gateway computer uses network address translation (NAT) to pass packets back and forth between the single public IP address and the many private IP addresses on the network.

# Reserved Private IP Addresses

## The Internet Assigned Numbers Authority (IANA) has reserved the following three blocks of the IP address space for use on private networks that are not directly connected to the Internet:

### 10.0.0.0 – 10.255.255.255

### 172.16.0.0 – 172.31.255.255

### 192.168.0.0 – 192.168.255.255

## In addition, the Automatic Private IP Addressing feature in Windows 98 and all later versions uses private IP addresses in the range from 169.254.0.0 to 169.254.255.255.

### See link Ch 23b

## Routers, switches, and residential gateways that use NAT almost always assign addresses from these private ranges

### The RG-1000 residential gateway from Agere uses 10.0.0.x addresses

### Linksys routers use 192.168.1.x

### Internet Connection Sharing in Windows XP uses 192.168.0.x

# To set a static IP address

## Start, right-click My Network Places, Properties, right-click the connection, Properties

## Select Internet Protocol (TCP/IP), click the Properties button

# To set up an alternate IP configuration

## On the General tab of the Internet Protocol (TCP/IP) Properties dialog box, select Obtain An IP Address Automatically.

## Click the Alternate Configuration tab and then select the User Configured option.

# Configuring a Dial-Up Internet Connection

## Control Panel, Network Connections

## Create A New Connection link in the Network Tasks pane

## Connect To The Internet

## Set Up My Connection Manually

## Connect Using A Dial-Up Modem

# Sharing an Internet Connection

## To share an Internet connection safely on a small network, you have two options:

### Install a router or residential gateway

### Use Internet Connection Sharing (ICS)

# Your Shared Internet Connection Isn’t Working.

## The Internet Connection Sharing service is not be running

### Control Panel, Administrative Tools, Services, restart the  Internet Connection Sharing service

## The wrong network adapter is shared

## The settings on other network computers are incorrect

### They should all be set to obtain an IP address automatically and obtain DNS servers automatically. Leave the default gateway field blank.

# Security Risks of a Peer-to-Peer Network

## If you have an external DSL or cable modem and you connect it to an Ethernet hub along with other computers on your network, one of two things will happen:

### If you use the Internet Connection Firewall on each computer, you are safer but you cannot share folders or printers

### If you disable the Internet Connection Firewall, every computer on your network can see the other computers and access shared resources, and so can everyone on the Internet.

# Making a Peer-to-Peer Network More Secure

## Upgrade to Service Pack 2

## Use a third-party firewall on each machine, like Zone Alarm

## Alternatively, you can Disable File and Printer Sharing on the TCP/IP protocol and enable it on the IPX/SPX protocol for each computer on your network

Chapter 24 - Tweaking and Troubleshooting a Small Network

# These commands work the same in Windows XP Home Edition and Windows XP Professional

# Command-line TCP/IP Troubleshooting Utilities

## PING

## PATHPING

## TRACERT

## NSLOOKUP

## IPCONFIG

# PING

## To see if you can communicate with a machine, use PING at a command prompt

### PING machine_name

### PING IP_Address

# PING

## If PING times out, you could have:

### A bad cable or hub

### A bad TCP/IP connection on your computer

### A problem with another computer on that network

# Loopback

## Ping your own machine using either of the following commands:

### ping 127.0.0.1

### ping localhost

## This standard IP address corresponds to your computer

## If you receive an error, then TCP/IP is not configured properly on your system

# Tracing a Path

## Ping your computer’s IP address.

## Ping the IP address of another computer on your network

## Ping the IP address of your router or the default gateway on your network

## Ping the address of each DNS server on your network

## Ping a known host outside your network

## PathPing the same known host

# DNS Problems and NSLOOKUP

## If you can ping a Web site using its IP address but not by using its name, DNS problems are indicated

## You can test DNS with

### NSLOOKUP domain_name

# Tracert

## tracert target_name

### Sends a series of packets out, measuring the time it takes to reach each "hop" along the route

### Timeouts or unusually slow performance indicate a connectivity problem

## PATHPING does the same thing, but also sends several packets to each site and shows % loss for each one

# Simplify the Problem

## If you’re using a third-party firewall program, disable it temporarily

## Try bypassing your router

### Connecting directly to a broadband connection such as a DSL or cable modem.

## Check your Firewall

### Look in the Network Connections folder for a padlock icon and the word Firewalled on the connection icon in question

# Diagnosing IP Address Problems

## To see details of your current IP configuration, follow these steps:

### Control Panel, Network Connections

### Double-click the connection

### Support tab

# Network Connection Details

## In the Connection Properties, on the Support tab, click the Details button

# IPCONFIG

## IPCONFIG displays the DNS suffix, IP address, subnet mask, and default gateway of your primary network connection

## For more details use IPCONFIG /ALL.

# IP Addresses to Look For

## 169.254.x.y

### Your computer is using Automatic Private IP Addressing (APIPA)

### Your computer was unable to reach a DHCP server

## If your address is a Private IP Address like 192.168.x.y or 10.x.y.z

### Private IP addresses cannot connect directly to the Internet

### Make sure that another computer (an Internet Connection Sharing host) or a router or residential gateway is routing your Internet requests to a properly configured public IP address

## 0.0.0.0 – No communication is possible with that IP address

### The network is disconnected, or

### The static IP address for the connection duplicates an address that already exists on the network

# Subnet Mask

## Compare IP settings on the machine that’s having problems with those on other computers on the network

## The default gateway and subnet mask should be identical for all network computers

## The first one, two, or three sets of numbers in the IP address for each machine should also be identical, depending on the subnet mask

### A subnet mask of 255.255.255.0 means the first three IP Address numbers of computers on your network must be identical

#### 192.168.0.83 and 192.168.0.223 can communicate

#### 192.168.1.101 will not be recognized as belonging to the network

## With a subnet mask of 255.255.0.0, the first two sets of numbers must match

### 172.16.2.34, 172.16.4.56, and 172.16.83.201 are all valid addresses on a subnet with this mask

### 172.17.4.1 would not be able to communicate on that subnet

## In every case, the gateway machine must also be a member of the same subnet

# Network and Host Portion

# Class A Addresses

## When the subnet mask is 255.0.0.0

### The first number in the IP address is called the network portion of the IP address

### The last three numbers are called the host portion

## Example: 10.15.19.101 with a subnet mask of 255.0.0.0 has

### Network portion 10

### Host portion 15.19.101

# Class B addresses

## When the subnet mask is 255.255.0.0

### The first two numbers in the IP address are called the network portion of the IP address

### The last two numbers are called the host portion

## Example: 172.16.1.101 with a subnet mask of 255.255.0.0 has

### Network portion 172.16

### Host portion 1.101

# Class C addresses

## When the subnet mask is 255.255.255.0

### The first three numbers in the IP address are called the network portion of the IP address

### The last number is called the host portion

## Example: 192.168.1.101 with a subnet mask of 255.255.255.0 has

### Network portion 192.168.1

### Host portion 101

# Repairing Your TCP/IP Configuration

## Run the Network Setup Wizard

## Use the automated repair option

### Double-click the connection icon in the Network Connections folder and click the Repair button on the Support tab

# What Repair Does

## The Repair button has the same effect as typing these commands :

### ipconfig /renew. Automatically renews your IP address from a DHCP server

### arp –d. Flushes the Address Resolution Protocol cache

### nbtstat –R. Reloads the NetBIOS name cache

### ipconfig /flushdns. Clears the DNS cache

### ipconfig /registerdns. Registers the computer’s name on the appropriate DNS server

# Resolving DNS Issues

## DNS servers translate host names (www.microsoft.com, for instance) into numeric IP addresses

### If you can use the Ping command to reach a numeric address outside your network but are unable to browse Web sites by name, the problem is almost certainly related to your DNS configuration

### To make sure, try NSLOOKUP domain_name at a command prompt

#### For example, NSLOOKUP YAHOO.COM

# DNS Problems

## Do your TCP/IP settings point to the right DNS servers?

## Is your ISP experiencing DNS problems?

## Have you installed any "Internet accelerator" utilities?

### These may use a static Hosts file on your machine which has become out-of-date

# The Hosts file

## Hosts can be used instead of DNS, but it is rarely used in modern networks

### Can be useful on a mid-size network where all computers have static IP addresses

### By entering computer names and IP addresses in a Hosts file, you eliminate the need to broadcast messages around the network looking for each machine

### Instead, Windows finds the machine name and matching IP address in the Hosts file and goes straight to the correct address

### %SystemRoot%\System32\Drivers\Etc\Hosts

# Fixing Problems with My Network Places

## Often, computers are missing from the My Network Places folder

### The Browser service fills that folder, and it is often very slow (up to 24 minutes)

#### See links Ch 30a-d

### You can usually connect by typing in the name of the other computer in  the Run box, such as

#### \\Instructor1\CNIT235

# Network Troubleshooting Tools

## Ipconfig

### Displays all current TCP/IP network configuration values and refreshes DHCP and DNS settings

## Nslookup

### Displays information about Domain Name System records for specific IP addresses and/or host names, so you can troubleshoot DNS problems

## PathPing

### Combines functions of Traceroute and Ping to identify problems at a router or network link

## Ping

### Verifies IP-level connectivity to another Internet address by sending ICMP packets and measuring response time in milliseconds

## Tracert

### Determines the path to an Internet address and lists the time required to reach each hop; useful for troubleshooting connectivity problems on specific network segments

## Nslookup

### Resolves a domain name to an IP address; useful to test DNS server function

# Bridging Two Networks

## Windows XP can create a network bridge, which brings two network segments together seamlessly and creates a virtual connection between them