Ch 9: Wireless Security

# Components of a Wireless (Wi-Fi) Network

## Wireless Router

## Wireless network interface card (WNIC)

## Ethernet cable to connect AP to Internet

# Wireless Router

## Wireless Router receives and sends radio signals

## It also connects to the Internet with an Ethernet cable

## It bridges the wireless network with the wired network

# Wi-Fi NICs

## To connect to a Wi-Fi network, you need a Network Interface Card (NIC)

## The most common type is the PCMCIA card

### Designed for laptop computers

# USB and PCI Wi-Fi NICs

## USB

### Can be used on a laptop or desktop PC

## PCI

### Installs inside a desktop PC

PCMCIA

PCI

USB

# Wireless Network Standards

## 802.11b

### Max. speed 11 megabits per second

### Range 100 to 150 feet

## 802.11g

### Max. speed 54 megabits per second

### Range 100 to 150 feet

## 802.11a

### Max. speed 54 megabits per second

### Range 25 to 75 feet

# All Devices Must Match

## Your Wireless Router must use the same standard as your Network Interface Card

### Older devices use 802.11b

### Newer ones can usually support 802.11g or 802.11b

### "Wireless-N" devices are even faster

# Service Set Identifiers (SSIDs)

## Name used to identify the wireless local area network (WLAN)

## The AP usually broadcasts the SSID

# Service Set Identifiers (SSIDs)

## The SSID is set on the Access Point

## The Channel is also set on the Access Point

### Usually channel 1, 6, or 11

# Configuring a Wireless Router

## Most devices allow access through any Web browser

### Connect a computer to the Wireless Router

### Open a browser

### Enter the Wireless Router's IP address

### Provide the Wireless Router's logon name and password

# Name and Password

## The Wireless Router name and password starts with something simple and obvious like 'admin' and no password

## You should change it to something only you know

## If you forget your password, hold down the reset button on the back of the router for 10 seconds to restore factory settings

# Encryption

## Everything you send through a wireless network can be received by any other computer nearby

## You need to encrypt the traffic for privacy

# Wireless Security Options

## Wired Equivalent Privacy (WEP) encryption

### Out of date

### Can be easily hacked into

## WPA (WiFi Protected Access )

### Much safer than WEP

### WPA-PSK (Pre-Shared Key) is the most common type

### WPA2 is even safer

# Essential Security Measures

## Change your router's password

### Choose a password that is not a dictionary word

## Use WPA (or WPA2) encryption

### Choose a key that is not a dictionary word

# Optional Security Measures

## Change your SSID

## Turn off SSID Broadcast

### These changes make your network a little harder for hackers to find

### Won't stop serious hackers

## WEP encryption

### If you have old equipment that cannot use WPA, WEP is better than no encryption at all

# Disabling SSID Broadcasts

# WEP Encryption is Risky

## WEP is not trustworthy

### TJX: Largest consumer data theft ever

### Because they used WEP for their wireless network

#### See links Ch 2a-d

Last modified 6-20-07