Ch 19: Implementing Network Security

# Network and Sharing Center

## Network discovery

## File and printer sharing

## Public folder sharing

## Media streaming

## File sharing connections

### Send authentication credentials with 128-bit encryption or older, less secure methods

## Password protected sharing

## HomeGroup connections

# Network Discovery

## Used to draw the Network Map, in Network and Sharing Center

# Password-Protected Sharing

## This makes sharing very inconvenient

## Each person who accesses a shared resource needs an account on the server

# HomeGroup Connections

## By default, Windows uses the HomeGroupUser$ account to

# Sharing Wizard

## Right-click a folder, Share with…, Specific people

## Simplifies sharing

# Deactivating the Sharing Wizard

## Uncheck it in Folder Options

# Advanced Sharing

## In a folder’s properties, on the Sharing tab, click "Advanced sharing", check "Share this folder", click Properties

## Error in book: this is possible with the Sharing Wizard activated

# Sharing and NTFS Permissions Work Together

## Share permissions are required to connect to a resource over the network

## NTFS permissions apply to both local and remote users

### NTFS permissions are a more powerful security barrier in most cases

# Example

## Share permissions on a folder

### Sam has Full Control

### Everyone has Read

## NTFS Permissions

### Sam has Read

### Everyone has Full Control

## Sam has Read whether he connects locally or remotely

## Other users have Full Control locally, but Read remotely

# Administrative Shares

## Windows shares the root of each drive

## It also makes ADMIN$ for remote administration and IPC$ for inter-process communication

## You can hack the Registry to remove the Administrative Shares, but some Windows features require them

# Hidden Shares

## The $ sign makes the Administrative Shares hidden--they won't appear in Windows Explorer on a remote machine

## You can put a $ at the end of your own shared folders to hide them

## This is not very secure--Linux can still see them

# Credential Manager

## Delete credentials you don't need

# Restricting Logon Hours

## From an Administrative Command Prompt

### net user username /time:M-F,8am-5pm

#### For more, see link Ch 19a

# Forcing Logoff when Logon Hours Expore

## In Local Security Policies (Start, SECPOL.MSC)

## Network security: Force logoff when logon hours expire

Ch 20: Wireless Network Security

Configuring Wireless Routers

# Sat. Nov 7, Noon

## This semester's wardrive (20 pts extra credit)

## Meet in S214

## Bring whatever you have

### Cars

### Laptops

### Antennas

### Nothing at all

## We'll drive around for an hour, then meet for lunch and compile the data

## Netgear WGR614 (v7)

### Popular, low-cost access point

### Four switch ports, routing capabilities

### Supports 802.11b, 802.11g transmission

## Configuration steps on other small wireless connectivity devices

### Differ somewhat

### Follow similar process, modify same variables

# Router Passwords

## If you don't have the password

### Hold down the reset switch for 10 seconds to restore router to factory defaults

#### Sometimes you need to power cycle the router with the button down

## Find the default password online

### routerpasswords.com

## Don't trust home router security much

### "Router Hacking Contest" at link Ch 20b

# Opening a Router's Configuration Page from the Network Map

## Right-click device

## View Device Webpage

### But it couldn't find my AT&T device at 192.168.1.254

# Wireless Encryption

## We wardrive San Francisco every semester

## Most wireless networks are insecure

# WPA v. WEP

## Open networks let anyone connect

## WEP is the older encryption technique, easily broken in a few minutes with the right network card

## WPA is much safer.  Just don't use a dictionary word as the passphrase.

## WPA-2 is even safer than WPA

# MAC Address Filtering

## Used on the CCSF Wi-Fi network

### www.ccsf.edu/wifi

## This is a very weak security measure

## Easily defeated, because MAC addresses are not concealed at all

### Use Cain to read all the MAC addresses on the network now

### Adjust network card properties to impersonate anyone you like

## This is, of course, dishonest and possibly illegal

Ch 21: Troubleshooting and Recovering from Problems

# Error Messages

## Google the exact text of the message you got

## Sometimes they aren't very helpful

### Links Ch 21a, 21b

# Event Viewer

# System Information

## Start, MSINFO32

## Useful items

### Conflicts/Sharing

### Problem Devices

# Automatic Restart

## Blue screen errors don't last long by default, because Windows automatically restarts

## To adjust that

### Start

### Systempropertiesadvanced

### In "Startup and Redovery" section, click Settings

# Debugging Information

## Small memory dump

### Includes the stop error and its description, running device drivers, and the processor state

## Kernel memory dump

### Only the Kernel (1/3 the size of RAM)

## Complete memory dump

### All of the RAM (2 GB on my machine)

# Repairing Applications

## In Programs and Features

## You can repair or uninstall/reinstall

# Recent Changes to Investigate

## Did you recently

### Edit the registry

### Change Windows settings

### Change application settings

### Install a new program

### Install a new device

### Install an unsigned driver

### Apply a Windows update

## These can all cause problems

# Troubleshooters

# Disk Diagnostics

## Self-Monitoring, Analysis, and Reporting Technology (SMART)

## Measures the health of a hard disk

### Spin-up time

### Drive temperature

### Error rates, etc.

## Windows 7 will automatically alert you if it detects a problem

# Memory Diagnostic

# Problem Reporting in Action Center

## Windows checks for a solution to any problem

## Asks for permission to send information back to Microsoft

# Online Resources

## Microsoft Product Support

### support.microsoft.com   (link Ch 21d)

### General help

## Microsoft Knowledge Base

### Specific, detailed, problem solutions  (link Ch 21e)

## Technet

### Technet.microsoft.com

### For IT Professionals (link Ch 21f)

# Online Resources

## Windows Update

## Microsoft Security

### microsoft.com/security  (link Ch 21g)

## Vendor websites

## Windows 7 Newsgroups

# Recovering from a Problem

## Last Known Good Configuration

### Press F8 during startup

### Only helps with driver problems, a weak solution

## System Restore

### Very powerful and easy

### Can be undone

### Often the best option

Last modified 10-23-09