Total Internal Reflection

# Refraction (Bending) of Light

## Sunlight shines down into the water

## Ray A comes from straight up into the water and does not bend much

## Ray B comes at a shallow angle and bends a lot more

### Image from seafriends.org.nz

# The View From Underwater

## Underwater, the light always shines down steeply, even when the Sun is low in the sky

## The whole sky appears in a limited round area called “Snells Window”

### Image from seafriends.org.nz

# Light is Trapped Underwater

## The light ray that comes from the fish to the origin cannot escape the water

## Total Internal Reflection

# Guiding Light With Water

## Light in a stream of water stays inside the water and bends with it

## This was first demonstrated in the 1840s

### Image from glenbrook.k12.il.us/gbssci

# Total Internal Reflection

## There is a critical angle at which no light can be refracted at all, so 100% of the light is reflected

### Light is trapped in the water and cannot escape into the air

### This works with any dense medium, such as plastic or glass, the same way it works with water

#### Image from glenbrook.k12.il.us

Types of Optical Fiber

# Bare Fiber

## During 1920-1950, thin, flexible rods of glass or plastic were used to guide light

## Such “bare” fibers require air outside each fiber

## Fibers degrade rapidly from handling

### Image from Wikipedia

# Fiber With Cladding

## Developed in 1954 by van Heel, Hopkins & Kapany

## Cladding is a glass or plastic cover around the core

## Protects the total-reflection surface from contamination

## Reduces cross-talk from fibers in bundles

# Corning

## Corning scientists developed low-attenuation silica glass fibers in 1970

## Corning Video: At The Speed of Light

### Link Ch 1c on my Web page (samsclass.info, CNIT 211 / ELEC 211)

Singlemode and Multimode Fiber

## Singlemode fiber has a core diameter of 8 to 9 microns

## Multimode fiber has a core diameter of 50 or 62.5 microns

## Both have a cladding diameter of 125 microns

# Singlemode Fiber

## Singlemode fiber has a core diameter of 8 to 9 microns, which only allows one light path or mode

### Images from arcelect.com (Link Ch 2a)

# Multimode Step-Index Fiber

## Multimode fiber has a core diameter of 50 or 62.5 microns (sometimes even larger)

### Allows several light paths or modes

### This causes modal dispersion – some modes take longer to pass through the fiber than others because they travel a longer distance

# Multimode Graded-Index Fiber

## The index of refraction gradually changes across the core

### Modes that travel further also move faster

### This reduces modal dispersion so the bandwidth is greatly increased

# Popular Fiber Types

## At first there were only two common types of fiber

### 62.5 micron multimode, intended for LEDs and 100 Mbps networks

#### There is a large installed base of 62.5 micron fiber

### 8 micron single-mode for long distances or high bandwidths, requiring laser sources

#### Corning’s SMF-28 fiber is the largest base of installed fiber in the world (links Ch 2j, 2k)

# Gigabit Ethernet

## 62.5 micron multimode fiber did not have enough bandwidth for Gigabit Ethernet (1000 Mbps)

LEDs cannot be used as sources for Gigabit Ethernet – they are too slow

LED

## So Gigabit Ethernet used a new, inexpensive source:

### Vertical Cavity Surface Emitting Laser (VCSEL)

# LEDs and VCSELs

## From www.mtmi.vu.lt/pfk/funkc_dariniai/diod/led_laser.htm

# Multimode Fiber Designed for VCSELs

## First came laser-rated 50 micron multimode

### Bandwidth 500 MHz-km at 850 nm

## Then came laser-optimized 50 micron multimode

### Bandwidth 2000 MHz-km at 850 nm

### Distinctive aqua-colored jacket

See links Ch 2g, 2h, 2i

VCSEL

Cable Types

# Loose-Tube Cable

## 1-12 fibers loose in a tube

Strength member outside the tube

## Used Outdoor

### Ducts or conduits

### Aerial lashed

### Directly buried (armored)

### Images from alphawire.com (link Ch 4k)

# Distribution Cable

## Contains tight-buffered fibers in bundles of up to 12 each

## Used for Riser and Office Cabling

## Must be terminated inside a patch panel or junction box

# Breakout Cable

### Reinforce each tight-buffer fiber with Kevlar and jacket it

### Each fiber can be broken out and individually connectorized

#### Image from arcelect.com (link Ch 4g)

# Ribbon cable

## Dozens of fibers packed together

## Can be mass fusion spliced or mass terminated

## Images from gore.com (link Ch 4b) and alcatel.com (link Ch 4c)

Connectors and Splices

# Connectors

# Rigid Ferrule Connectors

## 2.5 mm ferrule

###### ST

###### SC

###### FC

## 1.25 mm ferrule

## Small Form Factor

###### LC

###### MU

###### LX-5

## Images from thefoa.org (link Ch 6d)

# Fusion Splicing

## Melts the fibers together to form a continuous fiber

## Expensive machine

## Strongest and best join for singlemode fiber

### May lower bandwidth of multimode fiber

# Mass Fusion Splicing

## Video from fitel.fiberoptic.com (link Ch 6i)

# Mechanical Splicing

## Mechanically aligns fibers

## Contains index-matching gel to transmit light

## Equipment cost is low

## Per-splice cost is high

## Quality of splice varies, but better than connectors

## Fiber alignment can be tuned using a Visual Fault Locator

OTDR: Testing Cable Plants

# OTDR: Optical Time-Domain Reflectometer

## Image from exfo.com

# OTDR Uses

## Measure loss

## Locate breaks, splices, and connectors

## Produces graphic display of fiber status

### Can be stored for documentation and later reference

## Cable can be measured from one end

# Backscatter

## A small amount of light is scattered back to the source from the fiber itself

## Splices or connector pairs cause a larger reflection of light back to the source

### Figure from techoptics.com (link Ch 17a)

# OTDR Display

Start of cable

Connection Points

End of cable

# OTDR Trace

# of CCSF Fiber

# Loop

Total length:

66 km

The Future of Fiber Optics: Faster Networks

# History of Ethernet

## From Corning (link Ch 5b)

# Bandwidth Comparison

## Multimode Fiber is limited to 10 Gbps for reasonable lengths (30 m)

## Singlemode Fiber is MUCH faster – its bandwidth is estimated to be 100,000 Gbps (100 Tbps)

### But no electronics are available yet to transmit signals that fast

### From www.lucent.com/press/0601/010628.bla.html

# Singlemode Fiber is NOT Expensive

## Price of 500 feet of riser-rated indoor bulk cable, 12-fiber

### 62.5/125 micron MM		$889

### 50/125 micron MM 		$889

### 50/125 micron
laser-optimized MM 		$1143

### 8.5/125 micron SM		$584

#### From blackbox.com (link Ch 5c)

# Faster Electronics ARE More Expensive

## Price of electronic-to-optical media converters

### 100 Mbps Multimode:	$   229

### 1 Gbps Multimode:		$   760

### 1 Gbps Singlemode:		$1,180

#### Prices from L-Com.com (Links Ch 5d & 5e)

# Fiber to the Home

## One fiber cable can carry voice, data, and video

## Already available in many cities

### http://www22.verizon.com/FiOSForHome/channels/FiOS/root/about_installation.asp

The Future of Fiber Optics: More Secure Networks

# Quantum Encryption

## Encryption keys are sent as individual photons

### Undetected spying is impossible, because measuring the photons changes them

#### www.toshiba-europe.com/research/crl/QIG/securityfromeavesdropping.html

# Commercial Quantum Key Distribution Devices

## MagiQ and Clavis: www.magiqtech.com and www.idquantique.com/products/clavis.htm

# Sources

## Fiber Optic Technician's Manual, Third Edition by Jim Hayes

## City of Light : The Story of Fiber Optics (Sloan Technology Series) by Jeff Hecht

## Lennie Lightwave Guide To Fiber Optics www.lennielightwave.com

Last modified 6-5-06